### Chloride Data from DNR SWIMs database (samples from Urban Road Salt Study).

|  | (Upstream) | Root River Location |  |  |  |  |  | (Downstream) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @ Layton | @ Grange |  | @ Puetz | @ Ryan |  | @ Root River Canal | @ 60th St Bridge (Return Flow) |
| % Urban | 86% | 85% |  | 70% | 68% |  | 34% | 34% |
| Watershed Area (sq mi) | 12 | 15 |  | 46 | 49 |  | 69 | 126 |
| Sample Date (All from Urban Road Salt Study) | Chloride (mg/L) | Chloride (mg/L) | USGS Flow (cfs) | Chloride (mg/L) | Chloride (mg/L) | USGS Flow (cfs) | Chloride (mg/L) | Chloride (mg/L) |
| 2/13/2011 | 1360 | 1220 | 2.4 | - | 670 | 6.3 | - | - |
| 2/27/2011 | 2360 | 4130 | 3.6 | 763 | - | 42 | - | - |
| 3/1/2011 | 2290 | 2240 | 4.9 | 1100 | - | 41 | - | - |
| 3/25/2011 | 401 | 442 | 27 | 238 | - | 169 | - | - |
| 6/20/2011 | 129 | 112 | 25 | 144 | - | 72 | - | - |
| 9/28/2011 | 73.8 | 78.1 | 36 | 111 | - | 67 | - | - |
| 1/23/2012 | 2930 | 2130 | 20 | 417 | - | 26 | - | - |
| 2/16/2012 | 921 | 1100 | 4.2 | 465 | - | 15 | - | - |
| 5/31/2012 | 315 | 240 | 7.6 | 168 | - | 9.6 | - | - |
| Bold and Red Values exceed the WisCALM 2012 Chloride Acute Toxicity Threshold = 757 mg/L |  |  |  |  |  |  |  |  |
| % Urban calculated from SEWRPC land use data. Draft Root River Restoration Plan, Table IV-3. |  |  |  |  |  |  |  |  |

### Data from SEWRPC’s Root River Restoration Plan (Draft Report Figure IV-23)

| Picture 1 |  |
| --- | --- |
| Near Ryan Road |  |

| Picture 2 |  |
| --- | --- |
| At Proposed Return Flow Location (60th Street Bridge) |  |