Big Muskego Lake, Wuakesha County (WBIC 762400)

Rhodamine WT Dye and Herbicide Sample Plan, 2016

John Skogerboe

Big Muskego Lake is 2194 acres, and has a maximum depth of 23 ft.  The lake is classified as a eutrophic, drainage lake in the WI DNR lake web site.  A copper formulation is proposed to be applied to three sites within the lake in summer, 2016 to control starry stonewort (Nitellopsis obtusa).  The proposed target application concentration is 1.0 mg/L metallic copper equivalent (mce) applied as Cutrine.  Rhodamine WT dye (dye) will also be applied along with the herbicide to provide more detailed information water dissipation rates within the treated areas and to evaluate potential herbicide movement into non target areas.

Dye will be applied at a target concentration of 10 ppb (0.05 L/acre-ft) along with the herbicides.

Sample grids will be established for each treatment area to evaluate dye movement and dissipation rates (Figure 1, Figure 2, and Figure 3).  Dye will be measured at 1 hour intervals beginning immediately after treatment ends (HAT) in each treatment area.

Water samples will be collected at sites and depths listed below for herbicide analysis (Figure 4, Figure 5, and Figure 6).  Each herbicide sample sites correspond to a dye sample site.  A map of the herbicide sample site locations is attached and the coordinates are provided.  Data are in degrees and the datum is WGS84.

| Big Muskego Herbicide Sample Sites |  |
| --- | --- |
| Site | Sample Depth |
| A2 | Integrated |
| A3 | Integrated |
| A5 | Integrated |
| A6 | Integrated |
| A7 | Integrated |
| B2 | Integrated |
| B3 | Integrated |
| B4 | Integrated |
| B5 | Integrated |
| B6 | Integrated |
| C16 | Integrated |
| C17 | Integrated |
| C22 | Integrated |
| C23 | Integrated |
| C29 | Integrated |

Samples will need to be collected at different time intervals throughout the project and are listed below.  Late sampling intervals may be eliminated when dye concentrations drop to trace or background levels.

| Big Muskego Herbicide Sample Intervals, |  |  |
| --- | --- | --- |
| Interval | Samples/interval | Total Samples |
| 1 HAT | 5 | 5 |
| 2 HAT | 5 | 10 |
| 3 HAT | 5 | 15 |
| 4 HAT | 5 | 20 |
| 6 HAT | 5 | 25 |
| 8 HAT | 5 | 30 |
| 24 HT | 5 | 35 |

HAT denotes hours after treatment

- Water samples can be collected using an integrated water sampler.  I can provide one if necessary.  Water is collected by pushing the pipe straight down to within 1 ft of the bottom or as far as it will go in deep water.  The pipe is brought to the surface and emptied into a bucket using a screw driver to push open the check valve .

- Samples should be collected at each point using a GPS at each interval listed above (Figure 7).  If you cannot collect a sample at the interval listed above please collect the sample as soon as reasonably possible and record the change.  Sample bottles and labels will be provided.  If you need a GPS please let us know.

- The SLOH will provide a sampling kit which will include bottles, labels, label pens, acid, pipettes, instructions, a shipping cooler, and sample handling instructions.

SLOH Sample Data Sheet:  The SLOH will provide an example data sheet for use when collecting water samples. It is important to use a separate data sheet for each day that you monitor.

The WI DNR will need to provide the following information for the sample data sheet which can be reprinted by the sampling volunteer for each sample interval (Figure 8):

- Account number

- DNR user number

- Sample matrix

- Project

- WBIC

- Test Request

- Sample site number

- Station ID

- Sample Depth

The sampling volunteers will need to provide the following on a new data sheet for each sample date:

- Sampling Date

- Sampling Time

- Water Temperature:  If water temperature is measured in F, then it should be converted to C using the table included on the data sheet. We suggest printing out the conversion table and bringing it in the field if your thermometer only reads in Fahrenheit.

- Wind Direction and Speed

If you have any questions, please call or email one of us.   We can be reached at the following:

John Skogerboe

jskogerboe@comcast.net

Cell Phone (651) 328-0769

Curtis Hedman, PhD

SLOH

curtis.hedman@slh.wisc.edu

Office (608) 224-6271

Scott Van Egeren

WI DNR

scott.vanegeren@dnr.state.wi.us

Office (608) 264-8895

Figure 1.  Hunters Nest Dye Sample Locations

(50 m x 50 m grid)

Figure 2.  Durham Launch Dye Sample Locations

(50 m x 50 m grid)

Figure 3.  Hunters Nest Dye Sample Locations

(25 m x 25 m grid)

Figure 4.  Hunters Nest Herbicide Sample Locations

Figure 5.  Durham Launch Herbicide Sample Locations

Figure 6.  Boxhorn’s Launch Herbicide Sample Locations

Figure 7.  Big Muskego Lake 2016 herbicide sample site coordinates

North                        West         Sample Site

Hunter’s Nest

42.897373                -88.094871           A2

42.896917                -88.094988           A3

42.896019                -88.095098           A5

42.895563                -88.095142           A6

42.895120                -88.095142           A7

Durham Launch

42.895190                -88.087918           B2

42.894737                -88.088390           B3

42.894278                -88.088815           B4

42.893825                -88.089269           B5

42.893377                -88.089657           B6

Boxhorn’s Launch

42.881642                -88.091649          C17

42.881417                -88.091656          C23

42.881637                -88.091956          C16

42.881430                -88.091978          C22

42.881194                -88.091649          C29

Figure 8.  Example of a State Lab of Hygiene Sample Data Sheet