Willow Creek Baseline Monitoring Report

Sheboygan River Basin

December 2006

###### John Masterson

###### Wisconsin DNR

###### TABLE OF CONTENTS

Abstract	3

Introduction	4

Materials and Methods	6

Results	7

Discussion	15

Conclusions	17

Recommendations	17

References	18

###### Figures

###### Figure 1	Willow Creek Watershed in Sheboygan County, Wisconsin.

Figure 2	Willow Creek sample site locations.

Figure 3	Willow Creek at WC 01 looking upstream.

Figure 4	Willow Creek at WC 02 looking upstream.

Figure 5	Willow Creek at WC 03 looking upstream.

###### Tables

###### Table 1	Site locations and informational summary for Willow Creek, Sheboygan County, Wisconsin.

###### Table 2	Ratings for individual habitat categories and overall habitat ratings for selected sites on Willow Creek.

Table 3	Number of individual fish species, IBI scores, and biological integrity ratings for selected sites on Willow Creek.

Table 4	Benthic macroinvertebrates, HBI scores, and water quality rankings for selected sites on Willow Creek.

Table 5	Fish community IBI scores, macroinvertebrate HBI rankings, and habitat ratings for Willow Creek.

###### ABSTRACT

The purpose of this study was to determine the water quality and biological integrity of Willow Creek in Sheboygan County.  Past land use disturbances and future development in the watershed have impacted and continue to threaten the condition of the stream.

The quality of the stream was determined by surveying fish and benthic macroinvetebrate communities and analyzing stream habitat.  Instantaneous water quality was also measured at each sample site.  Indices of biological integrity (IBI) were applied to fish and invertebrate data to evaluate biological integrity.

Willow Creek supported limited biological communities with IBI scores of good to very poor for benthic macroinvertebrate and fish communities.  Steam habitat ratings were fair to good.  Selected sites were degraded from nonpoint source runoff, excessive high and low flows, and stream channelization.

INTRODUCTION

The purpose of this study was to determine the water quality and biological integrity for portions of Willow Creek.  Little or no historical work has been done on the stream included in this report.  The assessment of Willow Creek was done using protocols for the Department’s baseline monitoring program.  This program annually selects streams to be monitored for biological, physical, and chemical parameters.  Willow Creek was chosen due to its unique location and course through areas of varying land use.

Assessment of the biological community was done with benthic macroinvertebrate and fish communities.  Stream habitat was also analyzed to determine how the physical conditions of the stream were affecting the biological community.  Water chemistry was not included in this study due to limited budget.  However, the biological and habitat data will support the need for future water chemistry analysis where applicable.

Willow Creek Watershed

Willow Creek is located within the boundaries of the City and Township of Sheboygan, Village of Kohler and headwater areas within the Township of Sheboygan Falls (Figure 1).  The stream is approximately 5.12 miles in length with a drainage basin of 4.22 square miles.  Soil types in the watershed are glacial in origin and primarily consist of clays and hydric soils.  Land use in the watershed is approximately 41% agricultural, 17% transportation, 16% open space, 15% residential, and 11% industrial/commercial.

There are portions of the headwaters that have been impacted from past development.  This includes filling of wetlands, straightening of the stream channel for flood control, storm sewer discharges, thermal impacts, nutrient and sediment loading from nonpoint source runoff, and diversion of groundwater discharge to the stream.

Poor water quality and low stream flow are several factors that can influence the type of fish community found in a stream.  In this basin, poor land use practices have degraded the water quality and biological integrity of Willow Creek.

Figure 1.  Willow Creek Watershed in Sheboygan County, Wisconsin.

METHODS

Three sample locations were selected for Willow Creek from 2002 through 2004 that were representative of habitat found in upper or lower sections of each stream reach.  Most sites were located upstream of the downstream most road crossing.  Table 1 list all past sample locations on Willow Creek for baseline monitoring.

Table 1.  Site locations and informational summary for Willow Creek, Sheboygan County, Wisconsin.

| Stream | Date | Location | USGS Quad Map | Order | Legal Description | Latitude/ Longitude* |
| --- | --- | --- | --- | --- | --- | --- |
| WC 01 | 09/03/03 | 125 m upstream of confluence with Sheboygan River | Sheboygan South | 2 | T15N R23E Sec.28 SWNE | 43°44.4506'N/ 87°44.8187'W |
| WC 02 | 08/13/02 | 50 m upstream of Greendale Road | Sheboygan Falls | 1 | T15N R23E Sec.28 NWNW | 43°44.6566'N/ 87°45.5641'W |
| WC 03 | 09/28/04 | 24 m upstream of Interstate 43 | Sheboygan Falls | 1 | T15N R23E Sec.20 SESE | 43°44.9443'N/ 87°45.7818'W |

* WGS 84 Datum

Discharge

Stream flow and depth were measured at equally spaced locations along one transect at each sample site using a Marsh-McBirney current meter.  Stream discharge in cubic feet per second (CFS) was calculated by multiplying velocity by depth by the distance between measurement points, and then summing the products along the transect (WDNR, 1998).

Habitat Assessment

Stream habitat was evaluated using quantitative procedures (Simonson et al. 1994) (WDNR 2002).  Variables measured included gross morphology, bottom substrate, plant growth, shading from vegetation, fish cover, stream bank erosion and surrounding land use.

### Biological Assessments

Biological assessment work included collecting stream benthic macroinvertebrates and fish to assess each stream’s water quality and biological integrity.  Invertebrates were collected with a D-frame net, identified to genus or species level, and data was applied to Hilsenhoff’s Biotic Index (HBI) which relates species diversity, abundance and organic pollution sensitivity to water quality (Hilsenhoff 1987).

Fish were sampled using electrofishing gear (backpack shocker) and standard protocols (WDNR 2001).  One upstream pass was made and all fish observed were collected with small nets.  Fish were identified and the number and weights of each species was recorded.  Fish that could not be identified in the field were placed into 10% formalin for later identification.

The Index of Biotic Integrity (IBI) for small streams was applied to the fish community data for each sample location (Lyons, 2006) (Lyons, 1992).  Similar to the HBI, IBI relates community structure to community health and water quality.

#### RESULTS

WC 01 (Upstream of confluence with Sheboygan River)

The survey location on WC 01 is located within the City of Sheboygan near the southwest corner of the intersection of Taylor Drive and County Highway “PP” (Figure 2).  This sample site was 125 meters upstream of the confluence with the Sheboygan River and averaged 2.6 meters in width and was 100 meters in length.  Here WC 01 has a gradient of 3.21 meters per kilometer, a sinuosity of 1 to 1.94 and is a 2nd order stream.  This reach of stream has a predominately woodland landscape with some adjacent agricultural fields (Figure 3).

Figure 2.  Willow Creek sample site locations.

### Discharge

Water level was estimated to be 0.1 m below normal for the date of the habitat survey and water clarity was clear.  Flow was 0.43 cubic feet per second (CFS) at this site.

Habitat

Runs were the dominate habitat features for this stream reach, with small percentages of pools and riffles.  The overall habitat ranking for the site was fair.  Individual features that were ranked included riparian buffer width, bank erosion, pool area, width:depth ratio, riffle:riffle and bend:bend ratio, fine sediments, and cover for fish (Table 2).

Mean distance for major habitat features was 20.2 meters for runs, 8.5 meters for pools, and 5.3 meters for riffles.  Stream bank erosion was a problem, with an average of 1.0 m of bare soil within 5 meters of the stream bank.  Fine sediment (sand, silt, and clay) accounted for 53 percent of the bottom substrate.  This may be from upstream bank erosion and nonpoint source pollution from urban and rural storm water runoff.  The riparian buffer width was 10 m average within 10 meters of the stream bank and was dominated by woodland.  Fish cover was very minimal (1%) and was primarily limited by shallow water depth.

Sand and gravel were the most abundant substrates encountered at this location. Other substrates, such as silt and cobble were also encountered at this site although they were less abundant.

Figure 3.  Willow Creek at WC 01 looking upstream.

Table 2.  Ratings for individual habitat categories and overall habitat ratings for selected sites on Willow Creek.

| Location | Riparian Buffer Width | Bank Erosion | Pool Area | Width:Depth Ratio | Riffle:Riffle Ratio | Bend:Bend Ratio | Fine Sediment | Fish Cover | Overall Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WC 01 | Excellent | Fair | Poor | Fair | Good | Poor | Fair | Poor | Fair |
| WC 02 | Good | Fair | Fair | Good | Excellent | Poor | Fair | Fair | Good |
| WC 03 | Excellent | Fair | Poor | Good | Poor | Poor | Fair | Poor | Fair |

Biological Assessment

Fish were collected during a single upstream sample pass of the survey section.  A total of 48 individual fish representing 6 species were captured at this location (Table 3). The Johnny Darter was the most abundant species captured.  The collected fish species resulted in an IBI score of 30 indicating a fair fish community at this warm water location on Willow Creek.

Benthic macroinvertebrates were not collected at this site (Table 4).

Table 3. Number of individual fish species, IBI scores, and biological integrity ratings for selected sites on Willow Creek.

| Species | WC 01 | WC 02 | WC 03 |
| --- | --- | --- | --- |
| Blacknose Dace (Western) | 4 | 13 |  |
| Bluegill |  | 1 |  |
| Brook Stickleback | 2 | 23 |  |
| Brook Trout |  | 2 |  |
| Central Mudminnow |  | 4 |  |
| Coho Salmon |  | 3 |  |
| Creek Chub |  | 22 |  |
| Longnose Dace |  | 3 |  |
| Johnny Darter | 35 |  |  |
| Mimic Shiner | 2 |  |  |
| Smallmouth Bass | 1 |  |  |
| White Sucker | 4 | 4 | 1 |
| Total Number | 48 | 75 | 1 |
| Total Species | 6 | 8 | 1 |
| IBI Score | 30 | 20 | 15 |
| Biological Integrity Rating | Fair | Poor | Very Poor |

Table 4. Benthic macroinvertebrates, HBI scores, and water quality rankings for selected sites on Willow Creek.

| Species | WC 01* | WC 02 | WC 03 |
| --- | --- | --- | --- |
| PLECOPTERA |  |  |  |
| Capniidae Allocapnia Unidentified |  | 21 |  |
| EPHEMEROPTERA |  |  |  |
| Baetidae Baetis brunneicolor flavistriga |  | 36 1 | 16 |
| Heptageniidae Stenacron interpunctatum |  | 13 |  |
| TRICHOPTERA |  |  |  |
| Hydropsychidae Cheumatopsyche Unidentified Hydropsyche betteni Ceratopsyche slossonae Limnephilidae |  | 7 2 2 1 | 8 78 1 |
| COLEOPTERA |  |  |  |
| Elmidae Optioservus Unidentified Fastiditus |  | 8 43 | 3 |
| DIPTERA |  |  |  |
| Empididae Hemerodromia Unidentified Tipulidae Tipula Unidentified Diamesinae Diamesa Unidentifed Orthocladiinae Eukiefferiella Claripennis Group Orthocladius Unidentified Tanytarsini Micropsectra Unidentified Rheotanytarsus Unidentified |  |  | 1 1 2 1 1 1 1 7 |
| AMPHIPODA |  |  |  |
| Gammaridae Gammarus psuedolimnaeus |  | 12 | 1 |
| ISOPODA |  |  |  |
| Asellidae Caecidotea intermedia |  | 13 | 144 |
| TROMBIDIFORMES |  |  |  |
| Sperchontidae |  |  | 1 |
| GASTROPODA |  |  |  |
| Physidae Physa Unidentified |  |  | 2 |
| Totals for HBI |  | 159 | 269 |
| HBI Scores |  | 4.513 | 6.891 |
| Water Quality Rank |  | Good | Fairly Poor |
| Total Individuals |  | 159 | 269 |
| Individual Species |  | 12 | 17 |

* Not sampled.

WC 02 (Upstream of Greendale Road)

The survey location on WC 02 is located within the Township of Sheboygan, 50 meters upstream of Greendale Road (Figure 2).  This sample site averaged 3.5 meters in width and was 125 meters in length.  Here WC 02 has a gradient of 8.5 meters per kilometer, a sinuosity of 1 to 1.2 and is a 1st order stream.  This reach of stream has a predominately woodland landscape with some urban development (Figure 4).

### Discharge

Water level was estimated to be normal for the date of the habitat survey and water clarity was clear.  Flow was 0.70 cubic feet per second (CFS) at this site.

Habitat

Runs were the dominate habitat features for this stream reach, with an excellent ratio of riffles and a fair percentage of pools.  The overall habitat ranking for the site was good.  Individual features that were ranked included riparian buffer width, bank erosion, pool area, width:depth ratio, riffle:riffle and bend:bend ratio, fine sediments, and cover for fish (Table 2).

Mean distance for major habitat features was 11.5 meters for runs, 6.9 meters for riffles, and 4.4 meters for pools.  Stream bank erosion was a problem, with an average of 1.0 m of bare soil within 5 meters of the stream bank.  Fine sediment (sand, silt, and clay) accounted for 33 percent of the bottom substrate.  This may be from upstream bank erosion and nonpoint source pollution from urban and rural storm water runoff.  The riparian buffer width was 9 m average within 10 meters of the stream bank and was dominated by woodland.  Fish cover was fair (6%) and was limited to woody debris and boulders.

Gravel and sand were the most abundant substrates encountered at this location. Other substrates, such as cobble, silt, and detritus were also encountered at this site although they were less abundant.

Biological Assessment

Fish were collected during a single upstream sample pass of the survey section.  A total of 75 individual fish representing 8 species were captured at this location (Table 3).  Brook Stickleback and Creek Chubs were the most abundant species captured.  It should be noted that Brook Trout and young-of-year Coho Salmon were found in this stream reach.  This salmonid population in an urbanized stream has triggered additional investigations of Willow Creek.  The collected fish species resulted in an IBI score of 20 indicating a poor fish community for this coldwater reach of Willow Creek.

Invertebrates were collected from riffles within this stream site.  Results for the HBI index scored 4.513 for a water quality ranking of good (Table 4).  Elmidae, Baetidae, and Capniidae dominated the benthic community, respectively.

Figure 4.  Willow Creek at WC 02 looking upstream.

WC 03 (Upstream of Interstate 43)

The survey location on WC 03 is located within the Village of Kohler, 24 meters upstream of Interstate 43 (Figure 2).  This sample site averaged 2.4 meters in width and was 100 meters in length.  Here WC 03 has a gradient of 4.1 meters per kilometer, a sinuosity of 1 to 1.14 and is a 1st order stream.  This reach of stream has a predominately meadow and shrub landscape with some urban development (Figure 5).  This area of the stream has been channelized (100 %) for an extensive reach upstream.

### Discharge

Water level was estimated to be normal for the date of the habitat survey and water clarity was clear.  Flow was not collected at this site.

Habitat

Runs were the only habitat features for this stream reach.  The overall habitat ranking for the site was fair.  Individual features that were ranked included riparian buffer width, bank erosion, pool area, width:depth ratio, riffle:riffle and bend:bend ratio, fine sediments, and cover for fish (Table 2).

Mean distance for major habitat features was 100 meters for runs.  Stream bank erosion was a problem, with an average of 1.0 m of bare soil within 5 meters of the stream bank.  Fine sediment (sand, silt, and clay) accounted for 52 percent of the bottom substrate.  This may be from upstream bank erosion and nonpoint source pollution from urban and rural storm water runoff.  The riparian buffer width was 10 m average within 10 meters of the stream bank and was dominated by meadow and shrubs.  Fish cover was poor (3%) and was limited to overhanging vegetation.

Sand, silt and detritus were the most abundant substrates encountered at this location. Other substrates, such as gravel, clay, and cobble were also encountered at this site although they were less abundant.

Biological Assessment

Fish were collected during a single upstream sample pass of the survey section.  A total of one (1) individual fish representing one (1) species was captured at this location (Table 3).  The one fish species present at the time of the survey was White Sucker.  The collected fish species resulted in an IBI score of 15 indicating a very poor fish community for this warm water reach of Willow Creek.

Invertebrates were collected from riffles within this stream site.  Results for the HBI index scored 6.891 for a water quality ranking of fairly poor (Table 4).  Asellidae and Hydropsychidae dominated the benthic community, respectively.

Figure 5.  Willow Creek at WC 03 looking upstream.

#### DISCUSSION

A combination of biological, chemical and physical data will show trends of watershed health better than an individual metric.  However, chemical monitoring was very limited for this project and included instantaneous temperature, dissolved oxygen and % saturation, pH, total dissolved solids, turbidity and specific conductance.  Biological samples were collected as a part of this study which included macroinvertebrates for the Hilsenhoff’s Biotic Index (HBI) and fish for the Index of Biotic Integrity (IBI).  Habitat assessments were also conducted at each site.  Stream flow was measured and discharge was calculated for each location. Through this sampling effort, we hope that the collected data will be used to help make management decisions that will protect and improve the condition of these streams where needed.

### Discharge

Measured flow and the calculated discharges during the study most likely represented normal flow conditions for Willow Creek, with the exception of WC 01 in 2003, when drought conditions had water levels below normal.  Lower than normal flows can be a limiting factor to biological communities, especially in low order streams.

Habitat

Three types of habitat were measured during the survey, which included large-scale stream features: runs, pools and riffles; small-scale habitat: bottom type and fish cover; and adjacent land use: buffer type and width.

A variety of bottom habitat features are necessary if a diverse biological community is to exist at a particular stream location. Riffles are locations that support a mixture of life and are locations where stream waters are oxygenated. Pools provide cover and habitat for fish during summer and winter months and runs can be feeding or spawning locations for a number of fish species.  The lack of water depth and limited fish cover may limit both the type of species that are present and the size of adult fish.

Bare soil and stream bank erosion were evident at all three sites.  Some of the bare soil was likely caused by spring flooding and high stream flows.  Eroded banks and litter above the stream banks indicate that Willow Creek does have high, flash flows during storm events.

While restructuring of bottom sediments by natural causes such as flooding is normal and can be healthy for streams, sedimentation from anthropogenic source, such as nonpoint source runoff, can be detrimental to stream organisms and steps should be taken to reduce or eliminate these sediment inputs to the stream.  Land use planning for future development and implementing best management practices throughout the watershed could be very useful in reducing pollutant loading and improving stream habitat.

The variety in bottom substrate has allowed a diverse community of invertebrate and fish species to exist in Willow Creek. This was reflected in HBI and IBI scores at each site (Table 5).

Riparian corridors for most streams were well buffered with 10 meters of wooded, wetland, or meadow buffers at most locations.

Table 5.  Fish community IBI scores, macroinvertebrate HBI rankings, and habitat ratings for Willow Creek.  NS = Not Sampled.

| Location | Year of Survey | Fish IBI | Benthic Macroinvertebrate HBI Water Quality Ranking | Habitat Rating |
| --- | --- | --- | --- | --- |
| WC 01 | 2003 | Fair | NS | Fair |
| WC 02 | 2002 | Poor* | Good | Good |
| WC 03 | 2004 | Very Poor | Fairly Poor | Fair |

* Cold water IBI Score.  Others are warm water.

Biological Assessments

Results from the macroinvertebrate samples and the HBI showed fairly poor to good water quality at the two sample locations. Low flow and poor habitat may account for poor ratings at the upstream location (WC 03).   The presence of stoneflies (Capniidae) at the coldwater section of Willow Creek indicates that this section has good water quality.  Stoneflies are very sensitive to pollution and are often the first invertebrates to disappear following environmental disturbance.

Fish IBI ratings ranged from fair to very poor.  Willow Creek did not support large numbers of fish at the times of sampling.  Low flows and lack of fish cover are likely factors limiting higher numbers of fish.  Lack of fish may also be due to swings in dissolved oxygen levels, or from habitat loss caused by channelization, streambank erosion and sedimentation, especially in the upper half of Willow Creek, which is located upstream of Interstate 43 (I-43).  The culverts under I-43 may also be a barrier for fish passage and may explain the very low numbers of fish at WC 03.

The portion of Willow Creek downstream of I-43 did support a small salmonid population of adult Brook Trout and young-of-year Coho Salmon.  There appears to be evidence that the Coho Salmon are successfully reproducing in Willow Creek during their annual spawning from Lake Michigan.  Rainbow Trout and Chinook Salmon have also been observed in Willow Creek, but the success of their spawning run has yet to be determined.

Water Quality

Temperature, DO, pH and specific conductance measurements along with invertebrate and fish sampling were used to evaluate water quality on the monitored streams. Instantaneous water quality monitoring was done using a Hydrolab. Wisconsin Administrative Code NR 102 establishes water quality standards to maintain favorable aquatic life.  For fish and aquatic life classified streams (warm water), dissolved oxygen content may not be lowered to less than 5 mg/l at any time, water temperature shall not exceed 89ºF (31.7ºC), and pH shall be within the range of 6.0 to 9.0.  Water quality parameters were within the normal range for temperate streams.

CONCLUSIONS

- The absence or low numbers of fish from these streams may be due to seasonal low flow conditions or from periodic poor water quality caused by non-point source pollution.  Drought conditions prior to the project surveys did cause lower than normal water levels at the time of sampling at WC 01 in 2004.

- Water quality on the surveyed streams is fairly poor to good based on macroinvertebrate HBI scores.

- The riparian corridors of the three sample locations on Willow Creek are well buffered.  However, each site does exhibit stream bank erosion and deposition of soft sediments.  Land use planning and the implementation of best management practices throughout the watershed may protect and enhance portions of the stream.

- Stream habitat is in good to fair condition at the sample locations.  However, benthic macroinvertebrate and fish communities seem to be degraded based on biotic indices.

RECOMMENDATIONS

- Continue to monitor Willow Creek to fully understand its water quality and biological potential.

- Partner with local conservation groups and municipalities for smart growth planning for future development in the watershed.

- Further investigate the cold water reaches of Willow Creek to protect and eventually improve these unique resources.

- Implement best management practices and stream improvement projects on Willow Creek to improve its water quality and biological integrity.

- Continue information and educational efforts to gain attention to this significant resource.

REFERENCES

Hilsenhoff, W.L.  1987.  An Improved Biotic Index of Organic Stream Pollution.  The

Great Lakes Entomologist, 20(1):31-39.

Lyons, J.  2006.  A Fish-Based Index of Biotic Integrity To Assess Intermittent

Headwater Streams in Wisconsin, USA.  Environmental Monitoring and Assessment (2006) 122: 239-258.

Lyons, J.  1992.  Using the Index of Biotic Integrity (IBI) to Measure Environmental

Quality in Warmwater Streams of Wisconsin. U.S. Department of Agriculture, Forest Service, North Central Forest Experiment Station. General Technical Report NC-149. St. Paul, Minnesota.

Simonson T. D., J. Lyons and P.D. Kanehl. 1994. Guidelines for Evaluating Fish Habitat

in Wisconsin Streams. U.S. Department of Agriculture, Forest Service, North Central Forest Experiment Station, General Technical Report NC-164.

WI DNR.  1998.  WDNR Field Procedures Manual – Open Channel Flow Measurement.

WDNR.  Madison, WI.  11 pages.

WI DNR.  2001.  Guidelines for Assessing Fish Communities of Wadable Streams in

Wisconsin.  WDNR.  Madison, WI.  12 pages.

WI DNR.  2002.  Wisconsin Department of Natural Resources Guidelines for Evaluating

Habitat of Wadable Streams. Wisconsin Department of Natural Resources. May 2000 revision. 22 pages.