MIDDLE EAU CLAIRE LAKE AQUATIC PLANT MANAGEMENT (APM) SUMMARY

Goals and Objectives

  - Goal:  reduce the Curlyleaf pondweed population and prevent the spread within and beyond the lake.

  - Objective:  Curlyleaf pondweed will not exceed 1% FOO or 1 acre total area.

Waterbody Description

Middle Eau Claire Lake is a 880 acre mesotrophic lake located in Bayfield County.  It has a maximum depth of 66 feet, with a mean depth of 17 feet and a total shoreline of 11 miles.  The bottom is 80% sand, 5% gravel, 0% rock, and 15% muck.  The water is moderately clear and has a total alkalinity of 34PPM.  Visitors have access to the lake from a public boat landing.  Fish include Musky, Panfish, Largemouth Bass, Smallmouth Bass, Northern Pike and Walleye.

Water Use Intensity

Recreational use is moderate to high for a northern Wisconsin lake, although intensity is limited because of it’s distance from urban areas.  The nearest urban areas are Hayward, 20 miles south, and Duluth/Superior 45 miles northwest.  There are no public drinking water systems or discharges to the lake.

Management Locations

Curly-leaf pondweed is widespread in the southern half of the lake with a small isolated patch along the northeastern shore (Figure 1).  The total estimated cumulative surface area is less than one acre.

Evaluation of Alternative Methods

Since the locations are small beds nested within a thriving native plant community, manual/mechanical removal is the best management strategy.  Chemical control is unlikely to be effective because spot treatments tend to have minimal concentration exposure times resulting in limited control; plus native pondweeds could be harmed.  There is no biological control option for curlyleaf pondweed, and physical control is not feasible or likely to be permitted.

Integrated APM Strategy

Continuous volunteer monitoring and annual professional monitoring will measure the success of the BAISS strategy.  Manual removal will be encouraged.  If monitoring indicates our objectives are not being met, alternative methods, including chemical control will be revisited.

Figure 1:  2015 Curly-leaf pondweed locations in Middle Eau Claire Lake, Bayfield County.