*** NORTHEAST DISTRICT BIOTIC INDEX REPORT ***

Sample: 20080915-31-04 (yyyymmdd-Co-Field#)                 Reference Site: NO

Waterbody:UNNAMED TRIB. TO EAST TWIN R  Master Waterbody #: 3000212

Project: TREGA STREAM CLASSIFICATION    Storet Station:

Collector: BEHNCKE, JEFF/RIEBE, JUSTIN  Time Sampling: 3.0            # Reps: 1

Sampling Device: 1. D Frame

Taxonomist: DIMICK, JEFFREY

Sorter: JACKSON, JEREMY                    % Sample Sorted: 13

SWIMS ID:10029041            Name:                               Key:         0

===============================================================================

Location-Legal:       S 0 T 0N R 0    County: KEWAUNEE

Lat/Long: N44deg 25min 44.6sec  W 87deg 40min 47.4sec

Method: GARMIN GPS

Description: CHERNYVILLE RD.

===============================================================================

Width  Depth  Temp  DO   Measured Est Velocity

HBI    ft     ft     øC  mg/l  Vel fps     fps              Habitat

General        4.3    0.7    24.3 0.88           Slow     (0.2-0.5)  2. Run

Rep #1   9.948

Rep #2   0.000

Rep #3   0.000

Rep #4   0.000

Rep #5   0.000

Aquatic Vegetation   0 % of Total Stream Channel at Sampling Site

===============================================================================

Substrate at Rep Location %:

Bedrock Boulders Rubble Gravel Sand Silt Clay Detritus Muck Debris/Veg

Rep #1   0.0      0.0     0.0    0.0   0.0  0.0  0.0    0.0    0.0    0.0

Rep #2   0.0      0.0     0.0    0.0   0.0  0.0  0.0    0.0    0.0    0.0

Rep #3   0.0      0.0     0.0    0.0   0.0  0.0  0.0    0.0    0.0    0.0

Rep #4   0.0      0.0     0.0    0.0   0.0  0.0  0.0    0.0    0.0    0.0

Rep #5   0.0      0.0     0.0    0.0   0.0  0.0  0.0    0.0    0.0    0.0

Substrate Sampled %:  (Same as Above  NO )

Bedrock Boulders Rubble Gravel Sand Silt Clay Detritus Muck Debris/Veg

Rep #1   0.0      0.0     0.0   40.0  20.0 10.0  0.0   30.0    0.0    0.0

Rep #2   0.0      0.0     0.0    0.0   0.0  0.0  0.0    0.0    0.0    0.0

Rep #3   0.0      0.0     0.0    0.0   0.0  0.0  0.0    0.0    0.0    0.0

Rep #4   0.0      0.0     0.0    0.0   0.0  0.0  0.0    0.0    0.0    0.0

Rep #5   0.0      0.0     0.0    0.0   0.0  0.0  0.0    0.0    0.0    0.0

===============================================================================

NP=Not Present         Pollutant Sources:

Water Quality Indicators:       I=Insignificant     Livestock Pasturing    NP

Turbidity            NP         S=Significant       Barnyard Runoff         *

Chlorine/Toxic Scour  *                             Cropland Runoff         *

Macrophytes          NP  Factors Affecting Habitat: Tile Drains             *

Filamentous Algae    NP   Sludge Deposits      *    Septic Systems          *

Planktonic Algae      *   Silt and Sediment   NP    Streambank Erosion      *

Slimes                *   Channel Ditching     S    Urban Runoff            *

Iron Bacteria        NP   Impoundments         *    Construction Runoff    NP

Perceived Water Quality   Low Flows            *    Point Source (Spec)     *

at Site:                Wetlands             *    Other (Specify)         *

Comments:                 Comments:                 Comments:

UPSTREAM & LOCAL

CHANNELIZATION

DISSOLVED OXYGEN

*** NORTHEAST DISTRICT BIOTIC INDEX REPORT ***

SAMPLE ID#   20080915-31-04                                  PAGE 1

***   TAXA   ***        SPECIES TAXA  TOL ORGANISM    ORGANISM

STATUS KEY   VAL    ID        COUNT

CODE  USED               REP1 REP2 REP3

ODONATA

LIBELLULINAE

PLATHEMIS

LYDIA                        *1  8.00 03090601    1    0    0

DIPTERA

**UNIDENTIFIED**

*2       08000200    1    0    0

TIPULIDAE

HELIUS

**UNIDENTIFIED**             *3  4.00 08140400    1    0    0

CHIRONOMIDAE

**LARVAE**                   *3       08250001    1    0    0

CHIRONOMINI

CHIRONOMUS

**UNIDENTIFIED**             *4 10.00 08324400  151    0    0

**PUPAE**                    *5       08324401    1    0    0

TROMBIDIFORMES                     *6       11000000    1    0    0

OLIGOCHAETA

TUBIFICIDAE                      *7       16030000    9    0    0

*** TOTALS: ***            166

0

0

*** BIOTIC INDEX: ***    9.948

Taxanomic Key Code Ref:          Species Status Codes:

*1 Needham et al. 2000

*2 Merritt et al. 1996

*3 Hilsenhoff 1995

*4 Epler 2001

*5 Coffman, Ferring. 96

*6 Smith, Cook 1991

*7 Brinkh., Gelder 1991