Presence of Selected Endocrine Active Chemicals and Pharmaceuticals in Water and Sediment from the Saint Louis Bay, Wisconsin and Minnesota

## July 30, 2010

## Revised August 13, 2010

US Geological Survey, Mounds View, MN

Wisconsin Department of Natural Resources

# SUMMARY

The St. Louis Bay of Lake Superior receives substantial urban runoff and waste-water treatment plant effluent. Previous studies indentified St. Louis Bay as having the highest number of detections for emerging contaminants, such as pharmaceuticals and antibiotics. Water and sediment will be sampled from 10 sites in the St. Louis River, St. Louis Bay, and Superior Bay.  Samples will be analyzed for pharmaceuticals, hormones, and other waste compounds in order to increase understanding of the persistence and fate of these emerging contaminants.

# INTRODUCTION

Synoptic surveys of surface water quality across the United States have detected a large group of widespread, emerging contaminants.  Pharmaceutical compounds including prescription drugs and antibiotics, over-the-counter medications, reproductive hormones, and an array of other compounds including personal-care products, detergent metabolites, and flame retardants have been identified in surface waters (Loper and others, 2007; Fono and others, 2006; Brown and others, 2006; Sando and others, 2005; Lee and others, 2004; Kolpin and others, 2002; Buser and others, 1999).  Streams receiving agricultural, municipal, and industrial wastewaters appear to be the most affected (Kolpin and others, 2002; Lee and others, 2004) but other sources have been identified including on-site septic systems (Carrara and others, 2008; Godfrey and others, 2007).  From a functional stand-point, these emerging contaminants include endocrine-active compounds (EAC) capable of endocrine disruption as well as pharmaceutically-active compounds (PhACs) designed for human or animal consumption.

When emerging contaminants enter the effluent stream and are discharged into surface or ground waters, most are found at very low concentrations in water (Kolpin and others, 2002; Lee and others, 2004) but reach higher concentrations in sediment (e.g., Mayer and others, 2006; Pojana and others, 2007; Kim and Calrson, 2007).  Despite these low concentrations, questions remain regarding the health of aquatic organisms under long-term exposure to water or sediment contaminated with functional PhACs and EACs.  In the case of EACs, current research suggests fish and wildlife can be affected at environmentally relevant concentrations, producing developmental and reproductive problems. For example, feminization of fish has been documented (Iguchi and others, 2001), and intersex fish have been found in areas with known EACs (Hinck and others, 2006; Woodling and others, 2006). Endocrine disruption has been observed in the effluent of large waste-water treatment plants (WWTPs), including vitellogenin (the egg yolk precursor protein found in female fish) induction in male carp (Cyprinus carpio) and walleye (Sander vitreus) (Folmar and others, 1996, 2001; Lee and others, 2000).  Vitellogenin in male carp also was observed at numerous sites downstream of WWTP discharges (Lee and others, 2000).  Taken together, the potential ecological disruption from PhACs and EACs is substantial, and our current understanding of their distribution incomplete.

# PROBLEM

Previous work has documented the occurrence of EACs, PhACs and other waste-water compounds downstream of WWTP. One of the earliest studies by Lee and others (2004) analyzed samples from 65 sites for the presence of 91 organic wastewater compounds (OWCs), including pharmaceuticals and antibiotics.  In that study, surface water samples from around the state of Minnesota (and in waters shared with the state of Wisconsin) identified 56 emerging contaminants from the 32 surface water sites.  Some of the more commonly detected compounds included caffeine, cholesterol, and acetaminophen. Sites located on small streams with an upstream WWTP had the highest average number of detections, but small streams draining urban land use without WWTP effluent contributions had a disproportionate number of OWC detections.  This pattern was repeated in the small number of lakes sites sampled in this study.  As was the case with the wadeable streams, the one lake site with significant urban runoff and WWTP effluent discharge had the highest number of detections of emerging contaminants.  This site was the St. Louis Bay of Lake Superior, along the Minnesota-Wisconsin border.

Whereas these studies established the presence of PhACs and EACs in selected streams and lakes and identified the presence of endocrine disruption in wild fish, only a small percentage of the stream and river miles in the states of Wisconsin and Minnesota have been sampled for these compounds and less than 20 lakes have been included in these studies.  In addition, little is known about the persistence and fate of these compounds downstream of  WWTP discharges, including the nature and extent of downstream transport or partitioning between water, biota, and sediment in complex hydrological settings like the St Louis Bay complex.  This proposal outlines the sampling plan for water and sediments in the St. Louis River, St. Louis Bay, and Superior Bay to help develop an understanding of the distribution of EAC and PhAC compounds in this freshwater estuary of Lake Superior.  The Minnesota USGS Water Science Center (MN-WSC) will collect and analyze water and sediment for pharmaceuticals, hormones, and other waste compounds.  Additional field parameters will be collected at each site including pH, temperature, specific conductance, and dissolved oxygen.  The overall goal of these assessments is to document the extent of the presence of emerging contaminants and to understand how the characteristics of depositional environments may contribute to the observed distribution of these compounds.

# OBJECTIVES AND SCOPE

The objectives of this study are to identify the extent to which pharmaceuticals, including hormones, and other waste compounds occur in the St. Louis River, St. Louis Bay, and Superior Bay and to what extent the compounds may have accumulated in surface sediments of the study area.  The sampling design will include 10 randomly selected sites in the St. Louis River, St. Louis Bay, and Superior Bay on the Wisconsin side of the border (fig. 1).  In addition, one quality assurance sample (either blank or duplicate) will be collected for a total of 11 sample sets. Figure 1 contains ten possible locations, with the final locations to be agreed upon by the USGS and Wisconsin DNR. Water and sediment samples will be analyzed for the compounds listed in Table 1.  Details for each group of compounds by laboratory schedule are listed in the table.  Approximately 200 separate chemical determinations will be completed at each site.  The hormones in sediment sampling schedule (Schedule 6434)  is currently unavailable, therefore those samples will be frozen and stored in the Minnesota WSC. They will be shipped and analyzed when the sampling schedule becomes available.

- Map of St. Louis River, St. Louis Bay, and Superior Bay and tentative sampling sites.

# RELEVANCE AND BENEFITS

This study will contribute to an integrated framework for understanding the distribution and fate of PhACs and EACs in rivers and lakes. The results of this analysis will be analyzed in conjunction with a similar study, occurring in cooperation with the Minnesota Pollution Control Agency (MPCA). The USGS-MPCA study results will include samples from 30 sites on the St. Louis River, St. Louis Bay, and Superior Bay (fig. 1). The combined information from these two studies should assist the states of Wisconsin and Minnesota in their efforts to assess the nature and scope of this issue.  It is anticipated that all stakeholders including states, tribes, watershed organizations and all citizens seeking information on aquatic contaminants will benefit from this information.

# APPROACH

## Sample Collection Methods

All samples will be collected using protocols and procedures to obtain a representative sample and avoid sample contamination. Specific protocols and methods are documented for the collection and processing of water-quality and sediment samples (U.S. Geological Survey, 2006).  All samples will be collected with inert materials such as Teflon, glass, or stainless steel. A multi-parameter probe will be used to measure field parameters (specific conductance, pH, water temperature, and dissolved oxygen) at each site.

All collection and processing equipment will be cleaned between samples with a succession of native water, soapy tap water, tap water, de-ionized water, methanol, and organic-free water rinses. To avoid contamination of samples, use of personal care items (such as insect repellent, sunscreen, cologne, aftershave, and perfume) will be avoided for personnel collecting and processing samples. Caffeinated products and tobacco products will not be consumed during (or immediately prior to) collection or processing of samples. Powder-less, disposable gloves will be worn during bottom-sediment sample collection to avoid contamination of samples. Standard labeling and packing techniques (for example, double foam sleeves placed around bottles) will be used to ensure sample integrity. When not in use, sample processing equipment will be covered.

## Surface Water Samples

Surface water samples will be collected using USGS methods for trace organic compounds, except that a churn will not be used.  It is anticipated that water velocities will be below the USGS threshold for isokinetic sampling of water quality constituents.  Therefore, water samples from each site will be collected as composites of multiple vertical sub-samples using an appropriate weighted sampler equipped with Teflon bottles.  Teflon bottle samplers will be cleaned and then rinsed with organic-free blank water before use.  One-liter amber-glass bottles will be filled with sub-samples and placed on ice until they are shipped to analytical laboratories.

## Sediment Samples

Bottom sediment will be collected once at each site using techniques to obtain the most recent bottom sediment deposition (top 5 cm of bottom sediment).  At least 5 bottom sediment samples will be composited for each sample site. Bottom-sediment samples will be collected according to established protocols (U.S. Geological Survey, 2006). Samples will be collected with a stainless steel Eckman grab sampler or other stainless steel coring equipment. The bottom-sediment sample will be discarded if it contains a large amount of vegetation or appears to be disturbed. Bottom-sediment samples will be transferred to a glass container and homogenized for 5 minutes, and 500 mL of wet material will be placed in wide-mouth, glass containers, chilled, and sent to the appropriate USGS Laboratory.

Briefly, the method used to analyze bottom sediment is an accelerated solvent extraction (ASE), using either Soxhlet extraction or pressurized solvent extraction (PSE), followed by solid-phase extraction (SPE) for sample preparation, with analysis by LC/mass spectrometry or capillary-column gas chromatography/mass spectrometry. Both methods identify and quantify individual compounds using retention times and spectral matches, along with standard calibration curves. Compounds included are a group of pharmaceuticals (see Table 1 below) and a broad group of emerging contaminants including alkylphenol ethoxylate nonionic surfactants and their degradates, food additives, fragrances, antioxidants, flame retardants, plasticizers, industrial solvents, and disinfectants. Interim method detection limits for the 61 individual compounds range from about 12 to 850 ug/kg. Many of the individual compounds have an interim method reporting level of 50 ug/kg. For more details see Table 1.

# QUALITY-ASSURANCE (QA) PLAN

The USGS Minnesota District Quality Assurance Plan and the USGS National field manual (U.S. Geological Survey, 2010) will be used to guide data collection activities for the project. USGS guidelines and standard procedures for water and sediment collections will be used for this study to assure quality data collection.

All field personnel will be familiarized with study design and sampling protocols prior to field sampling or data processing to assure sample integrity. Laboratory quality-control samples will be used to validate analytical data. Field quality-assurance samples will be used to assess sample collection and processing. Laboratory quality-control samples include laboratory blanks and reagent spikes. Laboratory reagent blanks will be used to assess potential sample contamination. Most methods will have surrogate compounds added to samples prior to extraction to monitor method performance. Surrogates are chemicals that have similar properties to the analytes of interest, but do not interfere with quantitation of the compounds of interest.

The plan for QA samples assumes that the USGS-MPCA study will occur concurrently and thus take advantage of the same replicate and blank samples, with one additional blank or replicate sample collected concurrently with the 10 samples collected on the Wisconsin side of the border. Potential contamination of samples because of collection and sample processing will be assessed with field-blank samples. Field blanks will be prepared at the selected site prior to, or following, a scheduled field sample. Blank samples will be prepared by processing HPLC grade organic-free water through the same equipment used to collect and process field samples. Blank samples will be collected and analyzed to assess contamination introduced during sample collection and processing for sediment samples.  Replicate samples are used to determine variability of detection and concentration that result from sample processing techniques. Replicate samples consist of a split of the field sample so the field and replicate samples should be nearly equal in composition. Field blanks consisting of organic free blank water will be collected using the clean field equipment.  Additional quality assurance (QA) samples will be collected as described below.

# PRODUCTS AND OUTCOMES

The USGS will provide quarterly reports that summarize the data collection and analyses process through the study period. The USGS will provide data summaries to the Wisconsin DNR in electronic spreadsheets by May 15, 2011 for review.  A draft final report will be provided for review on or before December 31, 2011. A final interpretive report combining the results of this study (10 sites) with the USGS-MPCA study (30 sites) will be delivered by June 15, 2012.

# REFERENCES

Barber, L. B., Brown, G. K., Zaugg, S. D., 2000, Potential endocrine disrupting organic chemicals in treated municipal wastewater and river water: in Keith, L.H., Jones-Lepp, T.L., and Needham, L.L., eds., Analysis of Environmental Endocrine Disruptors, American Chemical Society Symposium Series 747, Washington DC, 2000; pp 97-123.

Barber, L.B., Lee, K.E., Swackhamer, D.L., and Schoenfuss, H.L. ( in review), Endocrine Disruption in Male Fathead Minnows Exposed to Municipal Wastewater Treatment Plant Effluent: Removal of Endocrine Responses through XAD8 filtration.

Brown, K. D., Kulis, J., Thomson, B., Chapman, T. H., and Mawhinney, D. B.. 2006, Occurrence of antibiotics in hospital, residential, and dairy effluent, municipal wastewater, and the Rio Grande in New Mexico, Science of the Total Environment, vol. 366, pp. 772–783.

Buser, H.R., Poiger, T., and Muller, M.D., 1999, Occurrence and environmental behavior of the chiral pharmaceutical drug ibuprofen in surface waters and in wastewater.  Environmental Science and Technology, vol. 33, p. 2529-2535.

Carrara, C., Ptacel, C.J., Robertson, W. D., Blowes, D. W., Moncur, M. C., Sverko, E., and Backus,S., 2008, Fate of pharmaceutical and trace organic compounds in three septic system plumes, Ontario, Canada. Environmental Science and Technology, vol. 42, pp. 2805–2811.

Folmar, L. C., Denslow, N. D., Kroll, K., Orlando, E. F., Enblom, J., Marcino, J., Metcalfe, C., Guillette, L. J. Jr., 2001, Altered serum sex steroids and vitellogenin induction in walleye (Stizostedion vitreum) collected near a metropolitan sewage treatment plant. Arch. Environ. Contam. Toxicol., vol. 40, pp. 392-398.

Folmar, L. C., Denslow, N. D., Rao, V., Chow, M., Crain, D. A., Enblom, J., Marcino, J., Guillette, L. J. Jr., 1996, Vitellogenin induction and reduced serum testosterone concentrations in feral male carp (Cyprinus carpio) captured near a major metropolitan sewage treatment plant. Environ. Health Perspect, vol. 104, pp. 1096-1101.

Fono, L.J., Kolodziej, E.P., and Sedlak, D.L., 2006, Attenuation of wastewater-derived contaminants in an effluent-dominated river.  Environmental Science and Technology, vol. 36, no. 6, pp. 7257-7262.

Glassmeyer, S.T., Furlong, E.T., Kolpin, d.W.,Cahill, J.D., Zaugg, A.D., Werner, S.L., Meyer, M.T., and Kryak, D.D., 2005, Transport of chemical and microbial compounds from known wastewater discharges: potential for use as indicators of human fecal contamination: Environmental Science and Technology, vol. 39, no. 14, pp. 5157 -5169.

Godfrey, E., Woessner, W. W., and Benotti, M. J., 2008, Pharmaceuticals in on-site sewage effluent and ground water, Western Montana. Ground Water, vol. 45, pp. 263–271.

Goni-Urriza, M., Capdepuy, M., Arpin, C., Raymond, N., Cau-mette, P., and Quentin, C., 2000, Impact of an urban effluent on antibiotic resistance of riverine Enterobacteriaceae and Aeromonas spp. Applied and Environmental Microbiology, vol. 66, pp. 125-132.

Hinck, J.E., Blazer, V.S., Denslow, N.D., Gross, T.S., Echols, K.R., Davis, A.P., May, T.W., Orazio, C.E., Coyle, J.J., and Tillet, D.E., 2006, Biomonitoring of environmental status and trends program—Environmental contaminants, health indicators, and reproductive biomarkers in fish from the Col-orado River Basin: U.S. Geological Survey Scientific Inves-tigations Report 2006—5163, 119 p.

Iguchi, T., Watanabe, H., and Katsu, Y., 2001, Developmental effects of estrogenic agents on mice, fish and frogs—A mini review.  Hormonal Behavior, vol. 40, pp. 248-251.

Kim, S.C., Carlson, K.H., 2007. Quantification of human and veterinary antibiotics in water and sediments using SPE/LC/ MS/MS method. Anal. Bioanal. Chem., vol. 387, pp. 1301-1315.

Kolpin, D. W., Furlong, E. T., Meyer, M. T., Thurman, E. M., Zaugg, S. D., Barber, L. B., Buxton, H. T., 2002, Pharmaceuticals, hormones, and other organic wastewater contaminants in U.S. streams, 1999-2000: A national reconnaissance. Environ. Sci. Technol., vol. 36, pp. 1202-1211.

Kostich, M.S. and  Lazorchak, J. M., 2008, Risks to aquatic organisms posed by human pharmaceutical use: Science of The Total Environment, vol 389,  no, 2-3, pp. 329-339.

Lee, K. E., Barber, L. B., Furlong, E. T., Cahill, J. D., Kolpin, D. W., Meyer, M. T., Zaugg, S. D., 2000-02, Presence and distribution of organic wastewater compounds in wastewater, surface, ground, and drinking waters, Minnesota, 2000-02: U.S. Geological Survey Scientific Investigation Report 2004--5138, 47 p.

Lee, K. E., Blazer, V. S., Denslow, N. D., Goldstein, R. M., Talmage, P. J., 2000, Use of biological characteristics of common carp (Cyprinus carpio) to indicate exposure to hormonally active agents in selected Minnesota streams, 1999: U.S. Geological Survey Water-Resources Investigations Report 00—4202, 47 p..

Lee, K.E., Blazer, V., 2005, Endocrine disruption  in Minnesota fish—A  comparison of fish species:  Cyprinus carpio and Micropterus, Society of Environmental Toxicology and Chemistry, Madison, Wisconsin, vol. 136, pp 381-388.

Loper, C.A., Crawford, J.K., Otto, K.L., Manning, R.L., Meyer, M.T., and Furlong, E.T., 2007, Concentrations of selected pharmaceuticals and antibiotics in south-central Pennsylvania waters, March through September 2006: U.S. Geological Survey Data Series 300, 101 p.

Mayer, T., Bennie, D., Rosa, F., Rekas, G. , Palabrica, V., and Schachtschneider, J., 2007,  Occurrence of alkylphenolic substances in a Great Lakes coastal marsh, Cootes Paradise, ON, Canada. Environmental Pollution, vol. 147, pp 683-690.

Pojana, G., Gomiero, A., Jonkers, N., and Marcomini, A., 2007,  Natural and synthetic endocrine disrupting compounds (EDCs) in water, sediment and biota of a coastal lagoon: Environment International, vol. 33, pp. 929–936.

Sando, S.K., Furlong, E.T., Meyer, M.T., and Bartholomay, R.C., 2005, Occurrence of organic wastewater compounds in wastewater effluent and the Big Souix River in the Upper Big Souix River Basin, South Dakota, 2003-2004:U.S. Geological Survey Scientific Investigations Report 2005-5249, 108 p.

Schoenfuss, H. L., 2003, The need for novel approaches in assessing the biological impact of biologically active compounds.,  in Proceedings of the 3rd International Conference on Pharmaceuticals and Endocrine Disrupting Chemicals in Water, R. Masters (ed.), Minneapolis, MN, March 19-21, 2003, pp. 103-121.

Schoenfuss, H. L., Levitt, J. T., Van Der Kraak, G., Sorensen, P. W., 2002, Ten-week exposure to treated sewage discharge has relatively minor, variable effects on reproductive behavior and sperm production in goldfish. Environ. Toxicol. Chem., vol. 21, pp. 2185-2190.

Schoenfuss, H. L., Martinovic, D., Sorensen, P. W., 2001, Effects of exposure to low levels of water-borne 17ß-estradiol on nest holding ability and sperm quality in fathead minnows: Water Resources Update vol., 120, pp. 49-55.

U.S. Geological Survey, 2006, National field manual for the collection of water-quality data: Techniques of Water-Resources Investigations, Book 9, (http://water.usgs.gov/owq/FieldManual/), variously paged.

Woodling, J.D., Lopez, E.M., Maldonado, T.A., Norris, D.O., and Vajda, A.M., 2006, Intersex and other reproductive disruption of fish in wastewater effluent dominated Colorado streams.  Comparative Biochemistry and Physiology, Part C, vol. 144, p. 10-15.

# WORK PLAN

The project period is June 2010 through June 2012.  Sampling is scheduled to occur in mid to late summer during August, 2010.  Laboratory analysis and database entry will occur September 2010 through May 2011.  A draft data report will be provided to Wisconsin DNR and collaborators by December 31, 2011, with a completed final report delivered by June 15, 2012.

# PERSONNEL

Personnel for this project will include a lead hydrologic technician and students to complete the field portion of this study. A certified boat operator will be used for all boat sites. A hydrologist will serve as project chief. A hydrologist and student will provide data analysis and report writing. Science support personnel also will be assigned as necessary. This includes discipline specialists, editors, and technical reviewers.

# BUDGET

Budget estimates below benefit from an economy of scale, in that the USGS-MPCA study refered to in the Relevance and Benefits section will take place concurrently. If the sampling for the two studies does not take place at the same time, the budget estimates below will need to be adjusted, to account for additional travel and personnel time.

Total Budget Estimate for Project

| Category | Budget |
| --- | --- |
| Salary and Benefits | $9,000 |
| Travel | $1,000 |
| Analytical | $38,500 |
| Supplies | $600 |
| Subtotal - Direct Costs | $49,100 |
| Indirect Costs - Facilities and Assessments | $29,000 |
| Total | $78,100 |

Fiscal Year Break-down Estimates

| Category | FY ending 6/30/2011 | FY ending 6/30/2012 | Total |
| --- | --- | --- | --- |
| Salary and Benefits | $2,000 | $7,000 | $9,000 |
| Travel | $1,000 | $0 | $1,000 |
| Analytical | $38,500 | $0 | $38,500 |
| Supplies | $600 | $0 | $600 |
| Subtotal - Direct Costs | $42,100 | $7,000 | $49,100 |
| Facilities and Assessments | $24,800 | $4,200 | $29,000 |
| Total | $66,900 | $11,200 | $78,100 |

- Analytical schedules.

| Analyte | USGS Parameter Code | CAS Number | Reporting Level | Unit | RL Type |
| --- | --- | --- | --- | --- | --- |
| Schedule 1433 - Dissolved Wastewater Compounds in Water |  |  |  |  |  |
| 11-Ketotestosterone | 64507 | 564-35-2 | 2 | ng/L |  |
| 17-alpha-Estradiol | 64508 | 57-91-0 | 0.8 | ng/L |  |
| 17-alpha-Ethynylestradiol-2,4,16,16-d4 | 90813 | 350820-06-3 |  | pct |  |
| 17-beta-Estradiol | 64510 | 50-28-2 | 0.8 | ng/L |  |
| 17-beta-Estradiol-13,14,15,16,17,18-13C6 | 91753 |  |  | pct |  |
| 17-alpha-Ethynylestradiol | 64509 | 57-63-6 | 0.8 | ng/L |  |
| Norethindrone | 64511 | 68-22-4 | 0.8 | ng/L |  |
| 4-Androstene-3,17-dione | 64513 | 63-05-8 | 0.8 | ng/L |  |
| cis-Androsterone | 64515 | 53-41-8 | 0.8 | ng/L |  |
| Bisphenol-A-d16 | 67308 | 96210-87-6 |  | pct |  |
| Cholesterol | 64514 | 57-88-5 | 200 | ng/L |  |
| Cholesterol-d7 | 90778 | 83199-47-7 |  | pct |  |
| 3-beta-Coprostanol | 64512 | 360-68-9 | 200 | ng/L |  |
| Dihydrotestosterone | 64524 | 521-18-6 | 4 | ng/L |  |
| Bisphenol A | 67304 | 80-05-7 | 200 | ng/L |  |
| Epitestosterone | 64517 | 481-30-1 | 4 | ng/L |  |
| Equilenin | 64518 | 517-09-9 | 2 | ng/L |  |
| Equilin | 64519 | 474-86-2 | 4 | ng/L |  |
| Estriol | 64520 | 50-27-1 | 2 | ng/L |  |
| Estrone | 64521 | 53-16-7 | 0.8 | ng/L |  |
| Estrone-13,14,15,16,17,18-13C6 | 91754 |  |  | pct |  |
| Mestranol | 64522 | 72-33-3 | 0.8 | ng/L |  |
| Mestranol-2,4,16,16-d4 | 90821 |  |  | pct |  |
| Progesterone | 64523 | 57-83-0 | 8 | ng/L |  |
| trans-Diethylstilbestrol | 64516 | 56-53-1 | 0.8 | ng/L |  |
| Testosterone | 64525 | 58-22-0 | 0.8 | ng/L |  |
| trans-Diethyl-1,1,1',1'-d4-stilbesterol-3,3',5,5'-d4 | 90817 | 91318-10-4 |  | pct |  |
| Schedule 2080 - Dissolved Human Health Pharmaceuticals in Water |  |  |  |  |  |
| Cotinine | 62005 | 486-56-6 | 0.038 | ug/L |  |
| 1,7-Dimethylxanthine (p-Xanthine) | 62030 | 611-59-6 | 0.1 | ug/L |  |
| Acetaminophen | 62000 | 103-90-2 | 0.12 | ug/L |  |
| Albuterol (Salbutamol) | 62020 | 18559-94-9 | 0.08 | ug/L |  |
| Sulfamethoxazole | 62021 | 723-46-6 | 0.16 | ug/L |  |
| Caffeine | 50305 | 58-08-2 | 0.06 | ug/L |  |
| Carbamazepine | 62793 | 298-46-4 | 0.06 | ug/L |  |
| Carbamazepine-d10 | 90797 | 132183-78-9 |  | pct |  |
| Codeine | 62003 | 76-57-3 | 0.046 | ug/L |  |
| Dehydronifedipine | 62004 | 67035-22-7 | 0.08 | ug/L |  |
| Diltiazem | 62008 | 42399-41-7 | 0.06 | ug/L |  |
| Diphenhydramine | 62796 | 147-24-0 | 0.036 | ug/L |  |
| Ethyl nicotinate-d4 | 99571 | 66148-16-1 |  | pct |  |
| Thiabendazole | 62801 | 148-79-8 | 0.06 | ug/L |  |
| Trimethoprim | 62023 | 738-70-5 | 0.034 | ug/L |  |
| Warfarin | 62024 | 81-81-2 | 0.08 | ug/L |  |
| Schedule 2434 - Dissolved Hormones in Water |  |  |  |  |  |
| 11-Ketotestosterone | 64507 | 564-35-2 | 2 | ng/L |  |
| 17-alpha-Estradiol | 64508 | 57-91-0 | 0.8 | ng/L |  |
| 17-alpha-Ethynylestradiol-2,4,16,16-d4 | 90813 | 350820-06-3 |  | pct |  |
| 17-beta-Estradiol | 64510 | 50-28-2 | 0.8 | ng/L |  |
| 17-beta-Estradiol-13,14,15,16,17,18-13C6 | 91753 |  |  | pct |  |
| 17-alpha-Ethynylestradiol | 64509 | 57-63-6 | 0.8 | ng/L |  |
| Norethindrone | 64511 | 68-22-4 | 0.8 | ng/L |  |
| 4-Androstene-3,17-dione | 64513 | 63-05-8 | 0.8 | ng/L |  |
| cis-Androsterone | 64515 | 53-41-8 | 0.8 | ng/L |  |
| Bisphenol-A-d16 | 67308 | 96210-87-6 |  | pct |  |
| Cholesterol | 64514 | 57-88-5 | 200 | ng/L |  |
| Cholesterol-d7 | 90778 | 83199-47-7 |  | pct |  |
| 3-beta-Coprostanol | 64512 | 360-68-9 | 200 | ng/L |  |
| Dihydrotestosterone | 64524 | 521-18-6 | 4 | ng/L |  |
| Bisphenol A | 67304 | 80-05-7 | 200 | ng/L |  |
| Epitestosterone | 64517 | 481-30-1 | 4 | ng/L |  |
| Equilenin | 64518 | 517-09-9 | 2 | ng/L |  |
| Equilin | 64519 | 474-86-2 | 4 | ng/L |  |
| Estriol | 64520 | 50-27-1 | 2 | ng/L |  |
| Estrone | 64521 | 53-16-7 | 0.8 | ng/L |  |
| Estrone-13,14,15,16,17,18-13C6 | 91754 |  |  | pct |  |
| Mestranol | 64522 | 72-33-3 | 0.8 | ng/L |  |
| Mestranol-2,4,16,16-d4 | 90821 |  |  | pct |  |
| Progesterone | 64523 | 57-83-0 | 8 | ng/L |  |
| trans-Diethylstilbestrol | 64516 | 56-53-1 | 0.8 | ng/L |  |
| Testosterone | 64525 | 58-22-0 | 0.8 | ng/L |  |
| trans-Diethyl-1,1,1',1'-d4-stilbesterol-3,3',5,5'-d4 | 90817 | 91318-10-4 |  | pct |  |
| Schedule 6434 - Hormones in Sediment |  |  |  |  |  |
| 11-Ketotestosterone | 64467 | 564-35-2 | 0.26 | ug/kg | mrl |
| 17-alpha-Estradiol | 64468 | 57-91-0 | 0.1 | ug/kg | irl |
| 17-alpha-Ethynylestradiol-2,4,16,16-d4 | 91805 | 350820-06-3 |  | pct |  |
| 17-beta-Estradiol | 63164 | 50-28-2 | 0.1 | ug/kg | irl |
| 17-beta-Estradiol-13,14,15,16,17,18-13C6 | 91757 |  |  | pct | irl |
| 17-alpha-Ethynylestradiol | 63207 | 57-63-6 | 0.1 | ug/kg | irl |
| Norethindrone | 63644 | 68-22-4 | 0.1 | ug/kg | irl |
| 4-Androstene-3,17-dione | 64473 | 63-05-8 | 0.1 | ug/kg | irl |
| cis-Androsterone | 63607 | 53-41-8 | 0.1 | ug/kg | irl |
| Bisphenol-A-d16 | 67310 | 96210-87-6 |  | pct |  |
| Cholesterol | 63196 | 57-88-5 | 250 | ug/kg | mrl |
| Cholesterol-d7 | 90772 | 83199-47-7 |  | pct |  |
| 3-beta-Coprostanol | 63170 | 360-68-9 | 250 | ug/kg | mrl |
| Dihydrotestosterone | 64484 | 521-18-6 | 0.1 | ug/kg | irl |
| Bisphenol A | 63188 | 80-05-7 | 12 | ug/kg | mrl |
| Epitestosterone | 64477 | 481-30-1 | 0.5 | ug/kg | irl |
| Equilenin | 63204 | 517-09-9 | 0.26 | ug/kg | irl |
| Equilin | 64479 | 474-86-2 | 0.5 | ug/kg | irl |
| Estriol | 64480 | 50-27-1 | 0.26 | ug/kg | irl |
| Estrone | 63205 | 53-16-7 | 0.1 | ug/kg | irl |
| Estrone-13,14,15,16,17,18-13C6 | 91758 |  |  | pct | irl |
| Mestranol | 63638 | 72-33-3 | 0.1 | ug/kg | irl |
| Mestranol-2,4,16,16-d4 | 91813 |  |  | pct |  |
| Progesterone | 63657 | 57-83-0 | 0.5 | ug/kg | irl |
| trans-Diethylstilbestrol | 63620 | 56-53-1 | 0.1 | ug/kg | irl |
| Testosterone | 64485 | 58-22-0 | 0.1 | ug/kg | irl |
| trans-Diethyl-1,1,1',1'-d4-stilbesterol-3,3',5,5'-d4 | 91809 | 91318-10-4 |  | pct |  |
| Schedule 5433 - Wastewater Compunds in Sediment |  |  |  |  |  |
| 4-Nonylphenol monoethoxylate, (sum of all isomers) aka NP1EO | 63221 |  | 500 | ug/kg | irl |
| 4-tert-Octylphenol diethoxylate, aka OP2EO | 63201 |  | 50 | ug/kg | irl |
| 4-tert-Octylphenol monoethoxylate, aka OP1EO | 63206 |  | 250 | ug/kg | irl |
| Anthraquinone | 63181 | 84-65-1 | 50 | ug/kg | irl |
| Acetophenone | 63178 | 98-86-2 | 150 | ug/kg | irl |
| Acetyl hexamethyl tetrahydronaphthalene (AHTN) | 63179 | 21145-77-7 | 50 | ug/kg | irl |
| Anthracene | 63180 | 120-12-7 | 50 | ug/kg | irl |
| Atrazine | 63182 | 1912-24-9 | 100 | ug/kg | irl |
| 1,4-Dichlorobenzene | 63163 | 106-46-7 | 50 | ug/kg | irl |
| Benzo[a]pyrene | 63183 | 50-32-8 | 50 | ug/kg | irl |
| Benzophenone | 63184 | 119-61-9 | 50 | ug/kg | irl |
| Bromacil | 63189 | 314-40-9 | 500 | ug/kg | irl |
| 3-tert-Butyl-4-hydroxy anisole (BHA) | 63172 | 25013-16-5 | 150 | ug/kg | irl |
| Camphor | 63192 | 76-22-2 | 50 | ug/kg | irl |
| Carbazole | 63194 | 86-74-8 | 50 | ug/kg | irl |
| Chlorpyrifos | 63195 | 2921-88-2 | 50 | ug/kg | irl |
| Cholesterol | 63196 | 57-88-5 | 250 | ug/kg | irl |
| 3-beta-Coprostanol | 63170 | 360-68-9 | 500 | ug/kg | irl |
| Isopropylbenzene | 63213 | 98-82-8 | 100 | ug/kg | irl |
| Fluoranthene-d10 | 90738 | 93951-69-0 |  | pct | irl |
| Bisphenol A-d3 | 90735 |  |  | pct | irl |
| Decafluorobiphenyl | 90737 | 434-90-2 |  | pct | irl |
| N,N-diethyl-meta-toluamide (DEET) | 63219 | 134-62-3 | 100 | ug/kg | irl |
| Diazinon | 63198 | 333-41-5 | 50 | ug/kg | irl |
| Bisphenol A | 63188 | 80-05-7 | 50 | ug/kg | irl |
| Fluoranthene | 63208 | 206-44-0 | 50 | ug/kg | irl |
| Hexahydrohexamethylcyclopentabenzopyran (HHCB) | 63209 | 1222-05-5 | 50 | ug/kg | irl |
| Indole | 63210 | 120-72-9 | 100 | ug/kg | irl |
| Isoborneol | 63211 | 124-76-5 | 50 | ug/kg | irl |
| Isophorone | 63212 | 78-59-1 | 50 | ug/kg | irl |
| Isoquinoline | 63214 | 119-65-3 | 100 | ug/kg | irl |
| d-Limonene | 63203 | 5989-27-5 | 50 | ug/kg | irl |
| Menthol | 63215 | 89-78-1 | 50 | ug/kg | irl |
| Metolachlor | 63218 | 51218-45-2 | 50 | ug/kg | irl |
| Naphthalene | 63220 | 91-20-3 | 50 | ug/kg | irl |
| 1-Methylnaphthalene | 63165 | 90-12-0 | 50 | ug/kg | irl |
| 2,6-Dimethylnaphthalene | 63167 | 581-42-0 | 50 | ug/kg | irl |
| 2-Methylnaphthalene | 63168 | 91-57-6 | 50 | ug/kg | irl |
| 4-Nonylphenol diethoxylate, (sum of all isomers) aka NP2EO | 63200 |  | 1000 | ug/kg | irl |
| p-Cresol | 63222 | 106-44-5 | 250 | ug/kg | irl |
| 4-Cumylphenol | 63173 | 599-64-4 | 50 | ug/kg | irl |
| 4-n-Octylphenol | 63174 | 1806-26-4 | 50 | ug/kg | irl |
| 4-tert-Octylphenol | 63176 | 140-66-9 | 50 | ug/kg | irl |
| 2,2',4,4'-Tetrabromodiphenylether (PBDE 47) | 63166 | 5436-43-1 | 50 | ug/kg | irl |
| Phenanthrene | 63224 | 85-01-8 | 50 | ug/kg | irl |
| Phenol | 63225 | 108-95-2 | 50 | ug/kg | irl |
| 4-Nonylphenol (sum of all isomers) | 63175 | 104-40-5 | 750 | ug/kg | irl |
| Tributyl phosphate | 63231 | 126-73-8 | 50 | ug/kg | irl |
| Triphenyl phosphate | 63234 | 115-86-6 | 50 | ug/kg | irl |
| Tris(2-butoxyethyl)phosphate | 63229 | 78-51-3 | 150 | ug/kg | irl |
| Tris(2-chloroethyl)phosphate | 63230 | 115-96-8 | 100 | ug/kg | irl |
| bis(2-Ethylhexyl) phthalate | 63187 | 117-81-7 | 250 | ug/kg | irl |
| Diethyl phthalate | 63202 | 84-66-2 | 100 | ug/kg | irl |
| Prometon | 63226 | 1610-18-0 | 50 | ug/kg | irl |
| Pyrene | 63227 | 129-00-0 | 50 | ug/kg | irl |
| Sample weight, grams | 91100 |  |  | g | irl |
| 3-Methyl-1(H)-indole (Skatole) | 63171 | 83-34-1 | 50 | ug/kg | irl |
| beta-Sitosterol | 63185 | 83-46-5 | 500 | ug/kg | irl |
| beta-Stigmastanol | 63186 | 19466-47-8 | 500 | ug/kg | irl |
| Triclosan | 63232 | 3380-34-5 | 50 | ug/kg | irl |
| Tris(dichlorisopropyl)phosphate | 63235 | 13674-87-8 | 100 | ug/kg | irl |
| Lab Code 9008 - Pharmaceuticals in Sediment |  |  |  |  |  |
| Cotinine | USGS Custom Organic Analyses |  |  | ug/kg | irl |
| 1,7-Dimethylxanthine (p-Xanthine) |  |  |  | ug/kg | irl |
| Acetaminophen |  |  |  | ug/kg | irl |
| Albuterol (Salbutamol) |  |  |  | ug/kg | irl |
| Sulfamethoxazole |  |  |  | ug/kg | irl |
| Caffeine |  |  |  | ug/kg | irl |
| Carbamazepine |  |  |  | ug/kg | irl |
| Carbamazepine-d10 |  |  |  | ug/kg | irl |
| Codeine |  |  |  | ug/kg | irl |
| Dehydronifedipine |  |  |  | ug/kg | irl |
| Diltiazem |  |  |  | ug/kg | irl |
| Diphenhydramine |  |  |  | ug/kg | irl |
| Ethyl nicotinate-d4 |  |  |  | ug/kg | irl |
| Thiabendazole |  |  |  | ug/kg | irl |
| Trimethoprim |  |  |  | ug/kg | irl |
| Warfarin |  |  |  | ug/kg | irl |