Wolf River TMDL Monitoring 2010-2012

Parent project: NER 303d and TMDL

NER_02_11
Purpose
This is the third year of monitoring for the Upper Fox Basin and the first year of monitoring for the Wolf River Basin. NER biologists and CO SWAT will collect water samples from the Upper Fox Basin waterbodies of Fox River, Puchyan River, Fond du Lac River, Montello River, Waukau Creek, the Wolf Basin waterbodies of the Wolf River, Little Wolf River, Embarrass River, Waupaca River, and lakes Poygan, Winneconne, Butte des Morts, and Winnebago to be analyzed for TSS, Total Phosphorus, and Dissolved Phosphorus that will be used in modeling the sediment and phosphorus loads to Lake Winnebago. Also in collaboration with the USGS, sediment core samples will be collected from Lake Winnebago, Lake Poygan, Lake Winneconne, and Lake Butte des Morts to be used for the quantification of the existing internal nutrient load within the system. Results of the modeling will be used in developing a TMDL for Lake Winnebago to drive management practices in the Upper Fox Basin.

NER_06_CMP12
Purpose
Finish the monitoring requirements of the Upper Fox Basin. **2nd year of monitoring for the Wolf River Basin. NER biologists and CO SWAT will collect water samples from the Upper Fox Basin waterbodies of Fox River, Puchyan River, Fon du Lac River, Montello River, Waukau Creek, the Wolf Basin waterbodies of the Wolf River, Little Wolf River, Embarrass River, Waupaca River, and lakes Poygan, Winneconne, Butte des Morts, and Winnebago to be analyzed for TSS, Total Phosphorus, and Dissolved Phosphorus that will be used in modeling the sediment and phosphorus loads to Lake Winnebago. Results of the modeling will be used in developing a TMDL for Lake Winnebago to drive management practices in the Upper Fox Basin.

17 Stations in projects NER_02_11 and NER_06_CMP12

Station ID:       Station Name:

713245	          Lake Winnebago – 3 Mi Oshkosh  
713244           Lake Winnebago – Deep Hole-South End  
713243           Lake Winnebago – 2 Mi From Neenah  
713282           Lake Winneconne – Central (Wi-2)
713279           Lake Poygan – East (P03)
713252           Lake Butte Des Morts – East End Bu3

10022877      Fond du Lac River At W. Arndt St. USGS Site ID 04083545

243020	          Fox River at Berlin, WI USGS Site ID 04073500

10022879      Montello River At 11th St. Bridge USGS Site ID 04072845

10022878      Puchyan River At N. Lawson St. Bridge USGS Site ID 04073473

713285	          Waukau Creek at Cth E USGS Site ID 04073970
593162           Embarrass River – Near Embarrass, WI USGS Site ID 04078500     
10033493      Embarrass River at New London Hwy 54     
713002           Fox River - Lake Winnebago Outlet         
693217           Little Wolf River at Royalton, WI USGS Site ID 04080000

693161           Waupaca River at Harrington Rd USGS Site ID 04081000  
693218           Wolf River at New London, WI USGS Site ID 04079000

CO_02_CMP_13

Purpose

3rd and final year of monitoring for TMDL development for the Wolf River Basin.  East District Biologists will collect water samples from the waters of the Wolf River Basin including: the Wolf River (241300), Little Wolf River (272400), Waupaca (257400) and Embarrass River (291900) and the Outlet of Lake Winnebago (117900-LFR). These samples will be analyzed for TSS, TP, and dissolved P and will be used in the modeling to calculate loads for the Upper Fox/Wolf TMDL.

14 sampling events will be covered in the Wolf Basin, with a total of 6 sample sites. Sample collection is projected to end in December 2012. These samples will be analyzed for TSS, TP, and dissolved P and will be used in the modeling to calculate loads for the Upper Fox/Wolf TMDL.

Sampling Method:  Samples were taken from bridges above the river.  The quart bottle and 250ml bottles were lowered down into the water and rinsed three times before the sample was collected.  Samples for Total Phosphorus and Total Suspended Solids were taken for each sampling event.  1ml of H2SO4 was added to every 250ml sample.  Every third sample event also collected Total Dissolved Phosphorus.  When preparing the Total Dissolved Phosphorus sample, water from the quart bottle was poured into a 60ml NORM-JECT syringe and filtered through a .45um Millipore filter into a 60ml rinsed bottle.  One ½ ml of H2SO4 was added to the 60ml sample.  The samples were collected once a week March-June, every other week July-November, and monthly December-February.

Rivers monitored for CO_02_CMP13

Station ID:       Station Name:

593162           Embarrass River – Near Embarrass, WI USGS Site ID 04078500      
10033493      Embarrass River at New London Hwy 54     			
713002          Fox River - Lake Winnebago Outlet         			
693217          Little Wolf River at Royalton, WI USGS Site ID 04080000

693161          Waupaca River at Harrington Rd USGS Site ID 04081000  	
693218          Wolf River at New London, WI USGS Site ID 04079000

Counties: Winnebago, Outagamie, Waupaca, Shawano

Samples taken:  Total Phosphorus =TP
	            Total Suspended Solids =TSS
  	            Total Dissolved Phosphorus =TDP (taken every third sample event)

Fox River at Lake Winnebago Outlet

Little Wolf River at Royalton

Wolf River at New London

Embarrass River at New London

Waupaca River near Harrington Rd

Embarrass River- Near Embarrass

17 Stations from projects NER_02_11, NER_06_CMP12, and CO_02_CMP13

| Fieldwork start | Embarrass River-Near Embarrass, WI | Waupaca River | Little Wolf River at Royalton | Wolf River at New London | Fox River at Lake Winnebago Outlet | Total Phosphorus mg/L Embarrass River at New London |
| --- | --- | --- | --- | --- | --- | --- |
| 07/23/2010 | 0.134 | 0.093 | 0.157 | 0.271 | 0.15 |  |
| 08/05/2010 | 0.09 | 0.049 | 0.103 | 0.254 |  |  |
| 08/27/2010 | 0.035 | 0.029 | 0.046 | 0.18 | 0.089 |  |
| 09/16/2010 | 0.035 | 0.043 | 0.035 | 0.079 |  |  |
| 09/25/2010 | 0.105 | 0.078 | 0.156 | 0.195 |  |  |
| 09/30/2010 | 0.063 | 0.059 | 0.072 | 0.115 | 0.083 |  |
| 11/12/2010 | 0.018 | 0.032 | 0.026 | 0.052 | 0.051 |  |
| 03/17/2011 | 0.063 | 0.054 | 0.115 | 0.051 | 0.06 |  |
| 03/30/2011 | 0.043 | 0.042 | 0.047 | 0.063 | 0.046 |  |
| 04/07/2011 | 0.092 | 0.052 | 0.057 | 0.041 | 0.044 | 0.075 |
| 04/14/2011 | 0.065 | 0.033 | 0.034 | 0.043 | 0.048 | 0.069 |
| 04/20/2011 | 0.035 | 0.049 | 0.053 | 0.062 | 0.054 | 0.043 |
| 04/28/2011 | 0.042 | 0.051 | 0.077 | 0.026 | 0.06 | 0.073 |
| 05/06/2011 | 0.028 | 0.03 | 0.031 | 0.055 | 0.038 | 0.042 |
| 05/13/2011 | 0.043 | 0.071 | 0.137 | 0.072 | 0.037 | 0.115 |
| 05/20/2011 | 0.083 | 0.035 | 0.044 | 0.123 | 0.031 | 0.034 |
| 05/25/2011 | 0.047 | 0.047 | 0.072 | 0.104 | 0.03 | 0.13 |
| 06/01/2011 | 0.049 | 0.043 | 0.06 | 0.101 | 0.039 | 0.112 |
| 06/08/2011 | 0.053 | 0.043 | 0.053 | 0.074 | 0.07 | 0.115 |
| 06/16/2011 | 0.051 | 0.048 | 0.05 | 0.132 | 0.08 | 0.108 |
| 06/23/2011 | 0.123 | 0.156 | 0.181 | 0.146 |  | 0.172 |
| 06/26/2011 |  |  |  |  |  | 0.117 |
| 06/30/2011 | 0.08 | 0.067 | 0.102 | 0.122 | 0.088 | 0.148 |
| 07/12/2011 | 0.055 | 0.057 | 0.083 | 0.168 | 0.074 | 0.141 |
| 07/26/2011 | 0.059 | 0.066 | 0.136 | 0.102 | 0.116 | 0.146 |
| 08/11/2011 | 0.095 | 0.049 | 0.091 | 0.098 | 0.181 | 0.135 |
| 08/25/2011 | 0.042 | 0.037 | 0.033 | 0.066 | 0.172 | 0.089 |
| 09/08/2011 | 0.028 | 0.027 | 0.038 | 0.062 | 0.207 | 0.064 |
| 09/22/2011 | 0.039 |  | 0.028 | 0.045 | 0.147 | 0.041 |
| 10/06/2011 | 0.037 | 0.025 | 0.04 | 0.071 | 0.104 | 0.089 |
| 10/20/2011 | 0.034 | 0.019 | 0.037 | 0.055 | 0.155 | 0.061 |
| 11/18/2011 | 0.02 | 0.02 | 0.034 | 0.048 | 0.118 | 0.046 |
| 12/21/2011 | 0.038 | 0.038 | 0.038 | 0.038 |  | 0.038 |
| 01/11/2012 | 0.03 | 0.03 | 0.03 | 0.03 |  | 0.03 |
| 02/17/2012 | 0.3 | 0.03 | 0.032 | 0.03 |  | 0.053 |
| 03/10/2012 | 0.156 | 0.083 | 0.153 | 0.152 |  | 0.192 |
| 03/20/2012 | 0.07 | 0.037 | 0.03 | 0.062 |  | 0.083 |
| 03/23/2012 | 0.071 | 0.044 | 0.068 | 0.068 | 0.07 | 0.082 |
| 03/30/2012 | 0.05 | 0.058 | 0.054 | 0.064 | 0.135 | 0.084 |
| 04/06/2012 | 0.037 | 0.026 | 0.037 | 0.059 | 0.057 | 0.082 |
| 04/10/2012 | 0.029 | 0.02 | 0.034 | 0.07 | 0.048 | 0.065 |
| 04/18/2012 | 0.027 | 0.02 | 0.035 | 0.066 | 0.082 | 0.059 |
| 04/24/2012 | 0.034 |  | 0.044 | 0.062 | 0.054 | 0.062 |
| 05/02/2012 | 0.032 | 0.033 | 0.039 | 0.064 | 0.069 | 0.065 |
| 05/09/2012 | 0.066 | 0.066 | 0.064 | 0.079 | 0.044 | 0.094 |
| 05/16/2012 | 0.043 | 0.038 | 0.044 | 0.089 | 0.068 | 0.099 |
| 05/24/2012 | 0.035 | 0.04 | 0.03 | 0.067 | 0.092 | 0.089 |
| 05/30/2012 | 0.053 | 0.049 | 0.065 | 0.065 | 0.081 | 0.105 |
| 06/06/2012 | 0.036 | 0.032 | 0.046 | 0.059 | 0.072 | 0.09 |
| 06/13/2012 | 0.035 | 0.037 | 0.033 | 0.065 | 0.087 | 0.082 |
| 06/20/2012 | 0.082 | 0.051 | 0.067 | 0.066 | 0.114 | 0.086 |
| 06/27/2012 | 0.075 | 0.041 | 0.054 | 0.07 | 0.108 | 0.111 |
| 07/05/2012 | 0.057 | 0.045 | 0.058 | 0.066 | 0.098 | 0.106 |
| 07/17/2012 | 0.046 | 0.047 | 0.049 | 0.074 | 0.194 | 0.083 |
| 08/01/2012 | 0.046 | 0.039 | 0.054 | 0.058 | 0.193 | 0.114 |
| 08/15/2012 | 0.031 | 0.089 | 0.032 | 0.052 | 0.226 | 0.076 |
| 08/28/2012 | 0.033 | 0.073 | 0.034 | 0.048 | 0.2 | 0.07 |
| 09/13/2012 | 0.031 | 0.035 | 0.025 | 0.055 | 0.263 | 0.065 |
| 09/25/2012 | 0.018 | 0.055 | 0.016 | 0.038 | 0.148 | 0.029 |
| 10/17/2012 | 0.046 | 0.067 | 0.074 | 0.049 | 0.313 | 0.071 |
| 10/26/2012 | 0.034 | 0.046 | 0.038 | 0.055 | 0.108 | 0.065 |
| 11/06/2012 | 0.014 | 0.023 | 0.015 | 0.028 | 0.084 | 0.037 |
| Average | 0.05640984 | 0.047389831 | 0.059344262 | 0.08072131 | 0.103461538 | 0.08550943 |

Total Suspended Solids

| Fieldwork start | Embarrass River-Near Embarrass, WI | Waupaca River | Little Wolf River at Royalton | Wolf River at New London | Fox River at Lake Winnebago Outlet | Embarrass River at New London |
| --- | --- | --- | --- | --- | --- | --- |
| 07/23/2010 | 33 | 27 | 20 | 5 | 22 |  |
| 08/05/2010 | 6 | 8 | 8 | 4 |  |  |
| 08/27/2010 | 4 | 6 | 4 | 22 | 3 |  |
| 09/16/2010 | 3 | 10 | 4 | 18 |  |  |
| 09/25/2010 | 32 | 30 | 41 | 27 |  |  |
| 09/30/2010 | 6 | 12 | 9 | 9 | 8 |  |
| 11/12/2010 |  | 5 | 5 | 6 | 5 |  |
| 03/17/2011 | 4 | 14 | 23 | 5 | ND |  |
| 03/30/2011 | 4 | 8 | 7 | 9 | 3 |  |
| 04/07/2011 | 14 | 15 | 13 | 5 | 4 | 7 |
| 04/14/2011 | 14 | 7 | 5 | 8 | 11 | 8 |
| 04/20/2011 | 3 | 15 | 12 | 9 | 12 | 4 |
| 04/28/2011 | 8 | 13 | 14 | 6 | 20 | 10 |
| 05/06/2011 | 3 | 9 | 6 | 10 | 17 | 6 |
| 05/13/2011 | 5 | 21 | 48 | 12 | 9 | 28 |
| 05/20/2011 | 5 | 11 | 6 | 14 | 8 | 11 |
| 05/25/2011 | 8 | 14 | 12 | 12 | 5 | 17 |
| 06/01/2011 | 6 | 15 | 12 | 20 | 3 | 20 |
| 06/08/2011 | 11 | 12 | 13 | 12 | 2 | 40 |
| 06/16/2011 | 8 | 14 | 12 | 29 | 3 | 39 |
| 06/23/2011 | 28 | 31 | 32 | 20 |  | 38 |
| 06/26/2011 |  |  |  |  |  | 8.1 |
| 06/30/2011 | 8 | 18 | 14 | 6 | 4 | 3 |
| 07/12/2011 | 8 | 21 | 16 | 15 | 2 | 43 |
| 07/26/2011 | 3 | 13 | 12 | 7 | 5 | 32 |
| 08/11/2011 | 9 | 11 | 11 | 20 | 6 | 30 |
| 08/25/2011 | 5 | 7 | 4 | 22 | 7 | 31 |
| 09/08/2011 | 2 | 7 | 5 | 20 | 19 | 24 |
| 09/22/2011 | 10 | 6 | 6 | 14 | 11 | 11 |
| 10/06/2011 |  | 6 | 6 | 16 | 13 | 16 |
| 10/20/2011 |  |  |  |  | 12 |  |
| 11/18/2011 |  | 4 | 4 | 4 | 37 | 4 |
| 12/21/2011 | 2.9 | 5.1 | 4.8 | 3.5 |  | 5.7 |
| 01/11/2012 | 2.8 | 8.9 | 9.5 | 2.9 |  | 2.5 |
| 02/17/2012 | 8.6 | 14 | 12 | 9.5 |  | 23 |
| 03/10/2012 | 14 | 12 | 18 | 40 |  | 22 |
| 03/20/2012 | 12 | 15.4 | 14.2 | 9.5 |  | 25.2 |
| 03/23/2012 | 6 | 9 | 13 | 6 | 15 | 11 |
| 03/30/2012 | 5 | 15 | 11 | 10 | 47 | 17 |
| 04/06/2012 | 4 | 6 | 7 | 18 | 13 | 18 |
| 04/10/2012 | 3 | 5 | 6 | 25 | 9 | 18 |
| 04/18/2012 | 5 | 5 | 13 | 20 | 19 | 19 |
| 04/24/2012 | 8 | 7 | 12 | 17 | 6 | 17 |
| 05/02/2012 | 8 | 12 | 12 | 24 | 11 | 28 |
| 05/09/2012 | 15 | 14 | 14 | 13 | 5 | 13 |
| 05/16/2012 | 4 | 15 | 7 | 13 | 5 | 16 |
| 05/24/2012 | 9 | 16 | 8 | 18 | 5 | 33 |
| 05/30/2012 | 8 | 23 | 19 | 19 | 5 | 51 |
| 06/06/2012 | 6 | 13 | 13 | 22 | 6 | 41 |
| 06/13/2012 | 4 | 13 | 7 | 17 | 7 | 33 |
| 06/20/2012 | 15 | 22 | 16 | 17 | 17 | 28 |
| 06/27/2012 | 5 | 12 | 5 | 17 | 6 | 38 |
| 07/05/2012 | 4 | 14 | 11 | 10 | 5 | 26 |
| 07/17/2012 | 3 | 16 | 30 | 14 | 21 | 16 |
| 08/01/2012 | 3 | 11 | 29 | 12 | 24 | 54 |
| 08/15/2012 | 3 | 14 | 5 | 12 | 24 | 38 |
| 08/28/2012 |  | 13 | 3 | 13 | 23 | 30 |
| 09/13/2012 |  | 11 |  | 16 | 30 | 31 |
| 09/25/2012 | 2 | 4 | 3 | 11 | 51 | 11 |
| 10/17/2012 | 6 | 8 | 21 | 14 | 98 | 23 |
| 10/26/2012 | 2 | 5 | 5 | 15 | 14 | 21 |
| 11/06/2012 |  | 2 | 2 | 3 | 8 | 3 |
| Average | 7.746296296 | 12.17333333 | 12.11016949 | 13.79 | 14.21568627 | 21.97115385 |

Total Dissolved Phosphorus

| Fieldwork start | Embarrass River-Near Embarrass, WI | Waupaca River | Little Wolf River at Royalton | Wolf River at New London | Fox River at Lake Winnebago Outlet | Embarrass River at New London |
| --- | --- | --- | --- | --- | --- | --- |
| 08/27/2010 | 0.014 | 0.016 | 0.033 | 0.124 | 0.078 |  |
| 09/25/2010 | 0.026 | 0.021 | 0.041 | 0.082 |  |  |
| 03/17/2011 | 0.044 | 0.027 | 0.058 |  | 0.045 |  |
| 04/07/2011 | 0.036 |  |  |  |  |  |
| 04/14/2011 |  | 0.026 | 0.033 | 0.038 | 0.018 | 0.045 |
| 05/13/2011 | 0.022 | 0.029 | 0.045 | 0.03 | 0.009 | 0.047 |
| 06/08/2011 |  | 0.022 | 0.028 | 0.051 | 0.043 | 0.038 |
| 06/23/2011 | 0.056 | 0.076 | 0.096 | 0.079 |  | 0.079 |
| 06/26/2011 |  |  |  | 0.093 |  | 0.118 |
| 07/12/2011 | 0.02 | 0.025 | 0.049 | 0.111 | 0.057 | 0.022 |
| 08/11/2011 | 0.068 | 0.029 | 0.059 | 0.055 | 0.125 | 0.08 |
| 10/06/2011 | 0.029 | 0.012 | 0.023 | 0.043 | 0.06 | 0.06 |
| 11/18/2011 | 0.012 | 0.01 | 0.02 | 0.032 | 0.016 | 0.03 |
| 12/21/2011 | 0.038 | 0.038 | 0.038 | 0.038 | 0.013 | 0.038 |
| 01/11/2012 | 0.03 | 0.03 | 0.03 | 0.03 |  | 0.03 |
| 02/17/2012 | 0.03 | 0.03 | 0.03 | 0.03 |  | 0.03 |
| 03/10/2012 | 0.13 | 0.049 | 0.083 | 0.05 |  | 0.127 |
| 03/20/2012 | 0.159 | 0.03 | 0.03 | 0.03 |  | 0.03 |
| 03/23/2012 | 0.042 | 0.019 | 0.033 | 0.04 |  | 0.052 |
| 04/10/2012 | 0.022 | 0.008 | 0.015 | 0.026 | 0.011 | 0.031 |
| 05/02/2012 | 0.016 | 0.013 | 0.018 | 0.026 | 0.024 | 0.027 |
| 05/24/2012 | 0.007 | 0.01 | 0.01 | 0.022 | 0.064 | 0.012 |
| 06/13/2012 | 0.022 | 0.014 | 0.017 | 0.021 | 0.056 | 0.026 |
| 07/05/2012 | 0.046 | 0.02 | 0.044 | 0.038 | 0.074 |  |
| 08/15/2012 | 0.026 | 0.069 | 0.02 | 0.028 | 0.127 | 0.029 |
| 09/25/2012 | 0.013 | 0.046 | 0.01 | 0.019 | 0.037 | 0.014 |
| 11/06/2012 | 0.01 | 0.018 | 0.01 | 0.018 | 0.051 | 0.022 |
| Average | 0.03825 | 0.02748 | 0.03492 | 0.04616 | 0.050444444 | 0.044863636 |

Stations from CO_02_CMP13

Fox River - Lake Winnebago Outlet         Station ID: 713002

Embarrass River at New London Hwy 54       Station ID: 10033493

Wolf River at New London, WI USGS Site ID 04079000          Station ID: 693218

Little Wolf River at Royalton, WI USGS Site ID 04080000               Station ID: 693217

Waupaca River at Harrington Rd USGS Site ID 04081000   	            Station ID: 693161

Embarrass River – Near Embarrass, WI USGS Site ID 04078500         Station ID: 593162