Date: 7/12/2016    Scenario: 1938 Diatom Infered TP Reconstruction

Lake Id: Big Blake Lake

Watershed Id: 0

Hydrologic and Morphometric Data

Tributary Drainage Area: 2133.4 acre

Total Unit Runoff: 8.00 in.

Annual Runoff Volume: 1422.3 acre-ft

Lake Surface Area <As>: 208.0 acre

Lake Volume <V>: 1872.0 acre-ft

Lake Mean Depth <z>: 9.0 ft

Precipitation - Evaporation: 3.3 in.

Hydraulic Loading: 1479.5 acre-ft/year

Areal Water Load <qs>: 7.1 ft/year

Lake Flushing Rate <p>: 0.79 1/year

Water Residence Time: 1.27 year

Observed spring overturn total phosphorus (SPO): 0.0 mg/m^3

Observed growing season mean phosphorus (GSM): 36.05 mg/m^3

% NPS Change: 0%

% PS Change: 0%

NON-POINT SOURCE DATA

Land Use        Acre        Low    Most Likely    High    Loading %   Low    Most Likely    High

(ac)     |---- Loading (kg/ha-year) ----|            |-----  Loading (kg/year) ----|

Row Crop AG           490.1       0.50       1.00       3.00       56.9         99        198        595

Mixed AG                0.0       0.30       0.80       1.40        0.0          0          0          0

Pasture/Grass         744.7       0.10       0.30       0.50       25.9         30         90        151

HD Urban (1/8 Ac)       0.0       1.00       1.50       2.00        0.0          0          0          0

MD Urban (1/4 Ac)       0.0       0.30       0.50       0.80        0.0          0          0          0

Rural Res (>1 Ac)      56.9       0.05       0.10       0.25        0.7          1          2          6

Wetlands              391.1       0.10       0.10       0.10        4.5         16         16         16

Forest                450.6       0.05       0.09       0.18        4.7          9         16         33

Lake Surface          208.0       0.10       0.30       1.00        7.2          8         25         84

POINT SOURCE DATA

Point Sources     Water Load     Low    Most Likely    High    Loading %

(m^3/year)  (kg/year)  (kg/year)   (kg/year)          _

SEPTIC TANK DATA

Description                                        Low    Most Likely   High     Loading %

Septic Tank Output (kg/capita-year)                0.30        0.50     0.80

# capita-years                          0.0

% Phosphorus Retained by Soil                      98.0        90.0     80.0

Septic Tank Loading (kg/year)                      0.00        0.00     0.00         0.0

TOTALS DATA

Description                      Low    Most Likely   High     Loading %

Total Loading (lb)               361.2       768.5      1949.6   100.0

Total Loading (kg)               163.8       348.6       884.3   100.0

Areal Loading (lb/ac-year)        1.74        3.69        9.37

Areal Loading (mg/m^2-year)     194.63      414.10     1050.61

Total PS Loading (lb)              0.0         0.0         0.0     0.0

Total PS Loading (kg)              0.0         0.0         0.0     0.0

Total NPS Loading (lb)           342.6       712.8      1764.0   100.0

Total NPS Loading (kg)           155.4       323.3       800.2   100.0

Wisconsin Internal Load Estimator

Date: 7/12/2016    Scenario: 19

Method 1 - A Complete Total Phosphorus Mass Budget

Method 1 - A Complete Total Phosphorus Mass Budget 36.05 mg/m^3

Phosphorus Inflow Concentration: 191.0 mg/m^3

Areal External Loading: 414.1 mg/m^2-year

Predicted Phosphorus Retention Coefficient: 0.74

Observed Phosphorus Retention Coefficient: 0.81

Internal Load: -52 Lb     -24 kg

Method 2 - From Growing Season In Situ Phososphorus Increases

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 0 mg/m^3

Hypolimnetic Volume: 0.0 acre-ft

Anoxia Sediment Area: 0.0 acres

Just Prior To The End of Stratification

Average Hypolimnetic Phosphorus Concentration: 0 mg/m^3

Hypolimnetic Volume: 0.0 acre-ft

Anoxia Sediment Area: 0.0 acres

Time Period of Stratification: 1 days

Sediment Phosphorus Release Rate: 0 mg/m^2-day     0 lb/acre-day

Internal Load: 0 Lb     0 kg

Method 3 - From In Situ Phososphorus Increases In The Fall

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 0 mg/m^3

Hypolimnetic Volume: 0 acre-ft

Anoxia Sediment Area: 0 acres

Just Prior To The End of Stratification

Average Water Column Phosphorus Concentration: 0 mg/m^3

Lake Volume: 1872.0 acre-ft

Anoxia Sediment Area Just Before Turnover: 0 acres

Time Period Between Observations: 1 days

Sediment Phosphorus Release Rate: 0 mg/m^2-day     0 lb/acre-day

Internal Load:   0 Lb       0 kg

Method 4 - From Phososphorus Release Rate and Anoxic Area

Start of Anoxia Anoxic Sediment Area: 0 acre

End of Anoxia Anoxic Sediment Area: 0 acre

Phosphorus Release Rate As Calculated In Method 2: 0 mg/m^2-day

Phosphorus Release Rate As Calculated In Method 3: 0 mg/m^2-day

Average of Methods 2 and 3 Release Rates: 0.0 mg/m^2-day

Period of Anoxia: 0 days

Default Areal Sediment Phosphorus Release Rates:

Low   Most Likely   High

6        14         24

Internal Load: (Lb)            0         0          0

Internal Load: (kg)            0         0          0

Internal Load Comparison (Percentanges are of the Total Estimate Load)

Total External Load: 769 Lb      349 kg

Lb         kg         %

From A Complete Mass Budget:                              -52       -24      -7.2

From Growing Season In Situ Phosphorus Increases:           0         0         0

From In Situ Phososphorus Increases In The Fall:            0         0       0.0

From Phososphorus Release Rate and Anoxic Area:             0         0         0

Predicted Water Column Total Phosphorus Concentration (ug/l)

Nurnberg+ 1984 Total Phosphorus Model:      Low    Most Likely   High

10          49       124

Osgood, 1988 Lake Mixing Index: 3.0

Phosphorus Loading Summary:

Low      Most Likely     High

Internal Load (Lb):       -52            0.0          0

Internal Load (kg):       -24            0.0          0

External Load (Lb):       361            769       1950

External Load (kg):       164            349        884

Total Load (Lb):          309            769       1950

Total Load (kg):          140            349        884

Phosphorus Prediction and Uncertainty Analysis Module

Date: 7/12/2016    Scenario: 15

Observed spring overturn total phosphorus (SPO): 0.0 mg/m^3

Observed growing season mean phosphorus (GSM): 36.0 mg/m^3

Back calculation for SPO total phosphorus: 0.53 mg/m^3

Back calculation GSM phosphorus: 0.53 mg/m^3

% Confidence Range: 70%

Nurenberg Model Input - Est. Gross Int. Loading: 49 kg

Lake Phosphorus Model              Low   Most Likely   High     Predicted  % Dif.

Total P   Total P    Total P   -Observed

(mg/m^3) (mg/m^3)   (mg/m^3)   (mg/m^3)

Walker, 1987 Reservoir                         30       65        164         29        81

Canfield-Bachmann, 1981 Natural Lake           37       63        117         27        75

Canfield-Bachmann, 1981 Artificial Lake        32       51         84         15        42

Rechow, 1979 General                           14       29         74         -7       -19

Rechow, 1977 Anoxic                            67      142        360        106       294

Rechow, 1977 water load<50m/year               31       67        169         31        86

Rechow, 1977 water load>50m/year              N/A      N/A        N/A        N/A       N/A

Walker, 1977 General                          N/A      N/A        N/A        N/A       N/A

Vollenweider, 1982 Combined OECD               33       62        133         44       244

Dillon-Rigler-Kirchner                        N/A      N/A        N/A        N/A       N/A

Vollenweider, 1982 Shallow Lake/Res.           27       53        121         35       194

Larsen-Mercier, 1976                          N/A      N/A        N/A        N/A       N/A

Nurnberg, 1984 Oxic                            50       76        151         40       111

Lake Phosphorus Model          Confidence Confidence  Parameter    Back       Model

Lower      Upper      Fit?    Calculation   Type

Bound      Bound               (kg/year)

Walker, 1987 Reservoir                       37        130         FIT         3       GSM

Canfield-Bachmann, 1981 Natural Lake         20        181         FIT         1       GSM

Canfield-Bachmann, 1981 Artificial Lake      16        147         FIT         1       GSM

Rechow, 1979 General                         16         59         FIT         6       GSM

Rechow, 1977 Anoxic                          83        283         FIT         1       GSM

Rechow, 1977 water load<50m/year             37        135           P         3       GSM

Rechow, 1977 water load>50m/year            N/A        N/A         N/A       N/A       N/A

Walker, 1977 General                        N/A        N/A         N/A       N/A       N/A

Vollenweider, 1982 Combined OECD             30        122         FIT         1       ANN

Dillon-Rigler-Kirchner                      N/A        N/A         N/A       N/A       N/A

Vollenweider, 1982 Shallow Lake/Res.         26        107         FIT         2       ANN

Larsen-Mercier, 1976                        N/A        N/A         N/A       N/A       N/A

Nurnberg, 1984 Oxic                          44        136           P      -187       ANN