Date: 3/16/2016    Scenario: 15

Lake Id: Big Blake Lake

Watershed Id: 1

Hydrologic and Morphometric Data

Tributary Drainage Area: 20066.1 acre

Total Unit Runoff: 8.00 in.

Annual Runoff Volume: 13377.4 acre-ft

Lake Surface Area <As>: 208.0 acre

Lake Volume <V>: 1872.0 acre-ft

Lake Mean Depth <z>: 9.0 ft

Precipitation - Evaporation: 3.3 in.

Hydraulic Loading: 13434.6 acre-ft/year

Areal Water Load <qs>: 64.6 ft/year

Lake Flushing Rate <p>: 7.18 1/year

Water Residence Time: 0.14 year

Observed spring overturn total phosphorus (SPO): 46.4 mg/m^3

Observed growing season mean phosphorus (GSM): 82.8 mg/m^3

% NPS Change: 0%

% PS Change: 0%

NON-POINT SOURCE DATA

Land Use        Acre        Low    Most Likely    High    Loading %   Low    Most Likely    High

(ac)     |---- Loading (kg/ha-year) ----|            |-----  Loading (kg/year) ----|

Row Crop AG          1023.8       0.50       1.00       3.00       24.8        207        414       1243

Mixed AG                0.0       0.30       0.80       1.40        0.0          0          0          0

Pasture/Grass        4790.6       0.10       0.30       0.50       34.8        194        582        969

HD Urban (1/8 Ac)       4.3       1.00       1.50       2.00        0.2          2          3          3

MD Urban (1/4 Ac)       0.0       0.30       0.50       0.80        0.0          0          0          0

Rural Res (>1 Ac)     691.9       0.05       0.10       0.25        1.7         14         28         70

Wetlands             1496.3       0.10       0.10       0.10        3.6         61         61         61

Forest              10759.2       0.05       0.09       0.18       23.5        218        392        784

Lake Surface          208.0       0.10       0.30       1.00        1.5          8         25         84

POINT SOURCE DATA

Point Sources     Water Load     Low    Most Likely    High    Loading %

(m^3/year)  (kg/year)  (kg/year)   (kg/year)          _

SEPTIC TANK DATA

Description                                        Low    Most Likely   High     Loading %

Septic Tank Output (kg/capita-year)                0.30        0.50     0.80

# capita-years                        164.4

% Phosphorus Retained by Soil                      98.0        90.0     80.0

Septic Tank Loading (kg/year)                      0.99        8.22    26.30         0.5

TOTALS DATA

Description                      Low    Most Likely   High     Loading %

Total Loading (lb)              1669.0      3682.4      8304.2   100.0

Total Loading (kg)               757.1      1670.3      3766.8   100.0

Areal Loading (lb/ac-year)        8.02       17.70       39.92

Areal Loading (mg/m^2-year)     899.40     1984.34     4474.95

Total PS Loading (lb)              0.0         0.0         0.0     0.0

Total PS Loading (kg)              0.0         0.0         0.0     0.0

Total NPS Loading (lb)          1648.3      3608.6      8060.6    99.5

Total NPS Loading (kg)           747.7      1636.8      3656.3    99.5

Wisconsin Internal Load Estimator

Date: 3/16/2016    Scenario: 1

Method 1 - A Complete Total Phosphorus Mass Budget

Method 1 - A Complete Total Phosphorus Mass Budget 69.64 mg/m^3

Phosphorus Inflow Concentration: 100.8 mg/m^3

Areal External Loading: 1984.3 mg/m^2-year

Predicted Phosphorus Retention Coefficient: 0.40

Observed Phosphorus Retention Coefficient: 0.31

Internal Load: 327 Lb     149 kg

Method 2 - From Growing Season In Situ Phososphorus Increases

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 73.68 mg/m^3

Hypolimnetic Volume: 1175.95 acre-ft

Anoxia Sediment Area: 97.12 acres

Just Prior To The End of Stratification

Average Hypolimnetic Phosphorus Concentration: 73.68 mg/m^3

Hypolimnetic Volume: 1175.95 acre-ft

Anoxia Sediment Area: 97.12 acres

Time Period of Stratification: 90 days

Sediment Phosphorus Release Rate: 0.0 mg/m^2-day     0.00E+000 lb/acre-day

Internal Load:   0 Lb       0 kg

Method 3 - From In Situ Phososphorus Increases In The Fall

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 73.68 mg/m^3

Hypolimnetic Volume: 1175.95 acre-ft

Anoxia Sediment Area: 97.12 acres

Just Prior To The End of Stratification

Average Water Column Phosphorus Concentration: 78.8 mg/m^3

Lake Volume: 1872.0 acre-ft

Anoxia Sediment Area Just Before Turnover: 97.12 acres

Time Period Between Observations: 30 days

Sediment Phosphorus Release Rate: 6.4 mg/m^2-day     1.73E-002 lb/acre-day

Internal Load: 166 Lb      75 kg

Method 4 - From Phososphorus Release Rate and Anoxic Area

Start of Anoxia Anoxic Sediment Area: 97.12 acre

End of Anoxia Anoxic Sediment Area: 97.12 acre

Phosphorus Release Rate As Calculated In Method 2: 0.0 mg/m^2-day

Phosphorus Release Rate As Calculated In Method 3: 0.0 mg/m^2-day

Average of Methods 2 and 3 Release Rates: 3.2 mg/m^2-day

Period of Anoxia: 90 days

Default Areal Sediment Phosphorus Release Rates:

Low   Most Likely   High

6        14         24

Internal Load: (Lb)          143       333        570

Internal Load: (kg)           65       151        259

Internal Load Comparison (Percentanges are of the Total Estimate Load)

Total External Load: 3682 Lb      1670 kg

Lb         kg         %

From A Complete Mass Budget:                              327       149       8.2

From Growing Season In Situ Phosphorus Increases:           0         0       0.0

From In Situ Phososphorus Increases In The Fall:          166        75       4.3

From Phososphorus Release Rate and Anoxic Area:           333       151       8.3

Predicted Water Column Total Phosphorus Concentration (ug/l)

Nurnberg+ 1984 Total Phosphorus Model:      Low    Most Likely   High

36          63       146

Osgood, 1988 Lake Mixing Index: 3.0

Phosphorus Loading Summary:

Low      Most Likely     High

Internal Load (Lb):       327           82.8        333

Internal Load (kg):       149           37.5        151

External Load (Lb):      1669           3682       8304

External Load (kg):       757           1670       3767

Total Load (Lb):         1996           3765       8637

Total Load (kg):          906           1708       3918

Phosphorus Prediction and Uncertainty Analysis Module

Date: 3/16/2016    Scenario: 2

Observed spring overturn total phosphorus (SPO): 46.4 mg/m^3

Observed growing season mean phosphorus (GSM): 82.8 mg/m^3

Back calculation for SPO total phosphorus: 70.34 mg/m^3

Back calculation GSM phosphorus: 70.48 mg/m^3

% Confidence Range: 70%

Nurenberg Model Input - Est. Gross Int. Loading: 63 kg

Lake Phosphorus Model              Low   Most Likely   High     Predicted  % Dif.

Total P   Total P    Total P   -Observed

(mg/m^3) (mg/m^3)   (mg/m^3)   (mg/m^3)

Walker, 1987 Reservoir                         25       56        127        -27       -33

Canfield-Bachmann, 1981 Natural Lake           35       69        136        -14       -17

Canfield-Bachmann, 1981 Artificial Lake        31       57        102        -26       -31

Rechow, 1979 General                           26       56        127        -27       -33

Rechow, 1977 Anoxic                            40       87        197          4         5

Rechow, 1977 water load<50m/year               30       66        149        -17       -21

Rechow, 1977 water load>50m/year              N/A      N/A        N/A        N/A       N/A

Walker, 1977 General                           34       75        170         29        63

Vollenweider, 1982 Combined OECD               27       53        102        -12       -19

Dillon-Rigler-Kirchner                         24       53        119          7        15

Vollenweider, 1982 Shallow Lake/Res.           22       45         91        -20       -31

Larsen-Mercier, 1976                           33       73        166         27        58

Nurnberg, 1984 Oxic                            31       64        141        -19       -23

Lake Phosphorus Model          Confidence Confidence  Parameter    Back       Model

Lower      Upper      Fit?    Calculation   Type

Bound      Bound               (kg/year)

Walker, 1987 Reservoir                       32        104         FIT      2096       GSM

Canfield-Bachmann, 1981 Natural Lake         21        199         FIT      1685       GSM

Canfield-Bachmann, 1981 Artificial Lake      18        164         FIT      2189       GSM

Rechow, 1979 General                         31        105         FIT      2090       GSM

Rechow, 1977 Anoxic                          50        160         FIT      1347       GSM

Rechow, 1977 water load<50m/year             36        123           P      1783       GSM

Rechow, 1977 water load>50m/year            N/A        N/A         N/A       N/A       N/A

Walker, 1977 General                         36        148         FIT      1558       SPO

Vollenweider, 1982 Combined OECD             25        100         FIT      2393       ANN

Dillon-Rigler-Kirchner                       30         97           P      2234       SPO

Vollenweider, 1982 Shallow Lake/Res.         21         85         FIT      2842       ANN

Larsen-Mercier, 1976                         43        134       P Pin      1601       SPO

Nurnberg, 1984 Oxic                          33        122           P      1836       ANN

Water and Nutrient Outflow Module

Date: 3/16/2016    Scenario: 1

Average Annual Surface Total Phosphorus: 69.64mg/m^3

Annual Discharge: 1.34E+004 AF => 1.66E+007 m^3

Annual Outflow Loading:    2436.5 LB =>    1105.2 kg

Expanded Trophic Response Module

Date: 3/16/2016    Scenario: 1

Total Phosphorus:    69.78 mg/m^3

Growing Season

Chorophyll a:        93.90 mg/m^3

Secchi Disk Depth:    1.75 m

Cholorphyll a Nuisance Frequency

Chla Mean Min: 5

Chla Mean Max: 100

Chla Mean Increment: 5

Chla Temporal CV: 0.62

Chla Nuisance Criterion: 20

Mean    Freq %    ml        z        v        w        x

5       0.5    1.4      2.546    0.016    0.541    0.005

10       7.7    2.1      1.428    0.144    0.678    0.077

15      21.9    2.5      0.774    0.296    0.795    0.219

20      37.8    2.8      0.310    0.380    0.907    0.378

25      52.0    3.0     -0.050    0.398    0.984    0.480

30      63.5    3.2     -0.344    0.376    0.897    0.365

35      72.3    3.4     -0.593    0.335    0.835    0.277

40      79.0    3.5     -0.808    0.288    0.788    0.210

45      84.1    3.6     -0.998    0.242    0.751    0.159

50      87.9    3.7     -1.168    0.202    0.720    0.121

55      90.7    3.8     -1.322    0.167    0.695    0.093

60      92.8    3.9     -1.462    0.137    0.673    0.072

65      94.4    4.0     -1.591    0.112    0.654    0.056

70      95.6    4.1     -1.711    0.092    0.637    0.044

75      96.6    4.1     -1.822    0.076    0.623    0.034

80      97.3    4.2     -1.926    0.062    0.609    0.027

85      97.8    4.3     -2.024    0.051    0.598    0.022

90      98.3    4.3     -2.116    0.043    0.587    0.017

95      98.6    4.4     -2.203    0.035    0.577    0.014

100      98.9    4.4     -2.286    0.029    0.568    0.011