Date: 3/4/2016    Scenario: 9

Lake Id: Big Blake Lake

Watershed Id: 0

Hydrologic and Morphometric Data

Tributary Drainage Area: 2150.7 acre

Total Unit Runoff: 8.00 in.

Annual Runoff Volume: 1433.8 acre-ft

Lake Surface Area <As>: 208.0 acre

Lake Volume <V>: 1872.0 acre-ft

Lake Mean Depth <z>: 9.0 ft

Precipitation - Evaporation: 3.3 in.

Hydraulic Loading: 1491.0 acre-ft/year

Areal Water Load <qs>: 7.2 ft/year

Lake Flushing Rate <p>: 0.80 1/year

Water Residence Time: 1.26 year

Observed spring overturn total phosphorus (SPO): 0.0 mg/m^3

Observed growing season mean phosphorus (GSM): 0.0 mg/m^3

% NPS Change: 0%

% PS Change: 0%

NON-POINT SOURCE DATA

Land Use        Acre        Low    Most Likely    High    Loading %   Low    Most Likely    High

(ac)     |---- Loading (kg/ha-year) ----|            |-----  Loading (kg/year) ----|

Row Crop AG           390.5       0.50       1.00       3.00       52.3         79        158        474

Mixed AG                0.0       0.30       0.80       1.40        0.0          0          0          0

Pasture/Grass         159.2       0.10       0.30       0.50        6.4          6         19         32

HD Urban (1/8 Ac)       0.0       1.00       1.50       2.00        0.0          0          0          0

MD Urban (1/4 Ac)     240.0       0.30       0.50       0.80       16.1         29         49         78

Rural Res (>1 Ac)       9.7       0.05       0.10       0.25        0.1          0          0          1

Wetlands              347.8       0.10       0.10       0.10        4.7         14         14         14

Forest               1003.6       0.05       0.09       0.18       12.1         20         37         73

Lake Surface          208.0       0.10       0.30       1.00        8.4          8         25         84

POINT SOURCE DATA

Point Sources     Water Load     Low    Most Likely    High    Loading %

(m^3/year)  (kg/year)  (kg/year)   (kg/year)          _

SEPTIC TANK DATA

Description                                        Low    Most Likely   High     Loading %

Septic Tank Output (kg/capita-year)                0.30        0.50     0.80

# capita-years                          0.0

% Phosphorus Retained by Soil                      98.0        90.0     80.0

Septic Tank Loading (kg/year)                      0.00        0.00     0.00         0.0

TOTALS DATA

Description                      Low    Most Likely   High     Loading %

Total Loading (lb)               347.4       666.2      1667.4   100.0

Total Loading (kg)               157.6       302.2       756.3   100.0

Areal Loading (lb/ac-year)        1.67        3.20        8.02

Areal Loading (mg/m^2-year)     187.22      359.01      898.54

Total PS Loading (lb)              0.0         0.0         0.0     0.0

Total PS Loading (kg)              0.0         0.0         0.0     0.0

Total NPS Loading (lb)           328.9       610.5      1481.9   100.0

Total NPS Loading (kg)           149.2       276.9       672.2   100.0