Date: 7/26/2016    Scenario: Big Blake Lake combined direct drainage

Lake Id: Big Blake Lake

Watershed Id: 0

Hydrologic and Morphometric Data

Tributary Drainage Area: 550.0 acre

Total Unit Runoff: 8.00 in.

Annual Runoff Volume: 366.7 acre-ft

Lake Surface Area <As>: 208.0 acre

Lake Volume <V>: 1872.0 acre-ft

Lake Mean Depth <z>: 9.0 ft

Precipitation - Evaporation: 3.3 in.

Hydraulic Loading: 169846.7 acre-ft/year

Areal Water Load <qs>: 816.6 ft/year

Lake Flushing Rate <p>: 90.73 1/year

Water Residence Time: 0.01 year

Observed spring overturn total phosphorus (SPO): 36.8 mg/m^3

Observed growing season mean phosphorus (GSM): 57.0 mg/m^3

% NPS Change: 0%

% PS Change: 0%

NON-POINT SOURCE DATA

Land Use        Acre        Low    Most Likely    High    Loading %   Low    Most Likely    High

(ac)     |---- Loading (kg/ha-year) ----|            |-----  Loading (kg/year) ----|

Row Crop AG           123.0       0.50       1.00       3.00        0.5         25         50        149

Mixed AG                0.0       0.30       0.80       1.40        0.0          0          0          0

Pasture/Grass          38.0       0.10       0.30       0.50        0.0          2          5          8

HD Urban (1/8 Ac)      19.0       1.00       1.50       2.00        0.1          8         12         15

MD Urban (1/4 Ac)     113.0       0.30       0.50       0.80        0.2         14         23         37

Rural Res (>1 Ac)      46.0       0.05       0.10       0.25        0.0          1          2          5

Wetlands               35.0       0.10       0.10       0.10        0.0          1          1          1

Forest                176.0       0.05       0.09       0.18        0.1          4          6         13

Lake Surface          208.0       0.10       0.30       1.00        0.2          8         25         84

POINT SOURCE DATA

Point Sources     Water Load     Low    Most Likely    High    Loading %

(m^3/year)  (kg/year)  (kg/year)   (kg/year)          _

SEPTIC TANK DATA

Description                                        Low    Most Likely   High     Loading %

Septic Tank Output (kg/capita-year)                0.30        0.50     0.80

# capita-years                        164.4

% Phosphorus Retained by Soil                      98.0        90.0     80.0

Septic Tank Loading (kg/year)                      0.99        8.22    26.30         0.1

TOTALS DATA

Description                      Low    Most Likely   High     Loading %

Total Loading (lb)               139.2     23580.3       745.9   100.0

Total Loading (kg)                63.1     10696.0       338.4   100.0

Areal Loading (lb/ac-year)        0.67      113.37        3.59

Areal Loading (mg/m^2-year)      75.02    12706.86      401.97

Total PS Loading (lb)              0.0     23289.4         0.0    98.8

Total PS Loading (kg)              0.0     10564.0         0.0    98.8

Total NPS Loading (lb)           118.5       217.1       502.4     1.2

Total NPS Loading (kg)            53.7        98.5       227.9     1.2

Wisconsin Internal Load Estimator

Date: 7/26/2016    Scenario: 24

Method 1 - A Complete Total Phosphorus Mass Budget

Method 1 - A Complete Total Phosphorus Mass Budget 49.85 mg/m^3

Phosphorus Inflow Concentration: 51.1 mg/m^3

Areal External Loading: 12706.9 mg/m^2-year

Predicted Phosphorus Retention Coefficient: 0.06

Observed Phosphorus Retention Coefficient: 0.02

Internal Load: 769 Lb     349 kg

Method 2 - From Growing Season In Situ Phososphorus Increases

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 62.3 mg/m^3

Hypolimnetic Volume: 715.6 acre-ft

Anoxia Sediment Area: 120.4 acres

Just Prior To The End of Stratification

Average Hypolimnetic Phosphorus Concentration: 70 mg/m^3

Hypolimnetic Volume: 595.61 acre-ft

Anoxia Sediment Area: 108.82 acres

Time Period of Stratification: 50 days

Sediment Phosphorus Release Rate: -0.2 mg/m^2-day     -4.18E-004 lb/acre-day

Internal Load:  -8 Lb      -4 kg

Method 3 - From In Situ Phososphorus Increases In The Fall

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 62.3 mg/m^3

Hypolimnetic Volume: 715.6 acre-ft

Anoxia Sediment Area: 120.4 acres

Just Prior To The End of Stratification

Average Water Column Phosphorus Concentration: 62.3 mg/m^3

Lake Volume: 1872.0 acre-ft

Anoxia Sediment Area Just Before Turnover: 108.82 acres

Time Period Between Observations: 30 days

Sediment Phosphorus Release Rate: 6.4 mg/m^2-day     1.74E-002 lb/acre-day

Internal Load: 196 Lb      89 kg

Method 4 - From Phososphorus Release Rate and Anoxic Area

Start of Anoxia Anoxic Sediment Area: 120.4 acre

End of Anoxia Anoxic Sediment Area: 108.82 acre

Phosphorus Release Rate As Calculated In Method 2: -0.2 mg/m^2-day

Phosphorus Release Rate As Calculated In Method 3: -0.2 mg/m^2-day

Average of Methods 2 and 3 Release Rates: 3.1 mg/m^2-day

Period of Anoxia: 50 days

Default Areal Sediment Phosphorus Release Rates:

Low   Most Likely   High

6        14         24

Internal Load: (Lb)           93       218        374

Internal Load: (kg)           42        99        170

Internal Load Comparison (Percentanges are of the Total Estimate Load)

Total External Load: 23580 Lb      10696 kg

Lb         kg         %

From A Complete Mass Budget:                              769       349       3.2

From Growing Season In Situ Phosphorus Increases:          -8        -4       0.0

From In Situ Phososphorus Increases In The Fall:          196        89       0.8

From Phososphorus Release Rate and Anoxic Area:           218        99       0.9

Predicted Water Column Total Phosphorus Concentration (ug/l)

Nurnberg+ 1984 Total Phosphorus Model:      Low    Most Likely   High

2          48         2

Osgood, 1988 Lake Mixing Index: 3.0

Phosphorus Loading Summary:

Low      Most Likely     High

Internal Load (Lb):       769           94.0        218

Internal Load (kg):       349           42.7         99

External Load (Lb):       139          23580        746

External Load (kg):        63          10696        338

Total Load (Lb):          908          23674        964

Total Load (kg):          412          10739        437

Phosphorus Prediction and Uncertainty Analysis Module

Date: 7/26/2016    Scenario: 18

Observed spring overturn total phosphorus (SPO): 36.8 mg/m^3

Observed growing season mean phosphorus (GSM): 57.0 mg/m^3

Back calculation for SPO total phosphorus: 37.14 mg/m^3

Back calculation GSM phosphorus: 57.24 mg/m^3

% Confidence Range: 70%

Nurenberg Model Input - Est. Gross Int. Loading: 48 kg

Lake Phosphorus Model              Low   Most Likely   High     Predicted  % Dif.

Total P   Total P    Total P   -Observed

(mg/m^3) (mg/m^3)   (mg/m^3)   (mg/m^3)

Walker, 1987 Reservoir                          0       47          1        -10       -18

Canfield-Bachmann, 1981 Natural Lake            0       47          2        -10       -18

Canfield-Bachmann, 1981 Artificial Lake         0       43          2        -14       -25

Rechow, 1979 General                            0       41          1        -16       -28

Rechow, 1977 Anoxic                             0       45          1        -12       -21

Rechow, 1977 water load<50m/year              N/A      N/A        N/A        N/A       N/A

Rechow, 1977 water load>50m/year                0       36          1        -21       -37

Walker, 1977 General                            0       46          1          9        24

Vollenweider, 1982 Combined OECD                1       36          2        -11       -23

Dillon-Rigler-Kirchner                          0       48          2         11        30

Vollenweider, 1982 Shallow Lake/Res.            0       30          1        -17       -36

Larsen-Mercier, 1976                            0       46          1          9        24

Nurnberg, 1984 Oxic                             1       48          2         -9       -16

Lake Phosphorus Model          Confidence Confidence  Parameter    Back       Model

Lower      Upper      Fit?    Calculation   Type

Bound      Bound               (kg/year)

Walker, 1987 Reservoir                       16         80          Tw     12997       GSM

Canfield-Bachmann, 1981 Natural Lake         15        135           L     12877       GSM

Canfield-Bachmann, 1981 Artificial Lake      13        124         FIT     14320       GSM

Rechow, 1979 General                         13         71          qs     14949       GSM

Rechow, 1977 Anoxic                          15         76         FIT     13573       GSM

Rechow, 1977 water load<50m/year            N/A        N/A         N/A       N/A       N/A

Rechow, 1977 water load>50m/year             14         57         FIT     16923       GSM

Walker, 1977 General                         13         84         FIT      8609       SPO

Vollenweider, 1982 Combined OECD             10         68          Tw     14943       ANN

Dillon-Rigler-Kirchner                       16         81    P L qs p      8226       SPO

Vollenweider, 1982 Shallow Lake/Res.          8         56          Tw     18320       ANN

Larsen-Mercier, 1976                         16         77     P Pin p      8598       SPO

Nurnberg, 1984 Oxic                          14         86        L qs     12655       ANN