Brush Creek – Pine River TWA

Watershed Evaluation Final Project Report

Jean Unmuth – DNR Water Resources Specialist/Author

UW-Platteville Interns: Joshua Delvaux, Christin Selle

Citizen Monitor: Jim Korb

05/17/2016

Overview, Monitoring Sites & Methods

Streams with permanent flow located within the Brush Creek – Pine River sub-watershed (HUC12 - 070700051107), in Richland County were monitored by DNR for a targeted watershed project between 2013 and 2014.  The watershed is 25,341 acres in size, and land use is primarily agriculture and forested, although development in the City of Richland Center has occurred along the lower segments of all but one of the streams.  Streams monitored were Brush creek (WBIC: 1226200), Pier Spring Creek (WBIC: 1226500), which is a tributary of Brush Creek, Center Creek (WBIC: 1225800) and an unnamed tributary of Center Cr. (WBIC: 1225900).  The streams are classified as either cold water or cool-cold water natural communities, and standard DNR protocols and methods were used to assess quantitative or qualitative habitat, fish, water quality, macroinvertebrates and continuous stream temperature at various SWIMS stations (Table 1).  This report summarizes applicable stream monitoring data that were collected over a 10 year period, between 2004 and 2014. The objective was to determine if streams are meeting designated uses and evaluate stream conditions, with a goal of developing strategies to improve stream water quality.

Table 1.  2004 –2014 stream site monitoring locations from upstream to downstream.

| Stream Name | SWIMS Station | SWIMS Station Location |
| --- | --- | --- |
| Brush Creek | 10031497 | US of Buckhorn Ln. |
| Brush Creek | 10031496 | US of Vivians Rd. |
| Brush Creek | 533073 | US of Covered Bridge Rd. |
| Brush Creek | 10031495 | US of High School Driveway |
| Brush Creek | 10041730 | US of Bancroft Dr. |
| Pier Spring Creek | 10042346 | Pier Spring Rd. |
| Pier Spring Creek | 10031753 | US of STH-14 |
| Center Creek | 10042283 | DS Hyatt Springs Ln. |
| Center Creek | 10020910 | US of Norman Dr. |
| Center Cr. | 10040189 | US of S. Cairns Ave. |
| Unnamed Trib. of Center Cr. WBIC: 1225900 | 10041733 | US of Mouth near S. Cairns Ave. |

Brush Creek and Pier Spring Creek

Brush Creek is 4.04 miles long, and Pier Spring Creek, a tributary of Brush Creek is 1.62 miles long.  Both are designated as a cold water class II trout streams.  Aerial photographs along with physical stream characteristics indicate Brush Creek has been historically ditched and disconnected from the floodplain between Vivians lane and Covered Bridge Road, in the mid-stream reach, and appears to be entrenched in the lower stream reaches.  Pier Spring Creek was also historically ditched in the lower segment near the mouth, and in the upper segment near the headwaters.  Continuous temperature monitoring of Brush Creek was conducted between June 2 and September 13, 2014 near the mouth at Bancroft Drive, in the coldwater natural community stream reach.  The maximum daily mean temperature was below 22.0 degrees C, indicating stream temperatures can support coldwater fishes.  Although continuous temperature was not monitored in Pier Spring Creek, instantaneous summer grab samples showed temperatures of below 14 degrees C.

The coldwater fish indices of biotic integrity (FIBI) at all sites in Brush Creek indicated the ratings were only fair, with scores ranging from 30 to 40 (Table 2).  Overall throughout Brush Creek, eleven different species of fish were caught, with brook stickleback (Culaea inconstans), western blacknose dace (Rhinichthys obtusus), brook trout (Salvelinus fontinalis), white sucker (Catostomus commersonii), and brown trout (Salmo trutta), dominating the catch (Table 3).  More brook trout were caught than brown trout, and both trout species appear to be naturally reproducing.  Brook trout were captured from the headwaters down to the stream mouth.  The stream has been stocked with brown trout for 37 years while brook trout have been stocked only twice, according to the fish stocking database.  FIBI ratings for Pier Spring Creek were good to excellent, with scores ranging from 70-90.  Brook trout dominated the catch, with nearly equal numbers of young of the year fish to adults, indicating good reproduction.  The stream is not stocked according to the fish stocking database.  Other fish captured were brown trout and brook stickleback in very low numbers. Verification of the stream natural community indicates that they are correctly classified as cool-cold water.  The fair FIBI rating indicate that Brush Creek has some degradation, mainly from heavily grazed riparian areas, cattle trampling the banks and loafing in the stream, but also likely from cropland runoff with very narrow riparian buffers. Much of the stream has been historically ditched from Hive Drive up to Buckhorn Lane, thus vertical eroding banks and in-stream sediment has exacerbated stream degradation.

Table 2. Brush Creek and Pier Spring Creek fish IBI coldwater ratings and scores from upstream to downstream.

| Stream Name/ SWIMS Station | Station Location | Year | FIBI Ratings | FIBI Scores |
| --- | --- | --- | --- | --- |
| Brush Cr./10031497 | US of Buckhorn Ln. | 2014 | Fair | 30 |
| Brush Cr./10031496 | US of Vivians Rd. | 2010 | Fair | 40 |
| Brush Cr./533073 | US Covered Bridge Rd. | 2013 | Fair | 40 |
| Brush Cr./10031495 | US High School Driveway | 2010 | Fair | 40 |
| Brush Cr./10041730 | US Bancroft Dr. | 2014 | Fair | 40 |
| Pier Spring/10042346 | Pier Spring Rd. | 2014 | Good | 80 |
| Pier Spring/10031753 | STH-14 | 2010 | Excellent | 90 |
| Pier Spring/10031753 | STH-14 | 2012 | Good | 70 |

Table 3. Fish Caught in Brush Creek and Pier Spring Creek.

| Common Name | Scientific Name | Number Caught |
| --- | --- | --- |
| Bluegill | Lepomis macrochirus | 13 |
| Brook Stickleback | Culaea inconstans | 102 |
| Brook Trout | Salvelinus fontinalis | 276 |
| Brown Trout | Salmo trutta | 58 |
| Creek Chub | Semotilus atromaculatus | 13 |
| Fantail Darter | Etheostoma flabellare | 2 |
| Mottled Sculpin | Cottus bairdii | 38 |
| Southern Brook Lamprey | Ichthyomyzon gagei | 16 |
| Lamprey spp. | Ichtyomyzon | 4 |
| Western Blacknose Dace | Rhinichthys obtusus | 75 |
| White Sucker | Catostomus commersonii | 134 |

Quantitative or qualitative habitat in Brush Creek rated good in the headwaters and near the mouth, but only fair in the middle stream segment (Table 4).    Pier Spring habitat was collected from a segment near the mouth and rated good.   Brush Creek banks were relatively stable near the headwaters, had a fair amount of erosion in the middle segment, and were highly unstable in the lower reaches (Table 5).  The immediate headwater riparian area appeared to be well buffered with vegetation, but from Buck Horn Lane downstream to below Vivians Lane vegetated buffers were virtually non-existent.  Vegetated buffers were only fair at Covered Bridge Road with cropland and pasture outside the buffer, while at Bancroft Lane, the stream had a minimal vegetated riparian buffer that was mostly mowed or developed, and the stream was deeply entrenched with vertical eroding banks.  Percent fines were very low in the headwaters, and high in the middle and lower stream segments.  While the lower segment near Bancroft Drive had poor riparian habitat and the stream highly erosive, conversely this stretch had the most diversity of riffle, run and pool habitat, while the middle segment was nearly all a run, and the upper segment was riffles and runs.  Fish cover was also good in the lower segment with an abundance of woody debris, only fair near Covered Bridge Road due to excessive beds of the invasive aquatic plant curly-leaf pondweed (Potamogeton crispus), and some undercut banks, while fish cover near the headwaters was limited to overhanging shrubs. Habitat from Pier Spring Creek indicated that the mainly forested vegetative buffer was good, banks were not eroded, and a fair amount of fish cover was present in the form of woody debris.

Table 4. Brush Creek and Pier Spring Creek quantitative/qualitative habitat ratings from upstream to downstream.

| Stream Name/ SWIMS Station | Station Location | Year | Habitat Rating | Habitat Score |
| --- | --- | --- | --- | --- |
| Brush Cr./10031497 | US Buck Horn Ln. | 2014 | Good | 50 |
| Brush Cr./533073 | US covered Bridge Rd. | 2013 | Fair | 40 |
| Brush Cr./10041730 | US Bancroft Dr. | 2014 | Good | 50 |
| Pier Spring Cr./10042346 | Pier Spring Rd. | 2014 | Good | 70 |

Table 5. Brush Creek and Pier Spring Creek qualitative and quantitative habitat summary.

| SWIMS Station | Mean Stream Width | Main Habitat Type | Bank Erosion | Vegetative Buffer | Amount/Type Fish Cover | Riparian Habitat Type |
| --- | --- | --- | --- | --- | --- | --- |
| 10031497 | 1.4 | Riffle/Run | Good | Excellent | Low Overhanging | Meadow Shrubs |
| 533073 | 3.7 | Run | Fair | Good | Moderate Undercuts Vegetation | Meadow Cropland Pasture |
| 10041730 | 3.8 | Pool/Riffle/ Run | Poor | Fair | Abundant Woody debris | Mowed Developed |
| 10042346 | 1.9 | Pool/Riffle/ Run | Good | Good | Moderate Woody debris | Forested Cropland |

Macroinvertebrate samples were collected during fall from two sites in Brush Creek and one site in Pier Spring Creek between 2013 and 2014, and macroinvertebrate (MIBI) and family level biotic integrity (FBI) were calculated.  Brush Creek results indicate MIBI and FBI rated fair with some organic pollution at Covered Bridge Road and a rating of good at Bancroft Drive (Figure 3). Conversely, Pier spring Creek rated excellent.

Figure 3. Brush Creek and Pier Spring Creek Macroinvertebrate and Family Level Biotic Integrity.

Water quality results indicate both the streams have relatively good water quality.  Six monthly phosphorus samples were collected from Brush Creek at Bancroft Drive near the stream mouth, and median  phosphorus levels calculated at 0.0552 mg/l, indicating the stream did not exceed the state phosphorus criteria of 0.75 mg/l.  A single sample of phosphorus was also collected from Brush Creek near the headwaters at Buck Horn Lane, and from Pier Spring Creek near the stream mouth and levels were low at 0.033 mg/l and 0.0216 mg/l, respectively, therefore no additional sampling of phosphorus was warranted. Total Nitrogen, total suspended solids were at low levels in Brush and Pier Spring Creeks (Figures 4 & 5).  pH was within the acceptable range between 7.9-8.0, and transparency was good to excellent, ranging from 80 – 120 centimeters.

Figure 4. Brush Creek and Pier Spring Creek Total Nitrogen

Figure 5. Brush Creek and Pier Spring Creek Total Suspended Solids

Center Creek and Unnamed Tributary WBIC: 1255900

Center Creek is 2.57 miles long, and the Unnamed Tributary of Center Cr. is 1.73 miles long.  The streams have a designated use of Fish and Aquatic Life waters by default, and the natural community designation of both streams are cool-cold headwaters but a short segment in the upper segment of both streams are listed as coldwater.  Aerial photographs, and physical stream characteristics indicate that extensive ditching was done historically in both streams.  In some ditched segments, the streams are disconnected from the floodplain    Continuous temperature monitoring of Center Creek was conducted between June 2 and September 19, 2014 at Norman Drive.  Maximum daily mean temperature was below 22.0 degrees C, indicating stream temperatures can support support coldwater fishes.  A single water temperature was collected on June 15, 2015 from the unnamed tributary and was below 13.0 degrees C.  The cool-cold water FIBI ratings for Center Creek ranged from good near the headwaters to excellent at Cairns Ave. near the mouth  (Table 6).  However, in the middle stream segment at Norman Drive, too few fish were caught to calculate the FIBI, and the 2007 FIBI at that site rated poor.  The natural community verification indicated too many tolerant fish were caught at Hyatt Ln. and Norman Dr. indicating that the stream is degraded, even though temperatures appear to be cold enough to support the cool-cold natural community designation.  The FIBI for the unnamed tributary could not be calculated as only 3 fish were caught in a 120 meter stream segment.  Overall throughout Center Creek, twelve different species of fish were caught, with western blacknose dace (Rhinichthys obtusus), Brassy Minnow (Hybognathus hankinsoni). white sucker (Catostomus commersonii), and creek chub (Semotilus atromaculatus) dominating the catch (Table 7.).  A few brook trout (Salvelinus fontinalis) were caught only at the downstream most site at Cairns Avenue, and they ranged in length from 4.5 – 8.2 inches.  No brown trout (Salmo trutta) were caught.  No fish have been stocked in Center Creek, according to the fish stocking database.  Natural community verification indicates Center Creek is correctly classified as cool-cold water, but too many pollution tolerant fish were found at all but the Cairns Avenue site.

Table 6. Center Creek and Unnamed Tributary fish cool-cold IBI ratings from upstream to downstream.

| Stream Name/ SWIMS Station | Station Location | Year | FIBI Rating | FIBI Score |
| --- | --- | --- | --- | --- |
| Center Cr./ 10042283 | DS Hyatt Ln. | 2014 | Good | 50 |
| Center Cr. 10020910 | US Norman Dr. | 2007 | Poor | 20 |
| Center Cr./ 10020910 | US Norman Dr. | 2013 | N/A* | N/A |
| Center Cr./ 10040189 | US S. Cairns Ave. | 2014 | Excellent | 100 |
| Unnamed Tributary of Center/10041733 | US of Mouth near S. Cairns Ave. | 2014 | N/A* | N/A |

*Could not calculate the FIBI due to low numbers of fish collected.

Table 7.  Fish collected from Center Creek and Unnamed Tributary.

| Common Name | Scientific Name | Number Caught |
| --- | --- | --- |
| Bluegill | Lepomis macrochirus | 15 |
| Brassy Minnow | Hybognathus hankinsoni | 34 |
| Brook Stickleback | Culaea inconstans | 5 |
| Brook Trout | Salvelinus fontinalis | 5 |
| Central Mudminnow | Umbra limi | 1 |
| Creek Chub | Semotilus atromaculatus | 37 |
| Green Sunfish | Lepomis cyanellus | 1 |
| Iowa Darter | Etheostoma exile | 1 |
| Mottled Sculpin | Cottus bairdii | 5 |
| Southern Brook Lamprey | Ichthyomyzon gagei | 5 |
| Western Blacknose Dace | Rhinichthys obtusus | 96 |
| White Sucker | Catostomus commersonii | 31 |

Quantitative or qualitative habitat in Center Creek rated fair except for a rating of good near the mouth at South Cairns Avenue (Table 8).  The unnamed tributary of Center Creek also rated fair.  The stream segment in the vicinity of Norman Drive to the stream mouth has degradation issues mainly from mowing up to the top of the bank, historical ditching, vertical sloughing banks and in stream sediment.  The stream is disconnected from the floodplain  Further upstream in the vicinity of Lime Kiln Drive and on up to Hyatt Lane, the stream is degraded from excessive grazing of the riparian area, cattle trampling the banks and loafing in the stream.  Stream banks within the city of Richland Center are unstable, and had fair to excessive erosion (Table 9).  Fine sediments were common to excessive in runs, riffles and pools.  Both streams appeared to have a diversity of riffle, run, and pool habitat, but fish cover was generally low in Center Creek, but abundant in the tributary stream.

Table 8.  Center Creek and Unnamed Tributary quantitative/qualitative habitat ratings from upstream to downstream.

| Stream Name/ SWIMS Station | Station Location | Year | Habitat Rating | Habitat Score |
| --- | --- | --- | --- | --- |
| Center Cr./10042283 | DS Hyatt Springs Ln. | 2014 | Fair | 35 |
| Center Cr./110020910 | US Norman Dr. | 2013 | Fair | 48 |
| Center Cr./10040189 | US S. Cairns Ave. | 2013 | Good | 58 |
| Unnamed Trib. WBIC 1225900./10041733 | US of mouth near S. Cairns Ave. | 2014 | Fair | 48 |

Table 9. Center Creek and Unnamed Tributary habitat summary.

| SWIMS Station | Mean Stream Width | Main Habitat Type | Bank Erosion | Vegetative Buffer | Amount/Type Fish Cover | Riparian Habitat Type |
| --- | --- | --- | --- | --- | --- | --- |
| 10042283 | 1.1 | Riffle/ Run/Pool | Poor | Poor | Low | Pasture |
| 10020910 | 0.89 | Riffle/ Run/pool | Fair | Fair | Low Wood Undercut | Mowed Developed Meadow |
| 10040189 | 1.25 | Riffle/ Run/Pool | Fair | Good | Moderate Undercut | Mowed Developed Forested |
| 10041733 | 2.0 | Riffle/ Run/Pool | Poor | Fair | Abundant Wood | Mowed Developed |

Macroinvertebrate information collected from Center Creek and the unnamed tributary between 2004 and 2014 were summarized, and macroinvertebrate index to biotic integrity (MIBI) and family level biotic index were calculated.  Center Creek near the headwaters at Hyatt Lane had a fair rating indicating there is organic pollution, while all other sites rated good (Figure 6). The unnamed tributary stream rated excellent.

Figure 6. Center Creek and Unnamed Tributary Macroinvertebrate and Family Level Biotic Index.

Six monthly phosphorus samples were collected from Center Creek at Norman Drive and the median  phosphorus levels calculated at 0.083 mg/l, indicating the stream should be placed on the “watch waters” list for phosphorus.  A single sample of phosphorus was also collected from Center Creek near the headwaters at Hyatt Springs Lane, and from the unnamed tributary of Center Creek (WBIC: 1225900) near the stream mouth.  While phosphorus was high in Center Creek at Hyatt Springs Lane at 0.36 mg/l, it was low at 0.03 mg/l in the tributary. Total Nitrogen and total suspended solids were also collected from two sites in Center Creek and in the tributary of Center Creek, and both parameters were low, although the nitrogen was approaching a level of 2 mg/l at Norman Drive (Figure 7).   pH was within the acceptable range in Center Creek and the tributary a levels ranging from 7.9-8.3 SU.  Transparency in Center Creek was  excellent (>120 cm) at Hyatt Springs and South Cairns sites but water was more turbid (60 cm) at Norman Drive.  Transparency was also excellent in the unnamed tributary of Center Cr.

Figure 7. August 2014 Water Chemistry from Center Creek and the Unnamed Tributary WBIC: 1225900.

Project Summary

- Water temperatures in Brush and Center Creeks indicate the stream can support cool-cold water fishes.

- Fish and macroinvertebrate indices to biotic integrity results indicate Brush creek rates fair to good, indicating some biological degradation, while Pier Spring Creek rated good to excellent.

- Brook trout were found all along Brush Creek and in Pier Spring Creek.

- Fish and macroinvertebrate indices to biotic integrity results indicate Center creek rated good to excellent except at the Norman drive site where the fish IBI rated poor and at Hyatt Springs Lane where the MIBI rated fair, indicating localized biological degradation, mainly in the form of organic pollution at Hyatt Springs Lane, but at Norman Creek Drive degradation appears to be related to habitat with erosion, sedimentation and lack of fish habitat.  The unnamed tributary WBIC 1225900 an FIBI could not be calculated since only three fish were caught.  Only five brook trout were found in Brush Creek, in the vicinity of S. Cairns Drive.

- The fish natural communities were assessed and verification indicates the streams are cool-cold but there are some tolerant fish in Brush Creek, and many tolerant fish in Center Creek, indicating there is some environmental degradation.

- Pier Spring Creek does not appear to have environmental degradation but there is noticeable bank erosion in the lower segment adjacent to STH-14, where it is excessively grazed, trampled by livestock and lacks a riparian buffer.

- Habitat conditions in Brush Creek rated good, except in the vicinity of Covered Bridge Road where it rated only fair.

- Habitat conditions in Center Creek and the unnamed tributary rated only fair, with the exception of a good rating at S. Cairns Ave.

- The water quality condition of Brush and Pier Spring Creeks is good, and phosphorus levels meet the state standard.

- The water quality condition of Center Creek is fair, and the stream is on the “Watch Water” list due to higher levels of instream phosphorus.

- Brush Creek and Pier Spring Creeks are meeting their designated uses as cold water class II trout, while Center Creek and the unnamed Tributary WBIC: 1225900 are not meeting their designated uses.

Project Recommendations/Management Actions

- Reclassify the unnamed tributary of Center Creek as a macroinvertebrate stream.  The stream channel is small, base flow is intermittent and only 0.019 cubic meters/second.

- DNR should list Center Creek as impaired biologically, specifically because of a poor fish IBI rating at Norman Drive in 2007 and too few fish captured at the same location during the 2013 survey.

- Brush Creek habitat improvement should be done in the vicinity of Covered Bridge Road.

- Habitat improvement should be done throughout Center Creek, specifically provide broader vegetative stream buffers, stabilize the banks, reduce sediment and nutrients from non-point sources.

- Reconnect of Center Creek to the floodplain where feasible through bank sloping and stabilization would help reduce stream bank erosion.

- Instream fish habitat should also be done throughout Center Creek, as the stream lacks cover for adult trout.

- Livestock were observed loafing in both Brush Cr. and Center Creek, and there are specific areas where cattle have eroded the banks on Livestock fencing and cattle crossings could help reduce erosion, sedimentation and non-point source nutrient pollution.