Water Quality Monitoring

Special Project Workplan Proposal - FY 2007

GENERAL INFORMATION

Author Name:	 Sorge, Michael		Telephone:	608.275.3247

Region: SCR					Program:  WT

Basin: Lower Wisconsin

PROJECT INFORMATION

Project Category: Category 1 			Activity Code:	 WTTM-02

Project Location(s):  Halfway Prairie Creek (8.0 miles impaired)

Watershed(s): Black Earth Creek Watershed

County or Counties: Dane

WORKPLAN DETAILS

- Project Description :

What: Monitor Halfway Prairie Creek for habitat impairments caused by sedimentation.  Conduct habitat surveys at four stations (Hudson Road, STH 78, CTH F, STH 19).

Conduct Fish IBI’s at four sites previously mentioned above.

Collect Macroinvertebrates at four locations previously mentioned above.

Deploy TIDBITS at four locations previously mentioned above.

Meet with county to go over inventory records and past BMP work as well as sediment sources.  Review USLE, RUSLE, and BARNY records.

Conduct watershed evaluation of current crop rotations, barnyards, slopes susceptible to erosion (rill/ gully).

When: Summer of 2006-June through 2007 June

Where: Halfway Prairie Creek, Black Earth Creek Watershed

Who:  Michael Sorge- Water Quality Biologist

2. Justification (why is this project necessary?):

Halfway Prairie Creek was identified as one of five TMDL’s selected by South Central Region.  This stream is currently on the 303d list for habitat with sediment being the pollutant.  Halfway Prairie Creek was part of the original Black Earth Creek Priority Watershed (1986-98), it has 8.0 miles listed as impaired.  This area has had a significant amount of resources allocated towards sediment reduction.  Data will be compared against pre-BMP work.  Basin plans list the potential for Halfway Prairie as COLD III, temperature and fish data will be used to determine if the potential is correctly listed.

3. Performance Measure(s) (including sample collection, report completion date, etc.):

Habitat HI- 4 locations listed in #1 above.

Fish IBI’s- IBI’s and trout frequency data will be collected at 4 locations.

Macroinvertebrates HBI’s- collected at 4 locations listed in #1.

Report will be generated in the winter of 2006-07.

4. Data Storage System (work station computer, lab portal, SWIMS, FH database, etc.):

Fish IBI data and Habitat HI data will be entered into the FH data base.

Temp data could be put in SWIMS if the data base is ready to accept temp data sets, otherwise data will be burned to CD and stored at region in individual file.

5. File Manager Responsible for Data Entry: Michael Sorge

| Water Quality Special Project Workplan Proposal |  |  |
| --- | --- | --- |
| #### Cost Worksheet |  |  |
| ### FY 2007 |  |  |
| July 1, 2006 - June 30, 2007 |  |  |
|  | Hours | Cost ($) |
| FTE Resources | 80 |  |
| LTE Resources | 100 | 1500 |
| Line S (supplies, contracts, travel) |  | $890 $460 (bugs) |
| SLOH Lab Services (attach lab spreadsheet) |  | 0 |
| Equipment <$5,000 |  | 475 |
| Capital Equipment =>$5,000 |  | 0 |
| Subtotal DNR contributions |  | 0 |
| Partner Contributions |  | 0 |
| Total Contributions |  | 3325 |