Date: 3/17/2016    Scenario: 17

Lake Id: Big Blake Lake

Watershed Id: 1

Hydrologic and Morphometric Data

Tributary Drainage Area: 20066.1 acre

Total Unit Runoff: 8.00 in.

Annual Runoff Volume: 13377.4 acre-ft

Lake Surface Area <As>: 208.0 acre

Lake Volume <V>: 1872.0 acre-ft

Lake Mean Depth <z>: 9.0 ft

Precipitation - Evaporation: 3.3 in.

Hydraulic Loading: 13434.6 acre-ft/year

Areal Water Load <qs>: 64.6 ft/year

Lake Flushing Rate <p>: 7.18 1/year

Water Residence Time: 0.14 year

Observed spring overturn total phosphorus (SPO): 46.4 mg/m^3

Observed growing season mean phosphorus (GSM): 82.8 mg/m^3

% NPS Change: 0%

% PS Change: 0%

NON-POINT SOURCE DATA

Land Use        Acre        Low    Most Likely    High    Loading %   Low    Most Likely    High

(ac)     |---- Loading (kg/ha-year) ----|            |-----  Loading (kg/year) ----|

Row Crop AG          1023.8       0.50       1.00       3.00       24.8        207        414       1243

Mixed AG                0.0       0.30       0.80       1.40        0.0          0          0          0

Pasture/Grass        4790.6       0.10       0.30       0.50       34.8        194        582        969

HD Urban (1/8 Ac)       4.3       1.00       1.50       2.00        0.2          2          3          3

MD Urban (1/4 Ac)       0.0       0.30       0.50       0.80        0.0          0          0          0

Rural Res (>1 Ac)     691.9       0.05       0.10       0.25        1.7         14         28         70

Wetlands             1496.3       0.10       0.10       0.10        3.6         61         61         61

Forest              10759.2       0.05       0.09       0.18       23.5        218        392        784

Lake Surface          208.0       0.10       0.30       1.00        1.5          8         25         84

POINT SOURCE DATA

Point Sources     Water Load     Low    Most Likely    High    Loading %

(m^3/year)  (kg/year)  (kg/year)   (kg/year)          _

SEPTIC TANK DATA

Description                                        Low    Most Likely   High     Loading %

Septic Tank Output (kg/capita-year)                0.30        0.50     0.80

# capita-years                        164.4

% Phosphorus Retained by Soil                      98.0        90.0     80.0

Septic Tank Loading (kg/year)                      0.99        8.22    26.30         0.5

TOTALS DATA

Description                      Low    Most Likely   High     Loading %

Total Loading (lb)              1669.0      3682.4      8304.2   100.0

Total Loading (kg)               757.1      1670.3      3766.8   100.0

Areal Loading (lb/ac-year)        8.02       17.70       39.92

Areal Loading (mg/m^2-year)     899.40     1984.34     4474.95

Total PS Loading (lb)              0.0         0.0         0.0     0.0

Total PS Loading (kg)              0.0         0.0         0.0     0.0

Total NPS Loading (lb)          1648.3      3608.6      8060.6    99.5

Total NPS Loading (kg)           747.7      1636.8      3656.3    99.5

Wisconsin Internal Load Estimator

Date: 3/17/2016    Scenario: 3

Method 1 - A Complete Total Phosphorus Mass Budget

Method 1 - A Complete Total Phosphorus Mass Budget 69.64 mg/m^3

Phosphorus Inflow Concentration: 100.8 mg/m^3

Areal External Loading: 1984.3 mg/m^2-year

Predicted Phosphorus Retention Coefficient: 0.40

Observed Phosphorus Retention Coefficient: 0.31

Internal Load: 327 Lb     149 kg

Method 2 - From Growing Season In Situ Phososphorus Increases

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 73.68 mg/m^3

Hypolimnetic Volume: 1175.95 acre-ft

Anoxia Sediment Area: 97.12 acres

Just Prior To The End of Stratification

Average Hypolimnetic Phosphorus Concentration: 78.8 mg/m^3

Hypolimnetic Volume: 1175.95 acre-ft

Anoxia Sediment Area: 97.12 acres

Time Period of Stratification: 90 days

Sediment Phosphorus Release Rate: 0.2 mg/m^2-day     5.71E-004 lb/acre-day

Internal Load:  16 Lb       7 kg

Method 3 - From In Situ Phososphorus Increases In The Fall

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 73.68 mg/m^3

Hypolimnetic Volume: 1175.95 acre-ft

Anoxia Sediment Area: 97.12 acres

Just Prior To The End of Stratification

Average Water Column Phosphorus Concentration: 140 mg/m^3

Lake Volume: 1872.0 acre-ft

Anoxia Sediment Area Just Before Turnover: 97.12 acres

Time Period Between Observations: 14 days

Sediment Phosphorus Release Rate: 39.3 mg/m^2-day     1.07E-001 lb/acre-day

Internal Load: 477 Lb     216 kg

Method 4 - From Phososphorus Release Rate and Anoxic Area

Start of Anoxia Anoxic Sediment Area: 97.12 acre

End of Anoxia Anoxic Sediment Area: 97.12 acre

Phosphorus Release Rate As Calculated In Method 2: 0.2 mg/m^2-day

Phosphorus Release Rate As Calculated In Method 3: 0.2 mg/m^2-day

Average of Methods 2 and 3 Release Rates: 19.8 mg/m^2-day

Period of Anoxia: 90 days

Default Areal Sediment Phosphorus Release Rates:

Low   Most Likely   High

6        14         24

Internal Load: (Lb)          143       333        570

Internal Load: (kg)           65       151        259

Internal Load Comparison (Percentanges are of the Total Estimate Load)

Total External Load: 3682 Lb      1670 kg

Lb         kg         %

From A Complete Mass Budget:                              327       149       8.2

From Growing Season In Situ Phosphorus Increases:          16         7       0.4

From In Situ Phososphorus Increases In The Fall:          477       216      11.5

From Phososphorus Release Rate and Anoxic Area:           333       151       8.3

Predicted Water Column Total Phosphorus Concentration (ug/l)

Nurnberg+ 1984 Total Phosphorus Model:      Low    Most Likely   High

0           0         0

Osgood, 1988 Lake Mixing Index: 0

Phosphorus Loading Summary:

Low      Most Likely     High

Internal Load (Lb):         0              0          0

Internal Load (kg):         0              0          0

External Load (Lb):         0              0          0

External Load (kg):         0              0          0

Total Load (Lb):            0              0          0

Total Load (kg):            0              0          0