Date: 3/17/2016    Scenario: 2014L-THIA

Lake Id: Big Blake Lake

Watershed Id: 1

Hydrologic and Morphometric Data

Tributary Drainage Area: 20066.1 acre

Total Unit Runoff: 8.00 in.

Annual Runoff Volume: 13377.4 acre-ft

Lake Surface Area <As>: 208.0 acre

Lake Volume <V>: 1872.0 acre-ft

Lake Mean Depth <z>: 9.0 ft

Precipitation - Evaporation: 3.3 in.

Hydraulic Loading: 13434.6 acre-ft/year

Areal Water Load <qs>: 64.6 ft/year

Lake Flushing Rate <p>: 7.18 1/year

Water Residence Time: 0.14 year

Observed spring overturn total phosphorus (SPO): 38.7 mg/m^3

Observed growing season mean phosphorus (GSM): 40.6 mg/m^3

% NPS Change: 0%

% PS Change: 0%

NON-POINT SOURCE DATA

Land Use        Acre        Low    Most Likely    High    Loading %   Low    Most Likely    High

(ac)     |---- Loading (kg/ha-year) ----|            |-----  Loading (kg/year) ----|

Row Crop AG          1023.8       0.50       1.00       3.00       24.8        207        414       1243

Mixed AG                0.0       0.30       0.80       1.40        0.0          0          0          0

Pasture/Grass        4790.6       0.10       0.30       0.50       34.8        194        582        969

HD Urban (1/8 Ac)       4.3       1.00       1.50       2.00        0.2          2          3          3

MD Urban (1/4 Ac)       0.0       0.30       0.50       0.80        0.0          0          0          0

Rural Res (>1 Ac)     691.9       0.05       0.10       0.25        1.7         14         28         70

Wetlands             1496.3       0.10       0.10       0.10        3.6         61         61         61

Forest              10759.2       0.05       0.09       0.18       23.5        218        392        784

Lake Surface          208.0       0.10       0.30       1.00        1.5          8         25         84

POINT SOURCE DATA

Point Sources     Water Load     Low    Most Likely    High    Loading %

(m^3/year)  (kg/year)  (kg/year)   (kg/year)          _

SEPTIC TANK DATA

Description                                        Low    Most Likely   High     Loading %

Septic Tank Output (kg/capita-year)                0.30        0.50     0.80

# capita-years                        164.4

% Phosphorus Retained by Soil                      98.0        90.0     80.0

Septic Tank Loading (kg/year)                      0.99        8.22    26.30         0.5

TOTALS DATA

Description                      Low    Most Likely   High     Loading %

Total Loading (lb)              1669.0      3682.4      8304.2   100.0

Total Loading (kg)               757.1      1670.3      3766.8   100.0

Areal Loading (lb/ac-year)        8.02       17.70       39.92

Areal Loading (mg/m^2-year)     899.40     1984.34     4474.95

Total PS Loading (lb)              0.0         0.0         0.0     0.0

Total PS Loading (kg)              0.0         0.0         0.0     0.0

Total NPS Loading (lb)          1648.3      3608.6      8060.6    99.5

Total NPS Loading (kg)           747.7      1636.8      3656.3    99.5

Wisconsin Internal Load Estimator

Date: 3/17/2016    Scenario: 2

Method 1 - A Complete Total Phosphorus Mass Budget

Method 1 - A Complete Total Phosphorus Mass Budget 40.58 mg/m^3

Phosphorus Inflow Concentration: 100.8 mg/m^3

Areal External Loading: 1984.3 mg/m^2-year

Predicted Phosphorus Retention Coefficient: 0.40

Observed Phosphorus Retention Coefficient: 0.60

Internal Load: -734 Lb     -333 kg

Method 2 - From Growing Season In Situ Phososphorus Increases

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 44.82 mg/m^3

Hypolimnetic Volume: 44.08 acre-ft

Anoxia Sediment Area: 13.44 acres

Just Prior To The End of Stratification

Average Hypolimnetic Phosphorus Concentration: 55.8 mg/m^3

Hypolimnetic Volume: 44.08 acre-ft

Anoxia Sediment Area: 13.44 acres

Time Period of Stratification: 60 days

Sediment Phosphorus Release Rate: 0.2 mg/m^2-day     4.97E-004 lb/acre-day

Internal Load:   1 Lb       1 kg

Method 3 - From In Situ Phososphorus Increases In The Fall

Start of Anoxia

Average Hypolimnetic Phosphorus Concentration: 44.82 mg/m^3

Hypolimnetic Volume: 44.08 acre-ft

Anoxia Sediment Area: 13.44 acres

Just Prior To The End of Stratification

Average Water Column Phosphorus Concentration: 57.6 mg/m^3

Lake Volume: 1872.0 acre-ft

Anoxia Sediment Area Just Before Turnover: 13.44 acres

Time Period Between Observations: 14 days

Sediment Phosphorus Release Rate: 171.5 mg/m^2-day     4.66E-001 lb/acre-day

Internal Load: 288 Lb     131 kg

Method 4 - From Phososphorus Release Rate and Anoxic Area

Start of Anoxia Anoxic Sediment Area: 13.44 acre

End of Anoxia Anoxic Sediment Area: 13.44 acre

Phosphorus Release Rate As Calculated In Method 2: 0.2 mg/m^2-day

Phosphorus Release Rate As Calculated In Method 3: 0.2 mg/m^2-day

Average of Methods 2 and 3 Release Rates: 85.8 mg/m^2-day

Period of Anoxia: 60 days

Default Areal Sediment Phosphorus Release Rates:

Low   Most Likely   High

6        14         24

Internal Load: (Lb)           13        31         53

Internal Load: (kg)            6        14         24

Internal Load Comparison (Percentanges are of the Total Estimate Load)

Total External Load: 3682 Lb      1670 kg

Lb         kg         %

From A Complete Mass Budget:                             -734      -333     -24.9

From Growing Season In Situ Phosphorus Increases:           1         1       0.0

From In Situ Phososphorus Increases In The Fall:          288       131       7.3

From Phososphorus Release Rate and Anoxic Area:            31        14       0.8

Predicted Water Column Total Phosphorus Concentration (ug/l)

Nurnberg+ 1984 Total Phosphorus Model:      Low    Most Likely   High

7          65       138

Osgood, 1988 Lake Mixing Index: 3.0

Phosphorus Loading Summary:

Low      Most Likely     High

Internal Load (Lb):      -734          144.6         31

Internal Load (kg):      -333           65.6         14

External Load (Lb):      1669           3682       8304

External Load (kg):       757           1670       3767

Total Load (Lb):          935           3827       8335

Total Load (kg):          424           1736       3781

Phosphorus Prediction and Uncertainty Analysis Module

Date: 3/17/2016    Scenario: 3

Observed spring overturn total phosphorus (SPO): 38.7 mg/m^3

Observed growing season mean phosphorus (GSM): 40.6 mg/m^3

Back calculation for SPO total phosphorus: 39.09 mg/m^3

Back calculation GSM phosphorus: 40.99 mg/m^3

% Confidence Range: 70%

Nurenberg Model Input - Est. Gross Int. Loading: 65 kg

Lake Phosphorus Model              Low   Most Likely   High     Predicted  % Dif.

Total P   Total P    Total P   -Observed

(mg/m^3) (mg/m^3)   (mg/m^3)   (mg/m^3)

Walker, 1987 Reservoir                         25       56        127         15        37

Canfield-Bachmann, 1981 Natural Lake           35       69        136         28        69

Canfield-Bachmann, 1981 Artificial Lake        31       57        102         16        39

Rechow, 1979 General                           26       56        127         15        37

Rechow, 1977 Anoxic                            40       87        197         46       113

Rechow, 1977 water load<50m/year               30       66        149         25        62

Rechow, 1977 water load>50m/year              N/A      N/A        N/A        N/A       N/A

Walker, 1977 General                           34       75        170         36        93

Vollenweider, 1982 Combined OECD               27       53        102         13        33

Dillon-Rigler-Kirchner                         24       53        119         14        36

Vollenweider, 1982 Shallow Lake/Res.           22       45         91          5        13

Larsen-Mercier, 1976                           33       73        166         34        88

Nurnberg, 1984 Oxic                            31       65        141         24        59

Lake Phosphorus Model          Confidence Confidence  Parameter    Back       Model

Lower      Upper      Fit?    Calculation   Type

Bound      Bound               (kg/year)

Walker, 1987 Reservoir                       32        104         FIT      1219       GSM

Canfield-Bachmann, 1981 Natural Lake         21        199         FIT       893       GSM

Canfield-Bachmann, 1981 Artificial Lake      18        164         FIT      1051       GSM

Rechow, 1979 General                         31        105         FIT      1215       GSM

Rechow, 1977 Anoxic                          50        160         FIT       784       GSM

Rechow, 1977 water load<50m/year             36        123           P      1037       GSM

Rechow, 1977 water load>50m/year            N/A        N/A         N/A       N/A       N/A

Walker, 1977 General                         36        148         FIT       866       SPO

Vollenweider, 1982 Combined OECD             25        100         FIT      1202       ANN

Dillon-Rigler-Kirchner                       30         97           P      1242       SPO

Vollenweider, 1982 Shallow Lake/Res.         21         85         FIT      1493       ANN

Larsen-Mercier, 1976                         43        134       P Pin       890       SPO

Nurnberg, 1984 Oxic                          34        122           P      1020       ANN

Expanded Trophic Response Module

Date: 3/17/2016    Scenario: 2

Total Phosphorus:    40.58 mg/m^3

Growing Season

Chorophyll a:        26.55 mg/m^3

Secchi Disk Depth:    1.58 m

Cholorphyll a Nuisance Frequency

Chla Mean Min: 5

Chla Mean Max: 100

Chla Mean Increment: 5

Chla Temporal CV: 0.62

Chla Nuisance Criterion: 20

Mean    Freq %    ml        z        v        w        x

5       0.5    1.4      2.546    0.016    0.541    0.005

10       7.7    2.1      1.428    0.144    0.678    0.077

15      21.9    2.5      0.774    0.296    0.795    0.219

20      37.8    2.8      0.310    0.380    0.907    0.378

25      52.0    3.0     -0.050    0.398    0.984    0.480

30      63.5    3.2     -0.344    0.376    0.897    0.365

35      72.3    3.4     -0.593    0.335    0.835    0.277

40      79.0    3.5     -0.808    0.288    0.788    0.210

45      84.1    3.6     -0.998    0.242    0.751    0.159

50      87.9    3.7     -1.168    0.202    0.720    0.121

55      90.7    3.8     -1.322    0.167    0.695    0.093

60      92.8    3.9     -1.462    0.137    0.673    0.072

65      94.4    4.0     -1.591    0.112    0.654    0.056

70      95.6    4.1     -1.711    0.092    0.637    0.044

75      96.6    4.1     -1.822    0.076    0.623    0.034

80      97.3    4.2     -1.926    0.062    0.609    0.027

85      97.8    4.3     -2.024    0.051    0.598    0.022

90      98.3    4.3     -2.116    0.043    0.587    0.017

95      98.6    4.4     -2.203    0.035    0.577    0.014

100      98.9    4.4     -2.286    0.029    0.568    0.011