Date: 7/26/2016    Scenario: L-THIA combined 20% reduction

Lake Id: Big Blake Lake

Watershed Id: 1

Hydrologic and Morphometric Data

Tributary Drainage Area: 20066.1 acre

Total Unit Runoff: 8.00 in.

Annual Runoff Volume: 13377.4 acre-ft

Lake Surface Area <As>: 208.0 acre

Lake Volume <V>: 1872.0 acre-ft

Lake Mean Depth <z>: 9.0 ft

Precipitation - Evaporation: 3.3 in.

Hydraulic Loading: 13434.6 acre-ft/year

Areal Water Load <qs>: 64.6 ft/year

Lake Flushing Rate <p>: 7.18 1/year

Water Residence Time: 0.14 year

Observed spring overturn total phosphorus (SPO): 36.8 mg/m^3

Observed growing season mean phosphorus (GSM): 57.0 mg/m^3

% NPS Change: -20%

% PS Change: 0%

NON-POINT SOURCE DATA

Land Use        Acre        Low    Most Likely    High    Loading %   Low    Most Likely    High

(ac)     |---- Loading (kg/ha-year) ----|            |-----  Loading (kg/year) ----|

Row Crop AG          1023.8       0.50       1.00       3.00       24.7        166        331        994

Mixed AG                0.0       0.30       0.80       1.40        0.0          0          0          0

Pasture/Grass        4790.6       0.10       0.30       0.50       34.6        155        465        776

HD Urban (1/8 Ac)       4.3       1.00       1.50       2.00        0.2          1          2          3

MD Urban (1/4 Ac)       0.0       0.30       0.50       0.80        0.0          0          0          0

Rural Res (>1 Ac)     691.9       0.05       0.10       0.25        1.7         11         22         56

Wetlands             1496.3       0.10       0.10       0.10        3.6         48         48         48

Forest              10759.2       0.05       0.09       0.18       23.3        174        314        627

Lake Surface          208.0       0.10       0.30       1.00        1.9          8         25         84

POINT SOURCE DATA

Point Sources     Water Load     Low    Most Likely    High    Loading %

(m^3/year)  (kg/year)  (kg/year)   (kg/year)          _

SEPTIC TANK DATA

Description                                        Low    Most Likely   High     Loading %

Septic Tank Output (kg/capita-year)                0.30        0.50     0.80

# capita-years                        164.4

% Phosphorus Retained by Soil                      98.0        90.0     80.0

Septic Tank Loading (kg/year)                      0.99        8.22    26.30         0.6

TOTALS DATA

Description                      Low    Most Likely   High     Loading %

Total Loading (lb)              1339.4      2960.7      6692.1   100.0

Total Loading (kg)               607.5      1342.9      3035.5   100.0

Areal Loading (lb/ac-year)        6.44       14.23       32.17

Areal Loading (mg/m^2-year)     721.75     1595.43     3606.21

Total PS Loading (lb)              0.0         0.0         0.0     0.0

Total PS Loading (kg)              0.0         0.0         0.0     0.0

Total NPS Loading (lb)          1318.6      2886.9      6448.5    99.4

Total NPS Loading (kg)           598.1      1309.5      2925.0    99.4

Phosphorus Prediction and Uncertainty Analysis Module

Date: 7/26/2016    Scenario: 22

Observed spring overturn total phosphorus (SPO): 36.8 mg/m^3

Observed growing season mean phosphorus (GSM): 57.0 mg/m^3

Back calculation for SPO total phosphorus: 37.14 mg/m^3

Back calculation GSM phosphorus: 57.24 mg/m^3

% Confidence Range: 70%

Nurenberg Model Input - Est. Gross Int. Loading: 65 kg

Lake Phosphorus Model              Low   Most Likely   High     Predicted  % Dif.

Total P   Total P    Total P   -Observed

(mg/m^3) (mg/m^3)   (mg/m^3)   (mg/m^3)

Walker, 1987 Reservoir                         22       48        109         -9       -16

Canfield-Bachmann, 1981 Natural Lake           28       57        114          0         0

Canfield-Bachmann, 1981 Artificial Lake        26       48         88         -9       -16

Rechow, 1979 General                           20       45        102        -12       -21

Rechow, 1977 Anoxic                            32       70        159         13        23

Rechow, 1977 water load<50m/year               24       53        120         -4        -7

Rechow, 1977 water load>50m/year              N/A      N/A        N/A        N/A       N/A

Walker, 1977 General                           27       61        137         24        65

Vollenweider, 1982 Combined OECD               23       44         86         -3        -6

Dillon-Rigler-Kirchner                         19       42         96          5        14

Vollenweider, 1982 Shallow Lake/Res.           18       37         76        -10       -21

Larsen-Mercier, 1976                           27       59        133         22        60

Nurnberg, 1984 Oxic                            26       53        114         -4        -7

Lake Phosphorus Model          Confidence Confidence  Parameter    Back       Model

Lower      Upper      Fit?    Calculation   Type

Bound      Bound               (kg/year)

Walker, 1987 Reservoir                       27         89         FIT      1595       GSM

Canfield-Bachmann, 1981 Natural Lake         18        164         FIT      1318       GSM

Canfield-Bachmann, 1981 Artificial Lake      15        138         FIT      1641       GSM

Rechow, 1979 General                         24         84         FIT      1697       GSM

Rechow, 1977 Anoxic                          41        129         FIT      1094       GSM

Rechow, 1977 water load<50m/year             29         99         FIT      1448       GSM

Rechow, 1977 water load>50m/year            N/A        N/A         N/A       N/A       N/A

Walker, 1977 General                         29        120         FIT       823       SPO

Vollenweider, 1982 Combined OECD             21         83         FIT      1469       ANN

Dillon-Rigler-Kirchner                       24         78           P      1180       SPO

Vollenweider, 1982 Shallow Lake/Res.         18         71         FIT      1801       ANN

Larsen-Mercier, 1976                         35        107       P Pin       845       SPO

Nurnberg, 1984 Oxic                          28         99         FIT      1468       ANN