Date: 7/26/2016    Scenario: L-THIA direct 30% reduction

Lake Id: Big Blake Lake

Watershed Id: 1

Hydrologic and Morphometric Data

Tributary Drainage Area: 20066.1 acre

Total Unit Runoff: 8.00 in.

Annual Runoff Volume: 13377.4 acre-ft

Lake Surface Area <As>: 208.0 acre

Lake Volume <V>: 1872.0 acre-ft

Lake Mean Depth <z>: 9.0 ft

Precipitation - Evaporation: 3.3 in.

Hydraulic Loading: 13434.6 acre-ft/year

Areal Water Load <qs>: 64.6 ft/year

Lake Flushing Rate <p>: 7.18 1/year

Water Residence Time: 0.14 year

Observed spring overturn total phosphorus (SPO): 36.8 mg/m^3

Observed growing season mean phosphorus (GSM): 57.0 mg/m^3

% NPS Change: -30%

% PS Change: 0%

NON-POINT SOURCE DATA

Land Use        Acre        Low    Most Likely    High    Loading %   Low    Most Likely    High

(ac)     |---- Loading (kg/ha-year) ----|            |-----  Loading (kg/year) ----|

Row Crop AG          1023.8       0.50       1.00       3.00       24.6        145        290        870

Mixed AG                0.0       0.30       0.80       1.40        0.0          0          0          0

Pasture/Grass        4790.6       0.10       0.30       0.50       34.5        136        407        679

HD Urban (1/8 Ac)       4.3       1.00       1.50       2.00        0.2          1          2          2

MD Urban (1/4 Ac)       0.0       0.30       0.50       0.80        0.0          0          0          0

Rural Res (>1 Ac)     691.9       0.05       0.10       0.25        1.7         10         20         49

Wetlands             1496.3       0.10       0.10       0.10        3.6         42         42         42

Forest              10759.2       0.05       0.09       0.18       23.3        152        274        549

Lake Surface          208.0       0.10       0.30       1.00        2.1          8         25         84

POINT SOURCE DATA

Point Sources     Water Load     Low    Most Likely    High    Loading %

(m^3/year)  (kg/year)  (kg/year)   (kg/year)          _

SEPTIC TANK DATA

Description                                        Low    Most Likely   High     Loading %

Septic Tank Output (kg/capita-year)                0.30        0.50     0.80

# capita-years                        164.4

% Phosphorus Retained by Soil                      98.0        90.0     80.0

Septic Tank Loading (kg/year)                      0.99        8.22    26.30         0.7

TOTALS DATA

Description                      Low    Most Likely   High     Loading %

Total Loading (lb)              1174.5      2599.8      5886.0   100.0

Total Loading (kg)               532.8      1179.3      2669.9   100.0

Areal Loading (lb/ac-year)        5.65       12.50       28.30

Areal Loading (mg/m^2-year)     632.93     1400.97     3171.84

Total PS Loading (lb)              0.0         0.0         0.0     0.0

Total PS Loading (kg)              0.0         0.0         0.0     0.0

Total NPS Loading (lb)          1153.8      2526.0      5642.5    99.3

Total NPS Loading (kg)           523.4      1145.8      2559.4    99.3

Phosphorus Prediction and Uncertainty Analysis Module

Date: 7/26/2016    Scenario: 23

Observed spring overturn total phosphorus (SPO): 36.8 mg/m^3

Observed growing season mean phosphorus (GSM): 57.0 mg/m^3

Back calculation for SPO total phosphorus: 37.14 mg/m^3

Back calculation GSM phosphorus: 57.24 mg/m^3

% Confidence Range: 70%

Nurenberg Model Input - Est. Gross Int. Loading: 65 kg

Lake Phosphorus Model              Low   Most Likely   High     Predicted  % Dif.

Total P   Total P    Total P   -Observed

(mg/m^3) (mg/m^3)   (mg/m^3)   (mg/m^3)

Walker, 1987 Reservoir                         20       44         99        -13       -23

Canfield-Bachmann, 1981 Natural Lake           25       51        103         -6       -11

Canfield-Bachmann, 1981 Artificial Lake        23       44         80        -13       -23

Rechow, 1979 General                           18       40         90        -17       -30

Rechow, 1977 Anoxic                            28       62        140          5         9

Rechow, 1977 water load<50m/year               21       47        106        -10       -18

Rechow, 1977 water load>50m/year              N/A      N/A        N/A        N/A       N/A

Walker, 1977 General                           24       53        121         16        43

Vollenweider, 1982 Combined OECD               21       39         77         -8       -17

Dillon-Rigler-Kirchner                         17       37         84          0         0

Vollenweider, 1982 Shallow Lake/Res.           16       33         68        -14       -30

Larsen-Mercier, 1976                           23       52        117         15        41

Nurnberg, 1984 Oxic                            23       47        101        -10       -18

Lake Phosphorus Model          Confidence Confidence  Parameter    Back       Model

Lower      Upper      Fit?    Calculation   Type

Bound      Bound               (kg/year)

Walker, 1987 Reservoir                       25         81         FIT      1537       GSM

Canfield-Bachmann, 1981 Natural Lake         16        147         FIT      1318       GSM

Canfield-Bachmann, 1981 Artificial Lake      14        127         FIT      1641       GSM

Rechow, 1979 General                         22         75         FIT      1697       GSM

Rechow, 1977 Anoxic                          36        114         FIT      1094       GSM

Rechow, 1977 water load<50m/year             26         87         FIT      1448       GSM

Rechow, 1977 water load>50m/year            N/A        N/A         N/A       N/A       N/A

Walker, 1977 General                         26        105         FIT       823       SPO

Vollenweider, 1982 Combined OECD             19         74         FIT      1469       ANN

Dillon-Rigler-Kirchner                       21         68           P      1180       SPO

Vollenweider, 1982 Shallow Lake/Res.         16         63         FIT      1801       ANN

Larsen-Mercier, 1976                         30         94       P Pin       845       SPO

Nurnberg, 1984 Oxic                          24         88         FIT      1468       ANN