| 2008 Impaired Waters Documentation Sheet |  |  |  |  | Date Prepared 8/20/07_____________________________________________ |  |
| --- | --- | --- | --- | --- | --- | --- |
| Author Erin Hanson_________________________________________ |  |  |  |  |  |  |
| Waterbody Name Apple Creek – Segment 2________________________________ |  |  |  |  |  | Watershed_ LF02__________________ |
| WADRS ID | 10839______ |  | WBIC 124100____________ |  |  |  |
| Choose One: |  |  |  |  |  |  |
| _______ | Proposed 303d List Change (new water, delisting, new pollutants) |  |  |  |  |  |
| X______ | General 303d Documentation |  |  |  |  |  |
| TMDL/303D Listing Proposal |  |  |  |  |  |  |
| 3.99_____ | Start Mile | 23.88___ | End Mile | 19.89___ | Size (Miles/Acres) |  |
| Current Assessment: |  |  |  |  |  |  |
| _________ | Current Fish & Aquatic Life (Aquatic Community) Use __WWFF___________________________________________________________ |  |  |  |  |  |
| _________ | Attainable Fish & Aquatic Life (Aquatic Community) Use__ WWFF ________________________________________________________ |  |  |  |  |  |
| _________ | Designated Fish & Aquatic Life Use__ WWFF _________________________________________________________________________ |  |  |  |  |  |
| _________ | Supporting Attainable Use = fully, partially, not meeting __NOT SUPPORTING___(WATERS)____________________________________ |  |  |  |  |  |
| _________ | Does a Specific Fish Advisory Exist? (Yes/No) |  |  |  |  |  |
|  | ________ | If so, what is the advice? |  |  |  |  |
| Pollutants & Impairments |  |  |  |  |  |  |
| X________ | Impairments (Circle One or More) |  |  |  |  |  |
|  | Degraded habitat |  | Acute Toxicity |  | Chronic Toxicity | Temperature (Water) |
|  | Contaminated Fish Tissue |  |  | Eutrophication |  | Excessive Aquatic Plant Growth |
|  | Aquatic Invasives (AIS) Infestation |  |  |  | Dissolved Oxygen, Low |  |
|  | Other ____________________________________________________________________ |  |  |  |  |  |
| X________ | Pollutants (Circle One or More) |  |  |  |  |  |
|  | Total Phosphorus |  | Sediment | PCBs | PAHs | Creosote |
|  | NH3 (Ammonia) |  | Runoff Water (elevated temperatures) |  |  | Other _____________________________ |
|  | Other _atmospheric depositon - Hg_____________________________________________________________________ |  |  |  |  |  |
| ________ | Assessment Level (monitored site specific data less than 5 years old) versus evaluated (other) |  |  |  |  |  |
| ________ | DNR Category (3, 4d (recommended), 5a, 5b, 5c) |  |  |  |  |  |
| Monitoring and Listing |  |  |  |  |  |  |
| X________ | Water Quality Exceedances (As applicable) Dissolved Oxygen - Station 453079, 2.46 mg/l (8/9/1995), data in SWIMS UW-Milwaukee Sonde Data (http://www.uwgb.edu/watershed/data/UWM.htm) at USGS Station on Apple Creek - Daily Minimum DO < 5 mg/l 10/23/2003 - 10/26/2003 & 90 days in 2005 (June through November) |  |  |  |  |  |
| X________ | Assessment Parameters (M-IBI, FISH IBI, HBI) - Station 10007908: Warmwater IBI Integrity Rating “very poor” & small stream habitat rating “fair”, (10/8/2000) data in Fish habitat Database Macroinvertebrate HBI max 10, data in SWIMS - Station 453256 average 7.2005 (1980) indicates “fairly poor” water quality rating with “significant” degree of organic pollution - Station 10007908 average 6.0655 (2000 &2005) indicates “fair” water quality rating with “fairly significant” degree of organic pollution (Lillie et al 2003) |  |  |  |  |  |
| Stations/Dates/Metric/Scores ____________________________________________________________________________________ |  |  |  |  |  |  |
| Stations/Dates/Metric/Scores ____________________________________________________________________________________ |  |  |  |  |  |  |
| Data from SWIMS: See map named 124100.pdf for sample station locations. - Station 10007908 - APPLE CREEK - APPLE CREEK - WEYERS RD UPSTREAM: macroinvertebrate data from 2000 & 2005 - Station 453256 - APPLE CREEK - AT HWY 55: macroinvertebrate data from 1980 (2 dates) - Station 453079 - APPLE CREEK - AT CTH JJ: water quality data 8/9/1995 - Station 53562 - APPLE CREEK - AT CTH U: water quality data 1999 (9 dates) & 7/27/2006 Macroinvertebrate Data: Parameter; Station ID; Average; Min; Max; Count. HBI Max 10. 453256; 7.2005; 5.837; 8.564; 2. 10007908; 6.0655; 5.66; 6.471; 2. INDEX OF BIOTIC INTEGRITY (IBI). 453256; 3.39513; 2.27717; 4.51309; 2. 10007908; 2.920505; 2.60901; 3.232; 2. Selected Water Quality Parameters: Parameter; Station ID; Average; Min; Max; Count. DISSOLVED OXYGEN FIELD, MG/L; 453079; 2.46; 1. TEMPERATURE FIELD, DEG C; 53562; 20.95; 19; 22.9; 2. TEMPERATURE FIELD, DEG C; 453079; 22.8; 1. PHOSPHATE ORTHO DISS, MG/L; 453079; 0.199; 1. PHOSPHORUS TOTAL, MG/L; 53562; 0.3315; 0.153; 0.925; 9. PHOSPHORUS TOTAL, MG/L; 453079; 0.481; 1. RESIDUE TOTAL NFLT (TOTAL SUSPENDED SOLIDS), MG/L; 453079; 52; 52; 52; 1. Data from USGS: Station 04085046, Apple Creek at Sniderville WI: 1999-04-05 to 2006-10-20, n=273 Parameter, units; Average; Min; Max; Count. Phosphorus, water, unfiltered, milligrams per liter; 0.508; 0.07; 4.96; 269. Phosphorus, water, filtered, milligrams per liter; 0.210; 0.04; 1; 105. Orthophosphate, water, filtered, milligrams per liter; 0.283; 0.12; 0.445; 2. Orthophosphate, water, filtered, milligrams per liter as phosphorus; 0.092; 0.039; 0.145; 2. Suspended sediment concentration, milligrams per liter; 488; 8; 1960; 26. Suspended sediment discharge, tons per day; 749; 0.35; 4380; 26. University of Wisconsin, Green Bay – Lower Fox River Watershed Monitoring Program summary of data: - 2004 & 2005 annual summaries of discrete samples and daily USGS data (Mean daily discharge, Suspended Sediment Load, Total Phosphorous Load) available at: http://www.uwgb.edu/watershed/data/index.htm - Stream Monitoring Sondes: UW-Milwaukee also installed YSI monitoring sondes at each of the 4 USGS monitoring stations to continuously record stage height, turbidity, conductivity, temperature, pH, and dissolved oxygen. The following links are Microsoft Excel files containing the 2003 and 2005 daily summary data for each sonde, which have been post-calibrated: http://www.uwgb.edu/watershed/data/UWM.htm Data from other sources: Baumgart, P. January, 2005. Lower Green Bay and Lower Fox Tributary Modeling Report: Source allocation of suspended sediment and phosphorus loads to Green Bay from the Lower Fox River Subbasin Using the Soil and Water Assessment Tool. “Simulated Comparisons: The SWAT-modeled average annual suspended sediment load from Duck, Apple, Ashwaubenon and Dutchman Creeks combined, and delivered to Green Bay (LF05 and LF02), is 18,600 tons, or 0.27 t/ha at the watershed outlets (1989-2000; 1992 Baseline Scenario). This figure is much lower than the sediment load estimated by the Duck, Apple and Ashwaubenon Creeks Priority Watershed Project where the total load "delivered to streams" from all sources in these watersheds was 100,700 tons, or 1.46 t/ha per year (WDNR 1997; 110,016 English t/yr in Table 3-11).” |