| 2008 Impaired Waters Documentation Sheet |  |  |  |  | Date Prepared _10/2/2007_____________________________________________ |  |
| --- | --- | --- | --- | --- | --- | --- |
| Author _Erin Hanson___________________________________________________ |  |  |  |  |  |  |
| Waterbody Name _Garners Creek__________________________________________ |  |  |  |  |  | Watershed_LF03 – Plum & Kankapot Creeks___ |
| WADRS ID | 10845 _____________ |  | WBIC _127700__________________ |  |  | Lower Fox |
| Choose One: |  |  |  |  |  |  |
| _X_____ | Proposed 303d List Change (new water, delisting, new pollutants) |  |  |  |  |  |
| _______ | General 303d Documentation |  |  |  |  |  |
| TMDL/303D Listing Proposal |  |  |  |  |  |  |
| 0________ | Start Mile | 5________ | End Mile | _5_____ | Size (Miles/Acres) |  |
| Current Assessment: |  |  |  |  |  |  |
| _________ | Current Fish & Aquatic Life (Aquatic Community) Use ___________________________________________________________________ |  |  |  |  |  |
| _________ | Attainable Fish & Aquatic Life (Aquatic Community) Use_WWSF___________________________________________________________ |  |  |  |  |  |
| _________ | Designated Fish & Aquatic Life Use__________________________________________________________________________________ |  |  |  |  |  |
| _________ | Supporting Attainable Use = fully, partially, not meeting _NOT ASSESSED___________________________________________________ |  |  |  |  |  |
| _________ | Does a Specific Fish Advisory Exist? (Yes/No) |  |  |  |  |  |
|  | ________ | If so, what is the advice? |  |  |  |  |
| Pollutants & Impairments |  |  |  |  |  |  |
| X________ | Impairments (Circle One or More) |  |  |  |  |  |
|  | Degraded habitat - NEW |  | Acute Toxicity |  | Chronic Toxicity | Temperature (Water) |
|  | Contaminated Fish Tissue |  |  | Eutrophication |  | Excessive Aquatic Plant Growth |
|  | Aquatic Invasives (AIS) Infestation |  |  |  | Dissolved Oxygen, Low |  |
|  | Other ______________________________________________________________________ |  |  |  |  |  |
| _X_______ | Pollutants (Circle One or More) |  |  |  |  |  |
|  | Total Phosphorus - NEW |  | Sediment - NEW | PCBs | PAHs | Creosote |
|  | NH3 (Ammonia) |  | Runoff Water (elevated temperatures) |  |  | Other _____________________________ |
|  | Other ______________________________________________________________________ |  |  |  |  |  |
| ________ | Assessment Level (monitored site specific data less than 5 years old) versus evaluated (other) |  |  |  |  |  |
| ________ | DNR Category (3, 4d (recommended), 5a, 5b, 5c) |  |  |  |  |  |
| Monitoring and Listing |  |  |  |  |  |  |
| ________ | Water Quality Exceedances (As applicable) - Dissolved Oxygen below 5 mg/l “several times” fall 1998, see memo below by Scott Szymanski Dec. 1998 (WDNR) |  |  |  |  |  |
| _X_______ | Assessment Parameters (M-IBI, FISH IBI, HBI) HBI max 10, data in SWIMS: - Station 10015871 average = 5.65 (1997) indicating “fair” water quality rating with “fairly significant” degree of organic pollution - Station 10016542 average = 7.05 (1992) indicating “fairly poor” water quality rating with “significant” degree of organic pollution |  |  |  |  |  |
| Stations/Dates/Metric/Scores ____________________________________________________________________________________ |  |  |  |  |  |  |
| Stations/Dates/Metric/Scores ____________________________________________________________________________________ |  |  |  |  |  |  |
| Attach any additional reports, analyses, etc. (Use Designation Survey, etc.) |  |  |  |  |  |  |
| Data in SWIMS: See map named 0410000.pdf for sample station locations. - Station 10016542 - GARNERS CREEK - BROOKHAVEN DR. - 500 FEET DOWN-STREAM JUST EAST OF CTH N AND NORTH OFCTH CE - Station 10015871 - GARNERS CREEK - HARTJES RD. - Station 453277 - GARNERS CREEK - @ PARK, 13 dates from 2004-2006 Macroinvertebrate Data: Parameter; Station ID; Average; Min; Max; Count. HBI Max 10. 10015871; 5.652; 5.512; 5.792; 2. 10016542; 7.0515; 6.532; 7.571; 2. INDEX OF BIOTIC INTEGRITY (IBI). 10015871; 3.376085; 3.09772; 3.65445; 2. 10016542; 3.85414; 2.62753; 5.08075; 2. Selected water quality parameters for Station 453277: Parameter; Average; Min; Max; Count. ALKALINITY TOTAL CACO3, MG/L; 224.3; 0; 270; 15. CONDUCTIVITY AT 25C, UMHOS/CM; 1259.4; 0; 2140; 15. TEMPERATURE FIELD, deg C; 22.9; 1. CHLORIDE, MG/L; 207.1; 0; 516; 15. NITROGEN NH3-N DISS, MG/L; 0.041; 1. PH LAB, SU; 8.0; 0; 8.37; 15. Data from Jim Reyburn, Wisconsin DNR Green Bay, Personal Files: 1992-1999 Flow; Tot-P; Tot-P; Tot-P; Susp. Solids; Susp.Solids; Susp. Solids; Ammonia. cfs; mg/l; lb/day; lb/sq mi; mg/l; lb/day; lb/sq mi; mg/l. Average for all; 8.69; 0.256; 17; 1.49; 109; 10268; 909; 0.110. n=23. Average low flow; 0.76; 0.109; 0.4; 0.04; 22; 126; 11; 0.086. n=11. Average event; 17.42; 0.391; 35; 3.10; 188; 21424; 1896; 0.118. n=12. Data from USGS: ### USGS 04084468: GARNERS CREEK AT PARK STREET AT KAUKAUNA, WI, 2004-02-10 through 2006-07-27, N=25 Parameter, units; Average; Min; Max; Count. Temperature, water, degrees Celsius; 17.025; 14.7; 21.7; 4. Dissolved oxygen, water, unfiltered, milligrams per liter; 8.6; 7.8; 10.1; 3. Total nitrogen (nitrate + nitrite + ammonia + organic-N), water, unfiltered, analytically determined, mg/l; 1.49; 0.76; 2.22; 2. Ammonia, water, filtered, milligrams per liter as nitrogen; 0.040; 0.04; 0.041; 3. Nitrite plus nitrate, water, filtered, milligrams per liter as nitrogen; 0.94; 0.34; 1.54; 2. Orthophosphate, water, filtered, milligrams per liter; 0.089; 0.034; 0.144; 2. Phosphorus, water, unfiltered, milligrams per liter; 0.0945; 0.082; 0.107; 2. Suspended sediment concentration, milligrams per liter; 143.5; 80; 207; 2. Suspended sediment discharge, tons per day; 1.22; 0.24; 2.2; 2. Other Data: - memo by Scott Szymanski Dec. 1998 (WDNR) in NER Central Files, Green Bay |