| 2008 Impaired Waters Documentation Sheet |  |  |  |  | Date Prepared 8/20/2007______________________________________________ |  |
| --- | --- | --- | --- | --- | --- | --- |
| Author Erin Hanson____________________________________________________ |  |  |  |  |  |  |
| Waterbody Name Kankapot Creek – Segment 2 |  |  |  |  |  | Watershed LF03 - Plum and Kankapot Creeks |
| WADRS ID | 357763______ |  | WBIC _126800___________________ |  |  | Lower Fox |
| Choose One: |  |  |  |  |  |  |
| X_______ | Proposed 303d List Change (new water, delisting, new pollutants) Total Phosphorus – NEW |  |  |  |  |  |
| _______ | General 303d Documentation |  |  |  |  |  |
| TMDL/303D Listing Proposal |  |  |  |  |  |  |
| 2.66_____ | Start Mile | 9.57_____ | End Mile | _6.91___ | Size (Miles/Acres) |  |
| Current Assessment: |  |  |  |  |  |  |
| _________ | Current Fish & Aquatic Life (Aquatic Community) Use __unknown_________________________________________________________ |  |  |  |  |  |
| _________ | Attainable Fish & Aquatic Life (Aquatic Community) Use_unknown_________________________________________________________ |  |  |  |  |  |
| _________ | Designated Fish & Aquatic Life Use_LFF (TFAL)_______________________________________________________________________ |  |  |  |  |  |
| _________ | Supporting Attainable Use = fully, partially, not meeting _not supporting_________________________________________________ |  |  |  |  |  |
| _________ | Does a Specific Fish Advisory Exist? (Yes/No) |  |  |  |  |  |
|  | ________ | If so, what is the advice? |  |  |  |  |
| Pollutants & Impairments |  |  |  |  |  |  |
| X_______ | Impairments (Circle One or More) |  |  |  |  |  |
|  | Degraded habitat |  | Acute Toxicity |  | Chronic Toxicity | Temperature (Water) |
|  | Contaminated Fish Tissue |  |  | Eutrophication |  | Excessive Aquatic Plant Growth |
|  | Aquatic Invasives (AIS) Infestation |  |  |  | Dissolved Oxygen, Low |  |
|  | Other ______________________________________________________________________ |  |  |  |  |  |
| X_______ | Pollutants (Circle One or More) |  |  |  |  |  |
|  | Total Phosphorus - NEW |  | Sediment | PCBs | PAHs | Creosote |
|  | NH3 (Ammonia) |  | Runoff Water (elevated temperatures) |  |  | Other _____________________________ |
|  | Other ______________________________________________________________________ |  |  |  |  |  |
| ________ | Assessment Level (monitored site specific data less than 5 years old) versus evaluated (other) |  |  |  |  |  |
| ________ | DNR Category (3, 4d (recommended), 5a, 5b, 5c) |  |  |  |  |  |
| Monitoring and Listing |  |  |  |  |  |  |
| X________ | Water Quality Exceedances (As applicable) Dissolved Oxygen = 1.2 mg/l at USGS Station 04084479 on 7/17/2001 |  |  |  |  |  |
| X________ | Assessment Parameters (M-IBI, FISH IBI, HBI) - No data for Fish Warmwater IBI Integrity Rating or Stream Habitat Rating in WDNR fisheries database - Macroinvertebrate HBI max 10 7.56 & 8.92 (2001 & 1992) indicating “poor” to “very poor” water quality rating, “very significant” to “severe” degree of organic pollution (Lillie et al 2003) |  |  |  |  |  |
| Stations/Dates/Metric/Scores ____________________________________________________________________________________ |  |  |  |  |  |  |
| Stations/Dates/Metric/Scores ____________________________________________________________________________________ |  |  |  |  |  |  |
| Attach any additional reports, analyses, etc. (Use Designation Survey, etc.) |  |  |  |  |  |  |
| Data from SWIMS: - Station 10016668, KANKAPOT CREEK - UPSTREAM OF CTH KK BRIDGE – macroinvertebrate data 7/4/1992 - Station 453276, KANKAPOT CREEK - NEAR KAUKAUNA, WI – macroinvertebrate data 10/16/2001 & water quality data 6/19/2001, 7/17/2001 & 8/21/2001 Macroinvertebrate data: Parameter; Station ID; Result. HBI Max 10. 10016668; 7.56. 453276; 8.92. INDEX OF BIOTIC INTEGRITY (IBI). 10016668; 1.494. 453276; 4.367. Water Quality Data: Parameter; Station ID; Average; Min; Max; Count. NITROGEN KJELDAHL TOTAL. 453276; 2.297; 1.09; 4.05; 3. NITROGEN NH3-N DISS. 453276; 0.555; 0.024; 1.5; 3. NITROGEN NO3+NO2 DISS (AS N). 453276; 5.228; 0.163; 14.4; 3. PHOSPHORUS TOTAL. 453276; 0.597; 0.306; 0.95; 3. PHOSPHORUS TOTAL DISS. 453276; 0.431; 0.26; 0.549; 3. TURBIDITY, LAB NEPHELOMETRIC NTU. 453276; 12.3; 1. Data from USGS: USGS Station 04084479 KANKAPOT CREEK NEAR KAUKAUNA, WI – sample dates 5/22/2001, 6/19/2001, 7/17/2001 & 8/21/2001 overlap data in SWIMS database Parameter, units; Average; Min; Max; Count. Temperature, water, degrees Celsius; 17.525; 13.5; 19.9; 4. Dissolved oxygen, water, unfiltered, mg/l; 6.525; 1.2; 10.2; 4. Ammonia plus organic nitrogen, water, unfiltered, mg/l as nitrogen; 2.275; 1.1; 4; 4. Ammonia, water, filtered, mg/l as NH4; 0.5675; 0.03; 1.93; 4. Ammonia, water, filtered, mg/l as nitrogen; 0.44175; 0.024; 1.5; 4. Total nitrogen, water, unfiltered, mg/l; 9.35; 2.2; 16; 4. Nitrite plus nitrate, water, filtered, mg/l as nitrogen; 7.02075; 0.163; 14.4; 4. Organic nitrogen, water, unfiltered, mg/l; 1.825; 1.1; 2.5; 4. Phosphorus, water, filtered, mg/l; 0.43875; 0.26; 0.549; 4. Phosphorus, water, unfiltered, mg/l; 0.5925; 0.306; 0.95; 4. Turbidity, water, unfiltered, nephelometric turbidity units; 12; 12; 12; 1. Data from other sources: Baumgart, P. January, 2005. Lower Green Bay and Lower Fox Tributary Modeling Report: Source allocation of suspended sediment and phosphorus loads to Green Bay from the Lower Fox River Subbasin Using the Soil and Water Assessment Tool. - “Mean and median concentrations of TSS from samples collected by the WDNR (Northeast Region, unpublished watershed monitoring data 1997 to 2002) from LF03 watersheds such as Plum, Kankapot and Garners Creeks were all at least twice as high as those from Mud Creek, which is a subwatershed of LF04 (Mud Cr. TSS: mean = 31 mg/L, median = 18 mg/L, n=23). In addition, phosphorus concentrations from LF03 watersheds Plum and Kankapot Creeks were all at least four times higher than those from Mud Creek (Mud Cr. phosphorus: mean = 0.17 mg/L, median = 0.11mg/L, n=23).” Pg. 10-1 |