| 2008 Impaired Waters Documentation Sheet |  |  |  |  | Date Prepared 8/20/2007______________________________________________ |  |
| --- | --- | --- | --- | --- | --- | --- |
| Author Erin Hanson____________________________________________________ |  |  |  |  |  |  |
| Waterbody Name Kankapot Creek – Segment 1 |  |  |  |  |  | Watershed _LF03 - Plum and Kankapot Creeks |
| WADRS ID | 10844______ |  | WBIC _126800__________________ |  |  |  |
| Choose One: |  |  |  |  |  |  |
| X_______ | Proposed 303d List Change (new water, delisting, new pollutants) Total Phosphorus – NEW |  |  |  |  |  |
| _______ | General 303d Documentation |  |  |  |  |  |
| TMDL/303D Listing Proposal |  |  |  |  |  |  |
| 0________ | Start Mile | 2.66_____ | End Mile | 2.66____ | Size (Miles/Acres) |  |
| Current Assessment: |  |  |  |  |  |  |
| _________ | Current Fish & Aquatic Life (Aquatic Community) Use unknown___________________________________________________________ |  |  |  |  |  |
| _________ | Attainable Fish & Aquatic Life (Aquatic Community) Use unknown_________________________________________________________ |  |  |  |  |  |
| _________ | Designated Fish & Aquatic Life Use_LFF (TFAL)________________________________________________________________________ |  |  |  |  |  |
| _________ | Supporting Attainable Use = fully, partially, not meeting _not supporting___________________________________________________ |  |  |  |  |  |
| _________ | Does a Specific Fish Advisory Exist? (Yes/No) |  |  |  |  |  |
|  | ________ | If so, what is the advice |  |  |  |  |
| Pollutants & Impairments |  |  |  |  |  |  |
| X________ | Impairments (Circle One or More) |  |  |  |  |  |
|  | Degraded habitat |  | Acute Toxicity |  | Chronic Toxicity | Temperature (Water) |
|  | Contaminated Fish Tissue |  |  | Eutrophication |  | Excessive Aquatic Plant Growth |
|  | Aquatic Invasives (AIS) Infestation |  |  |  | Dissolved Oxygen, Low |  |
|  | Other ______________________________________________________________________ |  |  |  |  |  |
| X________ | Pollutants (Circle One or More) |  |  |  |  |  |
|  | Total Phosphorus - NEW |  | Sediment | PCBs | PAHs | Creosote |
|  | NH3 (Ammonia) |  | Runoff Water (elevated temperatures) |  |  | Other _____________________________ |
|  | Other ______________________________________________________________________ |  |  |  |  |  |
| ________ | Assessment Level (monitored site specific data less than 5 years old) versus evaluated (other) |  |  |  |  |  |
| ________ | DNR Category (3, 4d (recommended), 5a, 5b, 5c) |  |  |  |  |  |
| Monitoring and Listing |  |  |  |  |  |  |
| X________ | Water Quality Exceedances (As applicable) Dissolved Oxygen ≤ 5 mg/L - Station 453245: 2.41 mg/l (10/21/1999) & 4.7 mg/l (6/19/1997) - Station 453261: 4.2 mg/l (8/1/2005) |  |  |  |  |  |
| ________ | Assessment Parameters (M-IBI, FISH IBI, HBI) - Macroinvertebrate HBI max 10 = 6.136 – 7.818 (n=4) at Station 453245 (1992 & 1997) indicating “Fair” to “Poor” water quality rating with “fairly significant” to “very significant” degree of organic pollution (Lillie et al 2003), data in SWIMS - No data for Fish Warmwater IBI Integrity Rating or Stream Habitat Rating in WDNR fisheries database |  |  |  |  |  |
| Stations/Dates/Metric/Scores ____________________________________________________________________________________ |  |  |  |  |  |  |
| Stations/Dates/Metric/Scores ____________________________________________________________________________________ |  |  |  |  |  |  |
| Attach any additional reports, analyses, etc. (Use Designation Survey, etc.) |  |  |  |  |  |  |
| Data From SWIMS: - Station 453261, KANKAPOT CREEK - CTH Z DODGE STREET, 100 FT UPSTREAM OF BRIDGE – no macroinvertebrate data, water quality samples 2005 (2 dates) & 2006 (15 dates) - Station 453245, KANKAPOT CREEK - CTH CE – macroinvertebrate data 4/7/92, 10/13/92, 5/22/97 & 10/20/97 - Water quality samples 1992 (2 dates), 1997 (monthly), 1998 (3 dates) & 5/22/2001 Macroinvertebrate data, Station 453245 Parameter; Average; Min; Max; Count. HBI Max 10; 6.93475; 6.136; 7.818; 4. INDEX OF BIOTIC INTEGRITY (IBI); 2.6808725; 2.01525; 3.4788; 4. Water Quality Data, selected parameters: Parameter; Station ID; Average; Min; Max; Count. TEMPERATURE FIELD. 453245; 11.0; 0.07; 25.6; 23. 453261; 12.6; 0.1; 24.5; 15. DISSOLVED OXYGEN FIELD. 453245; 9.7; 2.41; 17.1; 23. 453261; 10.5; 4.2; 14.9; 15. FECAL COLIFORM, MEMBRANE FILTER, MFC AGAR. 453245; 7149; 10; 67000; 18. NITROGEN KJELDAHL TOTAL. 453245; 3.22; 1.7; 5.9; 6. 453261; 1.31; 0.79; 2.29; 15. NITROGEN NH3-N DISS. 453245; 0.43; 0; 2.58; 21. 453261; 0.22; 0.019; 0.972; 15. NITROGEN NO3+NO2 DISS (AS N). 453245; 4.86; 0.09; 12.4; 6. 453261; 1.33; 0.041; 5.43; 15. PHOSPHATE ORTHO DISS. 453245; 0.47; 0; 1.8; 25. PHOSPHORUS TOTAL. 453245; 0.74; 0; 2.31; 28. 453261; 0.16; 0.025; 0.372; 15. PHOSPHORUS TOTAL DISS. 453245; 0.46; 1. RESIDUE TOTAL NFLT (TOTAL SUSPENDED SOLIDS). 453245; 61; 0; 445; 25. 453261; 13; 4; 21; 15. Data Summary provided by James Reyburn, WDNR Water Quality Biologist – Lower Fox Basin: WDNR data 1992-1999 Flow; Tot-P; Tot-P; Tot-P; Susp. Solids; Susp.Solids; Susp. Solids; Ammonia. cfs; mg/l; lb/day; lb/sq mi; mg/l; lb/day; lb/sq mi; mg/l. Average for all; 3.71; 0.866; 10; 0.40; 81; 1105; 44; 0.496. n=22. Average low flow; 0.67; 0.83; 1.4; 0.06; 27; 65; 3; 0.273. n=11. Average event; 7.44; 0.97; 20; 0.81; 148; 2376; 95; 0.346. n=11. Data from USGS: none Data from other sources: Baumgart, P. January, 2005. Lower Green Bay and Lower Fox Tributary Modeling Report: Source allocation of suspended sediment and phosphorus loads to Green Bay from the Lower Fox River Subbasin Using the Soil and Water Assessment Tool. - “Mean and median concentrations of TSS from samples collected by the WDNR (Northeast Region, unpublished watershed monitoring data 1997 to 2002) from LF03 watersheds such as Plum, Kankapot and Garners Creeks were all at least twice as high as those from Mud Creek, which is a subwatershed of LF04 (Mud Cr. TSS: mean = 31 mg/L, median = 18 mg/L, n=23). In addition, phosphorus concentrations from LF03 watersheds Plum and Kankapot Creeks were all at least four times higher than those from Mud Creek (Mud Cr. phosphorus: mean = 0.17 mg/L, median = 0.11mg/L, n=23).” Pg. 10-1 |