| 2010 Impaired Waters Documentation Sheet |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Author: Pamela Toshner/Alex Smith/Kristi Minahan |  |  |  |  |  |  |  | Date Prepared: 23 Sept 2009 |  |  |  |  |
| Waterbody Name: Big Butternut |  |  |  |  |  |  |  | Watershed Code and Name: SC10 Trade River |  |  |  |  |
| WADRS ID: 16680 |  | WBIC: 2641000 |  |  |  | If you don’t know the ID numbers, go to i-SWDV (CRTL + Click) to find them. |  |  |  |  |  |  |
| Choose from the following to indicate what you are recommending: |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed new listing |  |  |  |  |  |  |  |  |  |  |  |  |
| _X____ Reviewed but not recommended for listing |  |  |  |  |  |  |  |  |  |  |  |  |
| _____ Proposed changes for water already on 303(d) list (check type of change below)  TMDL ID Number: _______ |  |  |  |  |  |  |  |  |  |  |  |  |
|  | _____ Proposed change to existing list (new pollutants, impairments, mileages, etc.) |  |  |  |  |  |  |  |  |  |  |  |
|  | _____ Proposed for de-listing ( a report may be needed depending on the reasons) |  |  |  |  |  |  |  |  |  |  |  |
|  | _____ General 303d documentation for water already on list |  |  |  |  |  |  |  |  |  |  |  |
| Description of waterbody segment |  |  |  |  |  |  |  |  |  |  |  |  |
| Start Mile: ____________ End Mile: ____________ Total miles: ____________ Lake Acres: ___378_________ |  |  |  | Detail (describe segment using road crossings, convergence with other waterbodies, etc.): |  |  |  |  |  |  |  |  |
| Use Designation Categories |  |  |  |  |  |  |  |  | List use designation & data source for each category. |  |  |  |
| Current (Existing) Fish & Aquatic Life Use: (Aquatic Community) |  |  |  |  |  |  |  |  | Shallow Lowland |  |  |  |
| Attainable (Potential) Fish & Aquatic Life Use: (Aquatic Community) |  |  |  |  |  |  |  |  | Shallow Lowland |  |  |  |
| Designated (Codified) Fish & Aquatic Life Use: |  |  |  |  |  |  |  |  | Default |  |  |  |
| Is it supporting its Attainable Use? _____ Fully supporting _____ Supporting __X___ Not supporting Does a Specific Fish Advisory Exist? (This is different from the statewide mercury advisory.) ___ Yes __X__ No ___ Don’t know If so, what is the specific advisory: |  |  |  |  |  |  |  |  |  |  |  |  |
| Pollutants & Impairments Pollutants (Place an X next to all pollutants that you are recommending for listing, de-listing or monitoring needs. If you are recommending adding a new pollutant to a waterbody that is already on the list, write ADD.) |  |  |  |  |  |  |  |  |  |  |  |  |
| _____ Phosphorus |  |  | _____ Sediment |  |  |  | _____ Bacteria |  |  | _____ PAHs |  | _____ PCBs |
| _____ NH3 (Ammonia) |  |  | _____ Thermal (elevated temp.) |  |  |  | _____ Hg |  |  | _____ Creosote |  | _____ Metals |
| Other Pollutants: |  |  |  |  |  |  |  |  |  |  |  |  |
| Impairments (Place an X next to all impairments that you are recommending for listing, delisting, or monitoring needs. If you are recommending adding a new pollutant to a waterbody that is already on the list, write ADD. ) |  |  |  |  |  |  |  |  |  |  |  |  |
| _____ Degraded Habitat |  |  |  |  | _____ Eutrophication |  |  |  |  |  | _____ Temperature |  |
| _____ Contaminated Fish Tissue |  |  |  |  | _____ Chronic Toxicity |  |  |  |  |  | _____ Aquatic Toxicity |  |
| _____ Low Dissolved Oxygen |  |  |  |  | ____ Aquatic Invasives |  |  |  |  |  |  |  |
| Other Impairments: |  |  |  |  |  |  |  |  |  |  |  |  |
| Source of impairment (Place an X next to the source category.) if you need an explanation of source categories., see http://dnr.wi.gov/org/water/wm/wqs/303d/categories.html . |  |  |  |  |  |  |  |  |  |  |  |  |
| _____ NPS Dominated |  |  |  |  | ____ NPS/Pt. Source Blend |  |  |  |  |  | _____ Point Source |  |
| _____ Contaminated Sediment |  |  |  |  | _____ Atmospheric Deposition |  |  |  |  |  | _____ Physical Habitat |  |
| _____ Unknown |  |  |  |  | Other: _____________________________ |  |  |  |  |  |  |  |
| Specific causes of impairment (Describe to the best of your ability what you think is contributing to the impairment.) |  |  |  |  |  |  |  |  |  |  |  |  |
| Information is based on: Monitoring data (specific data) less than 5 years old (Circle One) YES NO |  |  |  |  |  |  |  |  |  |  |  |  |
| Map When possible, please create a map of your waterbody and submit it with this form. It is easy to create a 8.5 x 11 map of the area you are describing. Go to the go to the Intranet Surface Water Viewer i-SWDV (CRTL + Click). Choose “Find Location” to find the waterbody, then “Layers” to choose “Standards, Monitoring and Assessment”. If it is already on the 303(d) list, then click “Impaired Waters 303d". If you want to show the monitoring stations, also click on "SWIMS Station Points". Then choose “Print” (this will create a pdf map), add a title under "Map Title" and your name and date under “Map Notes", click "OK" and then "Open Map" at the next screen. Save the file and attach it when you send in this sheet. For additional help on how to make a map, check out page 12-14 on the website http://www.dnr.state.wi.us/org/water/SWDV/help/documents/SWDV_Basic_User_Guide_%209_07.pdf |  |  |  |  |  |  |  |  |  |  |  |  |
| Monitoring & Listing Data 1. Monitoring Study, Date, Results. List water quality exceedences indicating magnitude, duration and frequency. Stations: 493106 Parameters: - Average Total Phosphorus is .078 mg/l (2004-2008), which exceeds the threshold of .04 mg/l for shallow lowland lakes or .03 mg/l for deep headwater lakes (verification is needed on which natural community this lake should be in). The 12-year (1996-2008) median May-Oct phosphorous is 0.053 mg/L. - Using the Carlson TSI calculation for 2004-2008, average TSI-chlorophyll is 71, which puts it in the Poor category for shallow lowland lakes (or for deep headwater lakes). Using the Wisconsin TSI calculation method, the 12-year (1996-2008) average summer (07/15-09/15) chlorophyll a TSI is 63. Database where data is stored (Fish Database, SWIMS, FishSED, Personal PC): SWIMS |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Monitoring Study, Date, Results. List water quality exceedences indicating magnitude, duration and frequency. Stations: Parameters: Database where data is stored (Fish Database, SWIMS, FishSED, Personal PC): |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Monitoring Study, Date, Results. List water quality exceedences indicating magnitude, duration and frequency. Stations: Parameters: Database where data is stored (Fish Database, SWIMS, FishSED, Personal PC): |  |  |  |  |  |  |  |  |  |  |  |  |
| Narrative on why you are proposing this waterbody to be listed or delisted? Not recommended for 2010 listing; this lake system is thought to be naturally eutrophic and studies are currently underway on the lake system to investigate sources. Big Butternut and Long Trade Lakes in Polk County and Round Lake in Burnett County are all part of the Trade River watershed, which has been known historically to be highly eutrophic. While the TP and chl a TSI values for the three lakes exceed the thresholds for 303(d) listing, this area of Polk and Burnett Counties is known to contain high-phosphorous glacial outwash till that causes naturally eutrophic lake conditions. Local lore has it that Big Butternut Lake was called Big, Stinky, Green Waters in Objibwe prior to European settlement. There are currently open grant projects investigating the watershed and in-lake sources of Big Butternut Lake nutrients and monitoring the aquatic plant and Eurasian watermilfoil communities. Therefore, I support flagging the lakes for further monitoring until the additional information can be analyzed and considered for listing. - Average Total Phosphorus is .078 mg/l (2004-2008), which exceeds the threshold of .04 mg/l for shallow lowland lakes or .03 mg/l for deep headwater lakes (verification is needed on which natural community this lake should be in). The 12-year (1996-2008) median May-Oct phosphorous is 0.053 mg/L. - Using the Carlson TSI calculation for 2004-2008, average TSI-chlorophyll is 71, which puts it in the Poor category for shallow lowland lakes (or for deep headwater lakes). Using the Wisconsin TSI calculation method, the 12-year (1996-2008) average summer (07/15-09/15) chlorophyll a TSI is 63. |  |  |  |  |  |  |  |  |  |  |  |  |
| List and Attach any additional reports, updated watershed tables, analyses etc. including use designation survey. 1. USGS. December 16, 2008. Round and Long Trade Lakes Water Quality and Watershed Assessment Report. 2. 3. 4. 5. |  |  |  |  |  |  |  |  |  |  |  |  |

| Lake Name | County Name | Station ID |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Big Butternut Lake | Polk | 493106 |  |  |  |  |  |  |
| Group Seq No | Start Date | Total Phosphorus(ug/l) | Perception |  |  |  |  |  |
| 8190296 | 05/10/2004 | 28 | 1-Beautiful |  |  |  |  |  |
| 8190296 | 05/29/2004 |  | 1-Beautiful |  |  |  |  |  |
| 8190296 | 06/03/2004 |  | 1-Beautiful |  |  |  |  |  |
| 8190296 | 06/14/2004 |  | 1-Beautiful |  |  |  |  |  |
| 8190296 | 06/22/2004 | 23 | 1-Beautiful |  |  |  |  |  |
| 7000033 | 07/09/2004 |  |  |  |  |  |  |  |
| 8190296 | 07/19/2004 |  | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| 7000033 | 07/27/2004 |  |  |  |  |  |  |  |
| 8190296 | 07/27/2004 | 43 | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| 8190296 | 08/09/2004 |  | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| 8190296 | 08/21/2004 |  | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| 8190296 | 08/25/2004 | 126 | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| 8190296 | 09/12/2004 |  | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| 7000033 | 09/28/2004 |  |  |  |  |  |  |  |
| 8190296 | 10/11/2004 | 56 | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| Annual Average |  | 55.2 |  |  |  |  |  |  |
| 8191350 | 05/03/2005 | 33 |  |  |  |  |  |  |
| 8191350 | 06/20/2005 | 12 |  |  |  |  |  |  |
| 8191350 | 07/10/2005 |  |  |  |  |  |  |  |
| 8190296 | 07/27/2005 | 81 |  |  |  |  |  |  |
| 8191350 | 08/22/2005 | 179 | 5-Enjoyment substantially impaired (algae) |  |  |  |  |  |
| 8191350 | 09/20/2005 |  | 2-Very minor aesthetic problems |  |  |  |  |  |
| Annual Average |  | 76.25 |  |  |  |  |  |  |
| 8191350 | 05/15/2006 |  | 2-Very minor aesthetic problems |  |  |  |  |  |
| 8191350 | 06/05/2006 | 37 | 4-Would not swim but boating OK (algae) |  |  |  |  |  |
| 8191350 | 06/12/2006 | 194 | 2-Very minor aesthetic problems |  |  |  |  |  |
| 8191350 | 07/05/2006 |  | 2-Very minor aesthetic problems |  |  |  |  |  |
| 8191350 | 08/07/2006 | 98 | 2-Very minor aesthetic problems |  |  |  |  |  |
| 8191350 | 09/05/2006 | 152 | 2-Very minor aesthetic problems |  |  |  |  |  |
| Annual Average |  | 120.25 |  |  |  |  |  |  |
| 8191350 | 05/24/2007 |  | 4-Would not swim but boating OK (algae) |  |  |  |  |  |
| 8191350 | 05/30/2007 | 42 |  |  |  |  |  |  |
| 8191350 | 06/26/2007 | 63 | 4-Would not swim but boating OK (algae) |  |  |  |  |  |
| 8191350 | 07/24/2007 | 80 | 4-Would not swim but boating OK (algae) |  |  |  |  |  |
| 8191350 | 08/06/2007 | 69 | 4-Would not swim but boating OK (algae) |  |  |  |  |  |
| 8191350 | 09/12/2007 | 80 | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| Annual Average |  | 66.8 |  |  |  |  |  |  |
| 8191350 | 06/03/2008 | 34 |  |  |  |  |  |  |
| 8191350 | 07/15/2008 | 62 |  |  |  |  |  |  |
| 8191350 | 08/19/2008 | 94 |  |  |  |  |  |  |
| 8191350 | 09/08/2008 | 107 |  |  |  |  |  |  |
| Annual Average |  | 74.25 |  |  |  |  |  |  |
| 29302935 | 05/21/2009 |  | 1-Beautiful |  |  |  |  |  |
| 29302935 | 06/13/2009 |  | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| 29302935 | 06/21/2009 | 39 |  |  |  |  |  |  |
| 29302935 | 07/13/2009 | 40 | 4-Would not swim but boating OK (algae) |  |  |  |  |  |
| 29302935 | 08/15/2009 |  | 3-Enjoyment somewhat impaired (algae) |  |  |  |  |  |
| 1 | 08/16/2009 | 75 |  |  |  |  |  |  |
| (2009 values not used in calc.) |  |  |  |  |  | 78.55 |  | ug/l |
|  |  | 0.07855 | mg/l |  |  |  |  |  |
| Shallow Lowland lakes' impairment threshold is .04 mg/l. |  |  |  |  |  |  |  |  |
| Big Butternut Lake exceeds the impairment threshold. |  |  |  |  |  |  |  |  |