303(d) Listing Documentation

Waterbody Name: Swinns Valley Creek

Waterbody ID Code (WBIC): 1776000

Watershed: Middle Trempealeau (BT02)

County(ies): Buffalo

Impaired Stream Reach (location and mileage): Entire stream from its confluence with the Trempealeau River, upstream to STH 95 ending near Glencoe Valley Rd. (5.2 miles)

Type of impairment (water quality standards violations [ex. pH, DO, etc.], not meeting codified use [ex. WWFF should be Cold II], eutrophication, etc.):

Codified use = FFAL	Existing use = FFAL	Potential use = COLD II

Cause of impairment (ex. temperature, nutrients, sediment, BOD, toxics, etc.):

Sedimentation, Habitat

Source of impairment (ex. point source, nonpoint source, internal load, unknown, etc.):

NPS (cropland erosion, streambank erosion)

Description of monitoring conducted (including dates, results, quality of data and reports):

1996 Buffalo-Trempealeau Basin Report

Middle Trempealeau River Priority Watershed Water Resource Appraisal Final Report (Schreiber 1991). The stream was determined to have an HBI of 2.72, indicating no apparent organic pollution and a Habitat Rating of (Fair).

A 1990 fish survey found 11 brown trout ranging from 3-16 inches in length and seven minnow and other forage species.  There was evidence of natural reproduction in the headwater area.

A TMDL was completed in 2002.

Monitoring was completed on Swinns Creek in 2008 at STH 95 and upstream at CTH C.  Macroinvertebrate HBI ratings were very good indicating little organic loading to the stream.  Macroinvertebrate IBI ratings were excellent in upstream reaches and fair in the downstream segment suggesting overall quality decreased from the headwaters.  Continuous monitoring found water temperatures in the cold range (<69.3 degrees Fahrenheit) at both sites but coldest temperatures were found upstream.

Cool and cold fish IBI ratings were poor.  Two stock size brown trout and one young of year brook trout were found at CTH C but the rating was poor as a result of two few fish.  Qualitative habitat ratings were poor to fair.  The stream channel at both sites has been historically ditched or is very incised.  Overall stream habitat is limited by excessive streambank erosion, shallow channel depth, sedimentation and little cover for adult fish.  Most of the riparian and watershed  landuse consists of cropland, especially in middle and upper reaches.

Based on data collected in 2008 the fish community is impaired as a result of degraded habitat and supports the listing on the current 303d list.

Location of supporting data (electronic and hard copy files): WDNR Black River Falls

No new data as of 6/2/2008

Reviewer’s Name: Chris Flicek; Mark Hazuga; Revised by Mark Hazuga

Date Reviewed: 3-4-02, revised 12/22/02 by K. Schreiber, reviewed by Mark Hazuga 6/2/2008: Revised 12/22/2011