303(d) Listing Documentation

Waterbody Name: Newcomb Valley Creek

Waterbody ID Code (WBIC): 1777400

Watershed: Middle Trempealeau (BT02)

County(ies): Trempealeau

Impaired Stream Reach (location and mileage): From the mouth at Turton Creek and ending near CTH N and STH 95 intersection. (4.6 miles)

Type of impairment (water quality standards violations [ex. pH, DO, etc.], not meeting codified use [ex. WWFF should be Cold II], eutrophication, etc.):

Codified use = FFAL	Existing use = FFAL	Potential use = COLD III

Cause of impairment (ex. temperature, nutrients, sediment, BOD, toxics, etc.):

Sedimentation, lack of in-stream habitat, turbidity

Source of impairment (ex. point source, nonpoint source, internal load, unknown, etc.):

NPS (Streambank erosion)

Description of monitoring conducted (including dates, results, quality of data and reports):

1996 Buffalo-Trempealeau Basin Report

Middle Trempealeau River Priority Watershed Water Resource Appraisal Final Report (Schreiber 1991). The stream was determined to have an HBI of 3.21 indicating no apparent organic pollution and a Habitat Rating of 179 (Fair).

A 1990 fish survey found 5 brook trout (2-8.4 inches) suggesting some natural brook trout reproduction, however the stream is primarily a forage fishery.

TMDL completed in 2002.

No new data as of 6/2/2008.

Newcomb Valley Creek was surveyed in 2010 at three locations including STH 95, Soppa Road and CTH T.  Macroinvertebrate HBI ratings were very good indicating slight organic loading to the stream.  Macroinvertebrate IBI ratings were good to excellent indicating the overall quality of the stream is good.  Continuous monitoring found water temperatures in the cold and cool cold range indicating temperatures are suitable for brook trout.  Fish IBI ratings were fair at STH 95 and poor at Soppa Road and CTH T.  Poor IBI ratings at Soppa Road and CTH T were a result of too few fish, however two brook trout were captured at CTH T.  The trout were seven and eight inches in length and may have originated from Turton Creek which was stocked the fall prior to the 2010 surveys.

Qualitative habitat ratings were poor at all sites.  The stream channel is very incised and likely ditched in some areas.  In-stream habitat is also limited by shallow channel depth, streambank erosion, sedimentation and lack of adult fish cover.  The stream corridor is poorly protected and the riparian area is dominated by pasture and cropfields.

Based on data collected in 2010, the biology of Newcomb Valley Creek is impaired by degraded habitat conditions and supports the current 303d listing.

Location of supporting data (electronic and hard copy files): WDNR Black River Falls

Reviewer’s Name: Chris Flicek; Mark Hazuga

Date Reviewed: 3-4-02, revised 12/22/02 by K. Schreiber; reviewed by Mark Hazuga 6/2/2008; revised by Mark Hazuga 1/9/2011